Development of Kansas data for 2003 the RRCA Groundwater Model Update

David Barfield

April 14, 2004

This document provides documentation of procedures used to develop the Kansas 2003 data update to the RRCA Groundwater model. To fully utilize our data, new procedures and programs were developed. These procedures will be used in future data updates.

Data collection and processing – Annually, water use data (points of diversion, reported diversion quantity, reported acres irrigated, system types, etc) are collected statewide via the Kansas water use reporting system. Data for the model study domain is queried from our Water Right Information System where the wateruse data is stored, after entry and review.

Pumping files - Since wateruse is reported by point of diversion, little manipulation of the data is required to develop the pumping files. Each point of diversion is associated with a coordinate and its pumping is assigned to model cells based on a bilinear weighting scheme. This same procedure is used to estimate ground water withdrawals within the Almena Irrigation District as well as to estimate pumping for non-irrigation uses required by the Accounting Procedures.

Recharge quantity computation

Recharge quantity is computed by point of diversion by multiplying the pumping by the recharge rates prescribed in the accounting procedures for surface water systems with the addition of a new system type for pivots with drops at 12%.  Zero recharge is assumed for drip systems. For non-irrigation uses, 50% of the diversion is assumed to be recharge.

Recharge and irrigated acreage distribution

Note: See below for additional information regarding special handling of overlaps within the Almena Irrigation District.

For our 1940-2000 datasets, recharge was essentially applied to the location of the point of diversion, with adjusted when the resulting acreages as greater than the grid cell. For non-irrigation use, recharge will continue to be done in this manner. However, for years 2001, 2002, and beyond, Kansas’ method distributes recharge, as well as reported acres irrigated, to the authorized place of use.

Each water right in Kansas has a specified authorized place of use. In many cases, not all authorized lands are irrigated each year. While the annual wateruse report includes a reporting of the quantity of acres irrigated for that year, the report does not designate the particular acres irrigated. In addition, Kansas system of water rights allow for overlapping places of use and points of diversion between water rights.

To distribute estimated recharge as accurately as data will allow, we utilize a procedure developed by DWR to analyze water right groups with overlaps. A data query is utilized to develop “overlap groups” which is a listing of all the water rights and their points of diversions which have an association with one another, whether overlapping places of use or points of division. Our procedures for distributing recharge and acreages irrigated utilize these overlap groups.

Within WRIS, the authorized place of use is designated by a listing of each quarter-quarter section (commonly known as 40-acre tracts) and the number of acres within it which is authorized for wateruse under that right.

So for each overlap group the following tasks are done:

- the estimated recharge and reported acres irrigated for the year for each point of diversion within an overlap group is summed,

- a full listing of the authorized place of use for all water rights within the overlap group is developed, and

- the total recharge and reported acres irrigated for the overlap group is distributed proportionally over the groups’ total authorized places of use and then distributed to the model grid cells, based on a bilinear weighting scheme.

Additional specifics on the procedures for distributing recharge and acres irrigated is contained in the attached document which details in the program’s inputs and outputs.

Almena Irrigation District

The only area in the model domain in Kansas with significant co-mingled lands is the Almena Irrigation District of the Prairie Dog Creek sub-basin. In this area, groundwater is often used to supplement the surface supply of the Irrigation District. Procedures to determine the quantity of groundwater pumping for purposes of the model data sets did not need to be changed since they were determined based on water use records. However, distribution of recharge and determination of acres irrigated were determined based on the following procedures:

- A good approximation of the District’s lands was developed using GIS and available coverages. See Almena_qq01-02.xls for this listing of the quarter-quarter tracts within the district. The listing may be refined in subsequent years.

- Total surface-water diversions were determined based on Bureau records.

- Total return flows due to surface-water diversions were developed based on historic methods used by the RRCA.

- Groundwater points of diversion within the District boundary or within 0.5 miles outside of the District boundary were assumed to be co-mingled with the District’s surface water use. See PDsOverlappingWithAlmena.xls for this listing.

- Rather than use the commingled model data files, we assigned the surface water acres based on Almena’s report first and then prorated the reported groundwater acres within the boundary of the district so that total reported irrigated acreage was correct. We believe the will yield an appropriate number of irrigated acres in both the base run and the no-Kansas pumping run.

Recharge from other surface water irrigation use

The quantity of recharge from surface water uses was determined based on reported use and system type. The recharge was distributed to the authorized lands in the same manner as groundwater use.

Determination of non-irrigation uses less than 50 AF

As is discussed above, water rights which share either overlapping places of use and/or overlapping points over diversions were placed in a common “overlap groups” for purposes of developing the location of irrigation recharge and the location of the irrigated acres. Non-irrigation uses were also placed in a separate set of overlaps groups. Municipalities and industries should have their water use within a common overlap group. Within the spreadsheet RRCS_Overlap_Group_2003.xls, the page Non_Irr_Use_2003 has a full listing of all non-irrigation reported use within the model domain, regardless of quantity. A pivot table was built to summarize the reported non-irrigation use by group. The page Non_Irr_Use_2003_gt_50AF has all the records from the Non_Irr_Use_2003 page for which the overlap group has more than 50 AF of reported use. This page was used to determine the non-irrigation use over 50AF.

Data Files provided:

RRCS_Overlap_Groups_2003Final.xls provides the following data:

- Authorized acreage by overlap group for irrigation use, both surface water and groundwater (excluding Almena)

- Reported groundwater use by overlap group and point of diversion (PDIV_ID)

- Reported surface water use by overlap group and point of diversion (excluding Almena)

- Reported use by the Almena irrigation district

- Total reported non-irrigation use for each overlap group numbers

- Reported non-irrigation use by overlap group numbers and point of diversion

Note: The overlap groups numbers for irrigation and non-irrigation are independent of one another and the reported non-irrigation use includes both surface water and groundwater.

Almena-qq01-02.xls includes data needed to develop recharge estimates for the Almena Irrigation District including authorized place of use, reported use and acreage, and an estimate of return flows (recharge).

PDsOverlappingWithAlmena.xls includes a listing of wells (PD’s or points of diversions) with overlaps with the Almena Irrigation District.

## Documentation of computer programs and procedures to process Kansas data for the RRCA Groundwater Model for a given year

April 14, 2004

In the discussion below, references to the year 2003 are used to describe various files.  In application, the actual reference would be to the particular year being processed.

# Surface Water

Swrech.for calculate surface water return flows, surface water irrigated acreage and write monthly .rcs files and .asw file for given year.

## Input data

- Return flow factors for different system types (data is input via swrech.par file)

- Undefined 25%

- Gravity 30%

- Drip 0%

- Sprinkler (w/o drops) 17%

- Sprinkler (with drops) 12%

- Other sprinkler 17%

- center pivot and flood 17%

- Drop and other system(s) 0%

- Other 17%

- Monthly pumping distribution factors by county (uses cty.flg flag file to define county numbers and file named monthlyfactorsbycounty2003.txt to specify monthly fractions). Data file includes a listing of county numbers followed by factors for months 5 through 10 (May through October).

- Authorized acreage by overlap group (file: authacres2003.txt)

  - Overlap group number

  - Township

  - Range

  - Section

  - 40 acre tract ID (i.e.g quarter-quarter section; numbered 1-16)

  - Authorized acreage within the 40 acre tract

- Coordinates for 40-acre tracts (file: 40coords.dat)

  - Township

  - Range

  - Section

  - 40 Acre tract ID

  - X coordinate

  - Y coordinate

- Surface water diversion and acreage data (file: swrech2003.dat)

  - Overlap group number

  - Point of diversion number (PDIV_ID)

  - Diversion amount for year in AF

  - Reported acreage in acres

  - System type (see A. above for codes)

## Processing

Surface water diversion and reported acreage data are read from swrech2003.dat, the surface water return flows are computed based on system type specified, and then surface water returns and reported acreage are accumulated by overlap group.  Surface water return flows are computed based on system type specification.  A check is made to determine whether the overlap group has authorized acreage.  The reported acreage and the surface water returns associated with overlap groups that do not have authorized acreage are accumulated for later output.

The file containing authorized acreage by overlap group (authacres2003.txt) is then read to assign surface return flows and reported acreage to model cells.  The location for assigning the return flow and reported acreage is determined from the coordinate data for the particular 40-acre tract.  The amount of return flow and reported acreage assigned to a particular tract is prorated based on the amount of authorized acreage for the particular 40-acre tract divided by the total authorized acreage for the overlap particular group.

The prorated return flow and reported acreage are assigned to model cells based on a bilinear weighting scheme.  If less than 4 cells surround the particular location, the weights of the available surrounding cells are scaled so that the total weight equals one.  If the particular location is outside the model active grid such that none of the surrounding four cell locations active, the location is reported to an output file and return flow and reported acreage associated with the location is accumulated for later output.

## Output files

- Plot files – the total surface water return for the year and the surface water acreage for each model cell are written to two files, swretgrid2003.dat for return flows and swacresgrid2003.dat for reported acreage.  These files contain the model grid cell x and y coordinate followed by the amount of return flow in acre feet or the reported acreage in acres.

- Model year.month.rcs files – for each month (5 through 10; May through October), the total surface water return flow for each model cell is scaled by the monthly factor for the county in which the cell is located.  The scaled model grid array is written out in the format used by the rrpp.f program that constructs the actual model pumping and recharge files.

- Model year.asw file – the model grid array of reported acreage is written out in the format used by the rrpp.f program.

- Other output – as the program processes the data, various statistics for quality control and other information such as locations on the edge or outside the model grid are written to files.  Two files, check2003.out and output2003.dat contain this information.

# Groundwater

Pumprechv4.for calculates pumping, groundwater return flows, groundwater irrigated acreage, and M&I pumping and writes monthly .pmp and .rcg files and annual .agw and .mi files.

## Input data

- Monthly pumping distribution factors by county (uses cty.flg flag file to define county numbers and file named monthlyfactorsbycounty2003.txt to specify monthly fractions). Data file includes a listing of  county numbers followed by factors for months 5 through 10 (May through October).

- Groundwater diversion points within or adjacent to the Almena district and area factor (percentage of groundwater only land within district) (file: almenapdivs2003.txt)

  - Number of Almena groundwater diversion points and groundwater acreage factor

  - List of Almena groundwater diversion point numbers

- Authorized acreage by overlap group (file: authacres2003.txt)

  - Overlap group number

  - Township

  - Range

  - Section

  - 40 acre tract ID

  - Authorized acreage within each 40 acre tracts

- Groundwater pumping and irrigated acreage data (file: pumprech2003.txt)

  - Overlap group number

  - Point of diversion number (PDIV_ID)

  - X coordinate

  - Y coordinate

  - Return flow percentage

  - County number

  - Pumping amount in acre feet

  - Groundwater return amount in acre feet

  - Reported acreage in acres

- Coordinates for 40-acre tracts (file: 40coords.dat)

  - Township

  - Range

  - Section

  - 40-acre tract ID

  - X coordinate

  - Y coordinate

- M&I net pumping data (file: 2003mi.txt)

  - Overlap group number

  - Point of diversion number

  - X coordinate

  - Y coordinate

  - Return flow percentage

  - Pumping in acre feet

  - Groundwater return in acre feet

  - Net pumping in acre feet

## Processing

Pumping amounts are assigned to model cells based on the coordinates of the point of diversion.  The assignment is based on the same bilinear weighting scheme described above for surface water.  As the program processes the pumping data, groundwater return flows and reported acreage are accumulated by overlap group.

Groundwater return flows and reported acreage are then assigned to model cells for each overlap group based on the tracts of authorized acreage for the group.  The return flows and reported acreage for a given tract of authorized acreage are calculated by prorating the total return flow or acreage for the group by the ratio of the authorized acreage for a particular tract to the total authorized acreage for the group.  Groups for which no authorized acreage is provided are noted and the return flows and acreage for these groups are accumulated and reported in an output file.

M&I pumping is processed in a manner essentially the same as irrigation pumping except that net pumping is assigned rather than total pumping.  The location where the net pumping is assigned is based on the coordinates of the point of diversion.

## Output files

- Plot files – the total pumping, groundwater return for the year and the groundwater acreage for each model cell are written to three files, pumpgrid2003.dat for pumping, gwretgrid2003.dat for return flows and gwacresgrid2003.dat for reported acreage.  These files contain the model grid cell x and y coordinate followed by the amount of pumping or return flow in acre feet or the reported acreage in acres.  A similar file named 2003migrid.dat is output for the M&I net pumping data.

- Model year.month.pmp and year.month.rcg files - for each month (5 through 10; May through October), the total pumping and groundwater return flow for each model cell are scaled by the monthly factor for the county in which the cell is located.  The scaled model grid arrays are written out in the format used by the rrpp.f program that constructs the actual model pumping and recharge files.

- Model year.agw file – the model grid array of reported acreage is written out in the format used by the rrpp.f program.

- Model year.mi file – the model grid array of net M&I pumping is written out in the format used by the rrpp.f program.

- Other output - as the program processes the data, various statistics for quality control and other information such as locations on the edge or outside the model grid are written to files.  Two files, check2003.out and output2003.dat contain this information.