## Documentation of computer programs and procedures to process Kansas data for the RRCA Groundwater Model for a given year

February 28, 2004

In the discussion below, references to the year 2001 are used to describe various files.  In application, the actual reference would be to the particular year being processed.

# Surface Water

Swrech.for calculate surface water return flows, surface water irrigated acreage and write monthly .rcs files and .asw file for given year.

## Input data

- Return flow factors for different system types (data is input via swrech.par file)

- Undefined 25%

- Gravity 30%

- Drip 0%

- Sprinkler (w/o drops) 17%

- Sprinkler (with drops) 12%

- Other sprinkler 17%

- Other 17%

- Monthly pumping distribution factors by county (uses cty.flg flag file to define county numbers and file named monthlyfactorsbycounty2001.txt to specify monthly fractions). Data file includes a listing of county numbers followed by factors for months 5 through 10 (May through October).

- Authorized acreage by overlap group (file: authacres2001.txt)

  - Overlap group number

  - Township

  - Range

  - Section

  - 40 acre tract ID (i.e.g quarter-quarter section; numbered 1-16)

  - Authorized acreage within the 40 acre tract

- Coordinates for 40-acre tracts (file: 40coords.dat)

  - Township

  - Range

  - Section

  - 40 Acre tract ID

  - X coordinate

  - Y coordinate

- Surface water diversion and acreage data (file: swrech2001.dat)

  - Overlap group number

  - Point of diversion number (PDIV_ID)

  - Diversion amount for year in AF

  - Reported acreage in acres

  - System type (see A. above for codes)

## Processing

Surface water diversion and reported acreage data are read from swrech2001.dat, the surface water return flows are computed based on system type specified, and then surface water returns and reported acreage are accumulated by overlap group.  Surface water return flows are computed based on system type specification.  A check is made to determine whether the overlap group has authorized acreage.  The reported acreage and the surface water returns associated with overlap groups that do not have authorized acreage are accumulated for later output.

The file containing authorized acreage by overlap group (authacres2001.txt) is then read to assign surface return flows and reported acreage to model cells.  The location for assigning the return flow and reported acreage is determined from the coordinate data for the particular 40-acre tract.  The amount of return flow and reported acreage assigned to a particular tract is prorated based on the amount of authorized acreage for the particular 40-acre tract divided by the total authorized acreage for the overlap particular group.

The prorated return flow and reported acreage are assigned to model cells based on a bilinear weighting scheme.  If less than 4 cells surround the particular location, the weights of the available surrounding cells are scaled so that the total weight equals one.  If the particular location is outside the model active grid such that none of the surrounding four cell locations active, the location is reported to an output file and return flow and reported acreage associated with the location is accumulated for later output.

## Output files

- Plot files – the total surface water return for the year and the surface water acreage for each model cell are written to two files, swretgrid2001.dat for return flows and swacresgrid2001.dat for reported acreage.  These files contain the model grid cell x and y coordinate followed by the amount of return flow in acre feet or the reported acreage in acres.

- Model year.month.rcs files – for each month (5 through 10; May through October), the total surface water return flow for each model cell is scaled by the monthly factor for the county in which the cell is located.  The scaled model grid array is written out in the format used by the rrpp.f program that constructs the actual model pumping and recharge files.

- Model year.asw file – the model grid array of reported acreage is written out in the format used by the rrpp.f program.

- Other output – as the program processes the data, various statistics for quality control and other information such as locations on the edge or outside the model grid are written to files.  Two files, check2001.out and output2001.dat contain this information.

# Groundwater

Pumprechv4.for calculates pumping, groundwater return flows, groundwater irrigated acreage, and M&I pumping and writes monthly .pmp and .rcg files and annual .agw and .mi files.

## Input data

- Monthly pumping distribution factors by county (uses cty.flg flag file to define county numbers and file named monthlyfactorsbycounty2001.txt to specify monthly fractions). Data file includes a listing of  county numbers followed by factors for months 5 through 10 (May through October).

- Groundwater diversion points within or adjacent to the Almena district and area factor (percentage of groundwater only land within district) (file: almenapdivs2001.txt)

  - Number of Almena groundwater diversion points and groundwater acreage factor

  - List of Almena groundwater diversion point numbers

- Authorized acreage by overlap group (file: authacres2001.txt)

  - Overlap group number

  - Township

  - Range

  - Section

  - 40 acre tract ID

  - Authorized acreage within each 40 acre tracts

- Groundwater pumping and irrigated acreage data (file: pumprech2001.txt)

  - Overlap group number

  - Point of diversion number (PDIV_ID)

  - X coordinate

  - Y coordinate

  - Return flow percentage

  - County number

  - Pumping amount in acre feet

  - Groundwater return amount in acre feet

  - Reported acreage in acres

- Coordinates for 40-acre tracts (file: 40coords.dat)

  - Township

  - Range

  - Section

  - 40-acre tract ID

  - X coordinate

  - Y coordinate

- M&I net pumping data (file: 2001mi.txt)

  - Overlap group number

  - Point of diversion number

  - X coordinate

  - Y coordinate

  - Return flow percentage

  - Pumping in acre feet

  - Groundwater return in acre feet

  - Net pumping in acre feet

## Processing

Pumping amounts are assigned to model cells based on the coordinates of the point of diversion.  The assignment is based on the same bilinear weighting scheme described above for surface water.  As the program processes the pumping data, groundwater return flows and reported acreage are accumulated by overlap group.

Groundwater return flows and reported acreage are then assigned to model cells for each overlap group based on the tracts of authorized acreage for the group.  The return flows and reported acreage for a given tract of authorized acreage are calculated by prorating the total return flow or acreage for the group by the ratio of the authorized acreage for a particular tract to the total authorized acreage for the group.  Groups for which no authorized acreage is provided are noted and the return flows and acreage for these groups are accumulated and reported in an output file.

M&I pumping is processed in a manner essentially the same as irrigation pumping except that net pumping is assigned rather than total pumping.  The location where the net pumping is assigned is based on the coordinates of the point of diversion.

## Output files

- Plot files – the total pumping, groundwater return for the year and the groundwater acreage for each model cell are written to three files, pumpgrid2001.dat for pumping, gwretgrid2001.dat for return flows and gwacresgrid2001.dat for reported acreage.  These files contain the model grid cell x and y coordinate followed by the amount of pumping or return flow in acre feet or the reported acreage in acres.  A similar file named 2001migrid.dat is output for the M&I net pumping data.

- Model year.month.pmp and year.month.rcg files - for each month (5 through 10; May through October), the total pumping and groundwater return flow for each model cell are scaled by the monthly factor for the county in which the cell is located.  The scaled model grid arrays are written out in the format used by the rrpp.f program that constructs the actual model pumping and recharge files.

- Model year.agw file – the model grid array of reported acreage is written out in the format used by the rrpp.f program.

- Model year.mi file – the model grid array of net M&I pumping is written out in the format used by the rrpp.f program.

- Other output - as the program processes the data, various statistics for quality control and other information such as locations on the edge or outside the model grid are written to files.  Two files, check2001.out and output2001.dat contain this information.