# 3/30/2005

# Computations of Canal Seepage and Delivery

# For 2004

I. Federal Canals:

Federal canals are those with IDs 800 through 811 (806 skipped) in Table 4, a total of 11 canals.

DATA SOURCE (S):

Monthly diversion and delivery data for each federal canal were obtained from the U.S. Bureau of Reclamation (Craig Scott); six related Excel files are listed in Table 4.

METHOD:

Recharge from Federal Canals was computed according to the Accounting Method in Attachment 7 of the Settlement Document. All computations were carried out on a monthly basis.

[Canal loss] = [diversion] – [field delivery]

[Canal seepage] = [percent loss factor] * [canal loss]

[Field seepage] = [field loss factor] * [percent loss factor] * [field delivery]

*** For Courtland Canal above Lovewell Reservoir

[Canal loss] = [Headgate diversion] – [Nebraska delivery]

- [Kansas delivery] – [Courtland Canal inflow to Lovewell]

[Canal seepage] = [percent loss factor] * [canal loss]

The field loss factor and the percent loss factor for each canal are presented in Table 1.

Table 1 Loss Factors for Federal Canals

| Canal ID | Canal Name | Field Loss Factor | Percent Loss Factor |
| --- | --- | --- | --- |
| 800 | Culbertson | 0.30 | 0.82 |
| 801 | Culbertson Ext. | 0.30 | 0.82 |
| 802 | Meeker-Driftwood | 0.30 | 0.82 |
| 803 | Bartley | 0.30 | 0.82 |
| 804 | Red Willow | 0.30 | 0.82 |
| 805 | Cambridge | 0.30 | 0.82 |
| 807 | Franklin | 0.35 | 0.82 |
| 808 | Naponee | 0.35 | 0.82 |
| 809 | Franklin Pump | 0.35 | 0.82 |
| 810 | Superior | 0.31 | 0.82 |
| 811 | Courtland | 0.23 | 0.82 |

RESULTS:

Monthly delivery and seepage for each canal are presented in USBR_CANAL_SEEPAGE04.XLS.

II. Central Nebraska Public Power & Irrigation District (CNPPID) Canals:

The Central system was broken into seven sections.  Four of these (Jeffrey Hydro, Jeffrey Hydro – J1, J1 – J2, J2 – Platte) make up the whole of the Supply Canal.  The remaining three (Phelps, E-65, E-67) are the primary irrigation ditch systems into which water from the Supply Canal is diverted.  The canals IDs for Central system are 815, 816, 817, 818, 819, 820, and 821. Seepage loss for Elwood Reservoir (827) was also estimated.

DATA SOURCES:

Canal diversions, hydro discharges/returns, reservoir storage/outflow, etc. are form: 2001 to 2004 data.xls;

Annual deliveries for Phelps, E65, and E67 canals are from: IRREFNCY.xls

Direct irrigation information for Diversion Dam to Jeffrey, Jeffrey Hydro to J-1 hydro, and J-1 hydro to J-2 hydro reaches are from Supply canal irrigation 2001 to 2004.xls.

Jeremie Kerkman of CNPPID provides all files above. In the 2001 to 2004 data.xls file, three additional sheets were added to store the pre-processing results for:

- Monthly diversions/inflows/outflows/hydro returns;

- Annual storage changes for reservoirs; and

- Monthly storage changes of Elwood reservoir.

Central canal areas utilized for net evaporation computation are presented in Table2. They are pulled out from old computation sheets.

Table 2 Central Canal Areas Utilized for Net Evaporation

| Canal ID | Canal Name | Surface Area [acres] |
| --- | --- | --- |
| 815 | Jeffrey Hydro* | 917 |
| 816 | Jeffrey Hydro to J1* | 3790 |
| 817 | J1 to J2* | 296 |
| 818 | J2 to Platte | 94 |
| 819 | Phelps | 631 |
| 820 | E65 | 301 |
| 821 | E67 | 21 |
|  | Elwood Reservoir | 1200 |

* Total area includes reservoirs.

Net evaporation rate for Medicine Creek Dam (Harry Strunk Lake) is used for computing CNPPID system’s net evaporation losses, which was computed from the pan evaporation at the reservoir and precipitation at the dam and stored in MedicineCrdamNetEvap.xls.

METHOD:

The general process for computing seepage is as follows:

- Start with annual diversions.

- Subtract annual diversions into other canals, direct irrigation uses, storage changes, downstream deliveries, and any other such quantities relevant to a particular canal.

- Based on (1) and (2), arrive at an annual value of total losses from the canal.

- Subtract annual net evaporation from the total loss value.

- We now have an annual canal seepage loss value.

- Compute the ratio of annual seepage to annual diversion.  This fractional value is assumed to be valid for all months of the year.

- Generate monthly seepage loss values by a proportioning process.  Multiply the ratios computed in (6) by the known monthly diversion values to get estimates of monthly seepage.

- Monthly irrigation delivery was computed by subtracting monthly seepage and monthly net evaporation from the monthly diversion.

Headgate to Jeffrey Hydro Reach:

[total loss] = [Central diversion] – [change in canal storage] – [change in Jeffrey storage] – [direct irrigation use] – [Jeffrey hydro discharge]

[total seepage] = [total loss] – [annual net evaporation]

Annual net evaporation was computed as a sum of products of monthly Medicine Creek Dam net evaporation and canal reach surface area. Monthly canal reach seepage was computed using monthly diversion and the annual ratio of seepage to diversion. These same computation steps were used for the rest of Central canals.

Jeffrey Hydro to J-1 Reach:

[total loss] = [Jeffrey hydro discharge] – [Jeffery return] - [change in canal storage] – [change in Johnson storage] – [T. M. siphon] - [direct irrigation use] –

[E65 diversion (diversion for E65)] – [Elwood inflow (diversion for Elwood)] – [E67 diversion] – [J-1 hydro discharge]

[total seepage] = [total loss] – [annual net evaporation]

Note: Elwood Reservoir is located on the E-65 system, and inflow into this reservoir is actually diverted at the E-65 headgate. However, the data records separate these quantities.

J-1 to J-2 Reach:

[total loss] =  [J-1 hydro discharge] - [change in canal (J-2 Forebay) storage] –

[direct irrigation use] – [J-2 hydro discharge]

[total seepage] = [total loss] – [annual net evaporation]

J-2 to Platte Reach:

[total loss] =  [J-2 hydro discharge] – [Phelps diversion] – [J-2 return]

[total seepage] = [total loss] – [annual net evaporation]

Phelps Canal System:

[total loss] = [Phelps diversion] – [farm delivery]

[total seepage] = [total loss] – [annual net evaporation]

[Monthly seepage] = [monthly diversion] * [annual seepage]/[annual diversion]

[Monthly delivery] = [monthly diversion] – [monthly seepage]

– [monthly net evaporation]

E-65 Canal System:

[total loss] =  [E65 diversion] + [Elwood outflow] - [farm delivery]

[total seepage] = [total loss] – [annual net evaporation]

[Monthly seepage] = [monthly diversion] * [annual seepage]/[annual diversion]

[Monthly delivery] = [monthly diversion] – [monthly seepage]

– [monthly net evaporation]

E-67 Canal System:

[total loss] =  [E67 diversion] - [farm delivery]

[total seepage] = [total loss] – [annual net evaporation]

[Monthly seepage] = [monthly diversion] * [annual seepage]/[annual diversion]

[Monthly delivery] = [monthly diversion] – [monthly seepage]

– [monthly net evaporation]

Elwood Reservoir:

A monthly mass balance was computed for Elwood Reservoir in order to determine seepage losses from the reservoir. The mass balance was computed as

[Elwood Reservoir mass balance] = [Inflow] – [Outflow] – [Net Evaporation]

– [Storage Change]

Net evaporation was calculated using the Medicine Creek net evaporation data and a reservoir surface area of 1200 acres. Elwood Reservoir monthly change in storage was computed using its daily content (last day storage content minus first storage content of the month). If the mass balance calculation yielded a negative value, the seepage was set to zero.  Otherwise, seepage is equal to the result of the balance.

RESULTS:

The computed monthly seepage losses and irrigation deliveries are in CNPPID_System_Seepage04.xls.

III. Nebraska Public Power District (NPPD) Seepage:

NPPD system was broken into 4 reaches: Headgates to Sutherland Reservoir (Reach 1), Sutherland reservoir (Reach 2), Sutherland Reservoir discharge to Lake Maloney inflow (reach 3), and Lake Maloney inflow to South Platte return (Reach 4).

DATA SOURCES:

Daily flow and storage data are provided by NPPD (Paul Srock) in the file Info for state.xls, which was further processed to generate:

- Monthly Keystone diversion

- Monthly Korty diversion

- Monthly Sutherland Reservoir inflow and outflow

- Monthly Lake Maloney inflow and outflow (Sutherland power return)

- Monthly average storage for Sutherland and Maloney Lake, and

- Monthly storage changes for Sutherland and Maloney Lake

METHOD:

A monthly mass balance was carried out for each reach. The general equation is as follows:

[Mass balance] = [inflow] – [outflow] – [net evaporation] – [storage change]

If the balance is less than zero, the monthly seepage was set to be zero. Otherwise it is equal to the balance.

Net evaporation loss for each reach was computed as the product of net evaporation rate at Medicine Creek Dam and the reach’s surface area. NPPD system reach areas utilized for net evaporation loss computation are presented in Table 3. They are pulled out from old computation sheets.

Table 3 NPPD Reach Areas Utilized for Net Evaporation

| Reach Name | Surface Area [acres] |
| --- | --- |
| Headgates to Sutherland Reservoir | 222.5446 |
| Sutherland Reservoir | 3050 |
| Sutherland Reservoir discharge to Lake Maloney inflow | 209.4545 |
| Lake Maloney inflow to South Platte return | 1718.982 |

Total seepage loss of the NPPD system is the sum of the losses of 4 reaches.

Headgates to Sutherland Reservoir (Reach 1):

[Mass balance] = [Keystone + Korty diversions] – [Net evaporation] –

[Sutherland Reservoir inflow]

[Net Evaporation] = [Net Evap from Keystone diversion] +

[Net Evap from Korty diversion] +

[Sutherland Canal Net evap from Paxton to Sutherland Reservoir]

Due to the fact that Keystone to Paxton reach drains to North or South Platte River, the computed balance was subtracted by about 51.3%:

[Mass balance] = (1.0-20.4/39.8) * [Mass balance]

Sutherland Reservoir (Reach 2):

[Mass balance] = [Sutherland Reservoir inflow] – [Net evaporation] –

[Change in storage] – [Sutherland Reservoir outflow]

[Change in storage] = [last day of month storage] – [first day of month storage]

[Net evaporation] = [weather-induced evaporation] + [heat-induced evaporation]

The heat-induced evaporation from Sutherland Reservoir was based on 21 cfs estimate quoted in Cassidy (1998, 2000).

Sutherland Reservoir Discharge to Lake Maloney Inflow (Reach 3):

[Mass balance] = [Sutherland Reservoir discharge] – [Net evaporation] –

[Lake Maloney inflow]

Lake Maloney Inflow to South Platte Return (Reach 4):

[Mass balance] = [Maloney inflow] – [Net evaporation] –

[Change in Lake Maloney storage] – [Maloney discharge (South Platte return)]

RESULTS:

Computed seepage losses for each reach and total NPPD system are presented in

NPPD_Seepage04.xls.

IV. Private Canals Seepage:

Seven private canals are as follows:

Canal ID	Canal Name

812		Haigler

813		Champion

814		Riverside

822		Six Mile

823		Thirty Mile

824		Orchard Alfalfa

826		Western

Recharge from canal seepage is assumed to be zero. All recharge from these canals is assumed to be from application of surface water to irrigated lands. Surface water deliveries from private canals were assumed to be equal to canal diversions. Recharge from private canals is stipulated to be 40% of diversion.

NPPD/Bridgeport Field Office provides monthly diversion data for Six Mile, Thirty Mile, Orchard Alfalfa, and Western canal.

Cambridge Field Office provides monthly diversion data for Haigler and Riverside. Champion Canal was abandoned in 1993.

Table 4 Data Sources

| Canal ID | Canal Name | Data Source File |
| --- | --- | --- |
| 800 | Culbertson | F-VAL3MWD.XLS (USBR) |
| 801 | Culbertson Ext. |  |
| 802 | Meeker-Driftwood | F-CAMB3MWD.XLS (USBR) |
| 803 | Bartley |  |
| 804 | Red Willow |  |
| 805 | Cambridge |  |
| 807 | Franklin | NE-BOST3MWD.XLS (USBR) |
| 808 | Naponee |  |
| 809 | Franklin Pump |  |
| 810 | Superior |  |
| 811 | Courtland | NE-BOST3MWD.XLS; KS-BOST3MWD.XLS; C07-dly.xls; C348-dly.xls (USBR) |
| 812 | Haigler |  |
| 813 | Champion |  |
| 814 | Riverside |  |
| 815 | Jeffrey Hydro | CNPPID: 2001 to 2004 data.xls; IRREFNCY.xls; Supply Canal irrigation 2001 to 2004.xls USBR: Hs-pan.XLS; Hs-prec.XLS |
| 816 | Jeffrey Hydro to J1 |  |
| 817 | J1 to J2 |  |
| 818 | J2 to Platte River |  |
| 819 | Phelps |  |
| 820 | E65 |  |
| 821 | E67 |  |
| 822 | Six Mile |  |
| 823 | Thirty Mile |  |
| 824 | Orchard-Alfalfa |  |
| 825 | NPPD | Info for state.xls (NPPD) Hs-pan.XLS; Hs-prec.XLS (USBR) |
| 826 | Western |  |