Republican River Compact Accounting (RRCA):  
2025 Non-Federal Reservoir (NFR) Evaporation Procedures

March 27, 2026

Caitlin Kingsley

2025 Procedures Overview

As of July 1, 2025, the Nebraska Department of Natural Resources (NeDNR) and the Nebraska Department of Environment and Energy (NDEE) merged into the Nebraska Department of Water, Energy, and Environment (DWEE). Certain historical references to NeDNR within this document have been retained, while other references have been updated to DWEE.

In late 2012, NeDNR’s Dam Safety Division invested significant resources in updating the dams database within the Republican River Basin using the best readily available data, including aerial imagery, Light Detection and Ranging (LiDAR) data, and more than 50 field surveys of reservoirs.

Each year and on a rotating basis (by Basin), the DWEE Dam Safety Division utilizes aerial imagery and field investigations to update their database. Updates may include additions or removals to the database as the result of new or removed dams, dam breaches, etc.

Since the 2013 NFR evaluation, NeDNR/DWEE has been able to estimate evaporation using the presumptive average annual surface area described in the RRCA accounting procedures and reporting requirements.  As such, this is the method that DWEE implements for reservoirs with a storage capacity of less than 200 AF. For reservoirs with a storage capacity of 200 AF or greater, DWEE uses field observations to estimate the average annual surface area.  The results of field observations and area representations that were used for 2025 NFR calculations are listed in Table 1.

2025 Data Package Description

The data package for the 2025 RRCA NFR Evaporation Assessment includes the following files:

- NFR2025.gdb\NFR2025_DamPoints – a GIS layer containing dam points for 2025 non-federal reservoirs 15 AF or greater (455 features).

- SummariesBySubbasin2025.xlsx – A table summarizing total evaporation by Republican River subbasin.

- Data2025 (folder) – contains Excel spreadsheets of federal reservoir pan evaporation and precipitation measurements obtained by the Bureau of Reclamation.

- NFR2025_AddedRemovedReservoirs.xlsx- A workbook with two worksheets summarizing the name and location of new and removed non-federal reservoirs within the Republican River Basin in 2025.

- 2025FederalReservoirEvapCalcs.xlsx – A table showing precipitation and pan evaporation measurements for Federal Reservoirs used in NFR analysis, and calculated evaporation.

- 2025NFRprocedures.docx – this document, which outlines procedures and         summarizes field observations.

Table 1:  Summary of field investigation results for reservoirs 200 AF or greater and reservoir surface area representations used in the 2025 NFR analysis.