V2 (POTential YieLD Revised POTYLDR 2.0)

Water Budget Simulation Model

User’s Manual

Prepared by

James K. Koelliker

Biological and Agricultural Engineering Department

Kansas State University

Manhattan, KS

August 2013

This manual is one of the work products from the Project,

Modeling and Field Experimentation to Determine the Effects of Land Terracing and Non-Federal Reservoirs on Water Supplies in the Republican River Basin Above Hardy, Nebraska

Supported by the USDI, Bureau of Reclamation,

Billings, Montana

This work was done in cooperation with the

Biological Systems Engineering Department,

University of Nebraska – Lincoln

and

The Conservation Committee for

the Republican River Compact Administration

Author:  James Koelliker is Professor Emeritus of Biological and Agricultural Engineering at Kansas State University, Manhattan, KS 66506, 785-323-7157, koellik@ksu.edu.

This version of the code and the previous ones are written in FORTRAN.  All of the revisions and new code develop was done using the Lahey/Fujitsu Fortran 95 compiler and should be compatible with systems running Lahey LF95 or LF90.  The creating of and subsequent editing of most text files was done using NotePad++, http://notepad-plus-plus.org.

This manual was developed for the latest version of the model and add-on programs are used as part of a procedure to estimate the impact of land terracing and small federal reservoirs on streamflow and ground-water recharge in the Republican River Basin above Hardy, NE.

The user of this manual should be familiar with the final report from overall project so that the basis of this modeling work can be in put into proper perspective with the intent for the use for which it has been developed and modified to meet the demanding needs of the study.

The entire study was a combined effort of many individuals and organizations involved in the management and use of the water resources in the Republican River Basin above Hardy, NE.

Special thanks are extended to Professors Darrel Martin and Dean Eisenhauer of the University of Nebraska – Lincoln, Scott Guenther of the Bureau of Reclamation, and Scott Ross of the Kansas Department of Agriculture, Division of Water Resources for their expertise and reliable help with many of the details of the study that were necessary to incorporate into the modeling effort.

Table of Contents

Table of Contents	ii

List of Figures	iv

List of Tables	iv

BRIEF BACKGROUND	1

POTYLDR 2.0 MODEL DESCRIPTION	3

Input Meteorological Data	7

Input HRU Land-Use Data	8

Soil Characteristics Input Data	8

Interception Losses	9

Infiltration and Runoff	9

Evapotranspiration	12

Non-Growing Season	13

Snow Cover Condition	13

Soil Water Movement	14

Typical Pond	15

Dimensional Characteristics	16

Seepage Rate	16

Evaporation Rate	17

Overflow	17

Changing Watershed Conditions With Time	18

SPECIFIC APPLICATION INFORMATION FOR THE REPUBLICAN

RIVER BASIN STUDY	18

Some Background	18

STEP 1 – V2	21

STEP 2 – TERRACE POND	30

Background Information	30

Specifics for Terraces	31

STEP 3 – TERRACECHANNEL	34

STEP 4 – CNSLD, Consolidation of Simulation Results	38

STEP 5 – POST, Post-Processing Results for Each HUC 12	39

EXECUTION CHAIN for Batch Processing for Simulating the Results

for the Study Area	43

Other Files Required in the Directory for a Run of the Batch File	45

Using the MS-DOS Prompt	46

SOME FINAL THOUGHTS	46

REFERENCES	48

LIST OF ACRONYMS AND SYMBOLS USED	49

APPENDIX A	50

List of Tables

Table 1.  Example input file, LCBBTERRACEDATA.DAT, to represent the level,

closed-end broadbase type	32

Table 2.  Characteristics of the two terrace types used to estimate the amount of

overflow from and infiltration into the levels in the terrace channel	35

Table 3.  Factors used to represent land terraces in the area above small reservoirs

to account for the effect of the terraces on surface runoff	37

Table 4.  Portion of the FINALRESULTS file generated by POST	42

List of Figures

Figure 1.  Schematic of POTYLD, POTYLDR, and POTYLDR 2.0 water-budget

simulation model	5

Figure 2.  Relationship between actual RCN and ASM used in POTYLDR 2.0	10

Figure 3.  Relationship between ASM and deep percolation rate for a typical deep

silt loam in the study area that was added to POTYLDR 2.0	16

Figure 4.  Idealized HUC 12 showing those portions represented in the water-budget modeling process	21

Figure 5a. Complete operation to simulate effects of land terraces and small federal reservoirs on runoff and ground-water recharge in the Republican River Basin	23

Figure 5b. Complete operation to simulate effects of land terraces and small federal reservoirs on runoff and ground-water recharge in the Republican River Basin	24

Figure 6.  Terrace cross-section (not to scale) used to represent broadbase

terraces	31

Figure 7.  Terrace cross-section (not to scale) used to represent flat-channel

terraces	32

BRIEF BACKGROUND

The question of the effects of changes in land use, agricultural management practices, land conservation practices, small reservoirs, and irrigation on the surface and ground-water resources in the Great Plains Region has been ongoing for at least sixty years.  Some streamflow records have been continuous for nearly 100 years and they were started during a period when the amount of land used for crop production in the region was at or near maximum.  Subsequently, wind and water erosion had major effects on the area and the amount of cropland has generally decreased to around fifty percent since the 1950s.  Also, adopting of better adapted farming practices and management along with conservation practices to reduce wind and water erosion has resulted in less surface runoff from cropland.  Those areas that was taken out of cropping uses and returned to grassland, too, now yields less runoff than while they were cropped.  Continued development and use of conservation tillage and no-tillage production systems has reduced runoff even further.

Following the drought of the 1950’s extensive amounts of land terraces and small reservoirs were installed in the Region with technical and financial assistance from the U.S. government through programs operated by the U.S. Department of Agriculture.  At nearly the same the U.S. Interior, Bureau of Reclamation and the U.S. Army Corps of Engineers were adding large reservoirs to manage the streamflow in the Region for irrigation, to control flooding, and to provide recreation and other benefits in the Region.  As time has passed streamflow into most of those reservoirs has decreased.  Simultaneously, irrigation with groundwater and some surface water was developed to the extent that the amount of groundwater in storage in the major regional aquifer and in the alluvial aquifers along streams has been reduced.  The interaction between the ground-water system and the streams has generally resulted in reduced baseflow into the stream system and has added to the reduction in total streamflow in the Region.

Over the past thirty years, I have spent much of research efforts on attempting to understand and quantify these effects in the general region of Kansas and surrounding states.  I have been a part of projects with numerous colleagues and graduate students.  Specific study areas have included the South Fork of the Solomon River in Kansas, the Republican River Basin, and the Wet Walnut Creek Watershed for two different projects.  A general summary of some of this work can be found in Koelliker (1998) in Chapter 7 of Perspectives on Sustainable Development of Water Resources in Kansas that is based upon a number of simulations across the state to show the effects of agriculture on water yield.  Also, there I provide a long-term perspective on those effects since widespread agricultural utilization of the Region started for the South Fork of the Solomon River Basin in Kansas above Webster Reservoir that I developed about twenty-five years ago.  The continuing trends of less streamflow has developed to such an extent in some areas regulatory measures have been put into place to try to reverse the long-term trend of reductions in streamflow.  In addition, some of my recent analyses indicate that for about the last twenty years that the amount of precipitation has generally increased in the Region.

The original work with POTYLD for the South Fork of the Solomon River, Kansas above Webster Reservoir was completed in 1980 with information available through 1978.  At that time, I speculated on the historical water yield from the 1,040 square mile basin from 1860 to 2000 that included depletions from the various factors of dams, terraces, residue (conservation tillage), and irrigation.  The original version of that graph can be found in Koelliker (1984) and is adapted in Koelliker (1998).  My curiosity along with the recent improved water yield into the reservoir has prompted me to briefly evaluate what has occurred there since 1978.  The water yield from this watershed is dominated by surface runoff, but base flow has become more important in the total water yield since surface runoff has decreased markedly by all of the various factors and the fact that the alluvial aquifer along the main stream was closed to further development in the 1980’s.

So, POTYLDR 2.0 and this manual for it is my attempt to pass this work on to others who might find continuing to study and describe this work to others important.  Water resources in the Region are extremely valuable to sustain the economic activity.  Having the ability to understand the effects and consequences of technology and regulatory actions is needed.  It is my long-term hope that this model and its use by interested and competent hydrologists will provide insights and knowledge.

POTYLDR 2.0 MODEL DESCRIPTION

The original version of the model, POTential YieLD (POTYLD) was developed for the Bureau of Reclamation (1984) to assess the effects of land use and conservation practices in large watersheds (Koelliker et al., 1981, Koelliker et al., 1982).  POTYLD is the basis of POTYLD Revised (POTYLDR) that includes revisions and updates to the POTYLD model.  This work has been done mainly to describe the conditions in the semi-arid and sub-humid conditions of the Great Plains and Central United States.  This area generally is represented by limited precipitation and limited runoff.  In this area conservation practices are effective at reducing the limited amount of runoff by improving infiltration.  Subsequent long periods of evapotranspiration result in most of the retained runoff being lost as evaporation and transpiration.

POTYLD simulated the daily change in the water budget for different climatic and land-use conditions to estimate the dispensation of precipitation as interception, runoff, actual evapotranspiration, percolation, and change in water content in the soil.  The model is a unit area type and considers only water movement in the vertical direction in the soil profile.  The each unique combination of a condition that was simulated was called a subarea and up to 18 subareas could be simulated at a time.  Figure 1 shows a general schematic representation of land-use subareas on the right-hand side.  POTYLD utilized runoff curve numbers (RCN) values to predict the split between runoff and infiltration for different areas where precipitation and/or irrigation added in daily amounts.  Individual land uses and conservation practice conditions can be described by a RCN, and the RCN technique is used widely to predict runoff from design storms.  It follows that the RCN method can predict runoff over a period of time provided the antecedent moisture condition (AMC), a measure of the wetness of the soil at the time of each precipitation event, can be determined.  This technique to assess runoff through computer simulation modeling is used widely in watershed simulation models.

The POTYLD model included the simulation of the operation of a typical pond or small reservoir to predict their effect on runoff from the area above the reservoir.  To do this the pond is represented by an inverted frustum of a pyramid.  This simple shape has been found to adequately represent the stage-storage-volume relationship well provided the bottom dimensions and side-slope are adjusted properly.  The amount of runoff from subareas in the drainage area of the pond were determined by assigning an amount of  area of each one and multiplying the depth or runoff times the area and summing them to provide the inflow to the pond.  Seepage from the pond was estimated by a constant rate times the daily surface area of water in the pond.  Evaporation was estimated by the daily value of reference evapotranspiration times 1.1 and the precipitation added was the amount that fell on the water surface area of the pond.  Overflow or discharge occurred whenever the permanent storage volume in the pond was exceeded.  Figure 1 shows the schematic representation of the typical pond.  The primary use of POTYLD was to assess effects on surface runoff as required by the original use for the Bureau of Reclamation.  Those changes estimated were those attributed to changes on the land included land use changes from one use to another and conservation practices that included use of residue and tillage practices along with terraces.  Also, the effects of ponds on runoff were determined, too.  Since many areas had multiple changes, the effects attributed to the various factors depended on where they were located in the watershed.  Land uses from one use to another and changes in tillage practices occur upstream of terraces, so they were given full effects.  Next, terraces were assigned only the additional effects that they added that were mainly by the retention of detention of runoff.  Finally, the reservoirs were credited only with the additional effects they provided by retaining runoff.

The POTYLDR model included numerous changes from the original POTYLD model.  Initially, the changes were made in 1993 by Koelliker (1994) during a sabbatical leave to improve the surface evaporation processes and to add some components to it to provide estimates of runoff and ground-water recharge on a small watershed scale.  There, the sub-area and typical pond results were scaled up by assigning areas to the land-uses and including the number of typical ponds.  The capability to schedule and account for effects of irrigation on the surface water budget for irrigated sub-areas was added.  Also, a simple routine to estimate transmission losses for streamflow was added so that results could be compared to measured streamflow.  Additional

Figure 1.  Schematic of POTYLD, POTYLDR, and POTYLDR 2.0 water-budget

simulation model.  NOTE:  Land-use subareas are called HRUs in POTYLDR 2.0.

revisions included a way to assign amounts of the various land uses and number of typical ponds over time to provide a way to estimate changing watershed conditions with time so that results

could be compared with streamflow for an actual watershed.  Koelliker (1998) wrote a chapter in Perspectives on Sustainable Development of Water Resources in Kansas based upon a number of simulations across the state to show the effects of agriculture on water yield.  Those results showed that the potential for agricultural conservation practices and other changes in agricultural production practices on cropland in the Kansas portion of the study area to reduce streamflow from about 30% in the east to 80% in the west.

The most extensive use of POTYLDR was in a combination surface water and ground-water modeling project for the Wet Walnut Watershed in Central Kansas (Ramireddygari et al., 2000).  POTYLDR along with added stream-routing routines from Arnold et al. (1993) provided the surface water to the major stream and the ground-water recharge under the watershed.  Also, POTYLDR was used to estimate the irrigation demand in the watershed.  The MODFLOW model was used to simulate the ground-water movement and stream-aquifer interactions in the main stream reach.  The typical pond portion of POTYLDR also operated the numerous watershed structures in the watershed.  This work was the first to use the percolation and seepage values and ground-water pumpage for irrigation produced by POTYLDR to provide values that would result in values that resulted in a good fit of results to the total surface and ground-water budget for the approximately 1,700 square mile watershed.

As time passed interest in applying to POTYLDR model to estimate the impact of land terraces and small federal reservoirs on streamflow and ground-water recharge in the Republican River Basin above Hardy, NE developed in about 2004.  The Republican River Conservation Committee and the Bureau of Reclamation realized that the amounts of these practices in the Basin were extensive and that their effects were of interest to the Compact for the Basin.  So, in 2005 a study was started for this purpose.  POTYLDR was selected as the water-budget simulation model for that study.  As the study proceeded there were several additional changes to POTYLDR to add to its capabilities to simulate the particular conditions of interest.  Of great interest was determining the effects and quantifying the effects of storage-type terraces on ground-water recharge and additional evapotranspiration.  Also, the getting better understanding of operation of the water budget of small federal reservoirs was of needed.  So, the current version, POTYLDR 2.0, described in this report is the outcome of the work for the Republican River Basin Study done by the author.  This is a more targeted version that limits its purpose to simulating the water budget for up to 24 land-use conditions.   It does not include any of the post-processing aspects of POTYLDR because none of that was of direct use for the Republican Study.  The author may incorporate those aspects into POTYLDR 2.0 at some future time.  Also, previous versions of the model have been used to attempt to simulate all of the major different conditions that are in a particular land area such as a small watershed or those around a particular weather station.

From this point on, the POTYLDR 2.0 is referred to simply as V2.  Following is a description of the water budget processes for the land-based portion of the V2.  As the Republican study evolved, the term to identify the land-use sub-areas was changed to hydrologic response unit (HRU) which represents a single unique set of conditions – soil type, cropping sequence, conservation practice, residue level, RCN, and irrigation or no irrigation for which the operation of the water budget is desired.  Also, the modeling work for the Republican Study was limited to predicting the effects of land terraces and small federal reservoirs on the streamflow and ground-water recharge in the Basin.  Since land terraces are located almost exclusively on cropped or previously-cropped areas that are subject to water erosion and small reservoirs are predominantly is areas with similar conditions, modeling was limited to only those conditions that were predominant across the Basin.  So, V2 and the results from it should not used to make inferences about the water budget for the entire Basin.

The information about the water budget processes that follow are described in a vertical sequence from input of meteorological data down through percolation from the lower zone (LZ).  V2 is considers only vertical movement of water except for runoff which is a lateral outflow from the surface.

Input Meteorological Data

All versions of the model have always been run in daily time steps, so daily meteorological data is needed and it includes precipitation, maximum temperature, and minimum temperature.  A total of 32 stations were used across the study area from data collected by Derrel Martin (2007) from the Great Plains Climate Center for the period, 1950-2008.  For V2, continues to be in the input format of monthly blocks for each station for ease of reading and to assure that the correct record is used.  This also allows for use of different data for other stations if desired.

Before the latest revisions, daily reference evapotranspiration had to be estimated based upon average monthly input values and geographical location that corresponded to the weather station.  Now, reference evaporation data are available for as long as 30 years or more some locations and techniques to make estimates for longer periods are available.  So, those values can be used directly in the model.  Derrel Martin (2007) provided these data for the same 32 weather stations for this project.  So, the current version of the model uses the grass reference evapotranspiration (Eto) directly from input files that are also stored in monthly blocks for ease of reading.  All of the meteorological data can be found in the Subdirectory – Meteorological Stations Files.  Also they are included in the subdirectory, Example Full Model Run Set - 1999-2008, that has all of the necessary files to make a full simulation for the basin for the last 10 years of data.

Input HRU Land-Use Data

The 6-year rotation scheme for land use was added to V2 to replace the limited 2-year wheat-fallow as the only rotation.  This was necessary to handle the 3-year, wheat-rowcrop-fallow, rotation widely used in the Basin.

Standard runoff curve numbers for each land use for each month of the year were built in.  An overall adjustment factor in the input file was added to adjust the standard values for a run, too.  Further, the input information for HRUs was modified to include the factor to adjust the runoff curve number for each one with a factor in the input files times the standard runoff curve number as a way to make further adjustments for runoff.  The requirement to add runoff curve numbers for each land use was removed.

A table of standard residue level effects on evaporation from the soil when plants were not growing was added along with an overall adjustment factor in the input file to adjust the standard values for a run was added.

Three sets of water-use coefficients for each land use on a weekly basis were derived by Martin for three regions, east, middle, and west, and are read in as an input file for the grass-reference values, Eto.

Soil Characteristics Input Data

V2 continues to use the 2-layer soil system, but several modifications were made.

Only soil types 5, 7, and 11 are used in V2.  Previous versions of the model had paid less attention to getting the amount of percolation or ground-water recharge to agree with what was judged to be acceptable values.  Also, from previous work with long-term moisture storage under terraces around Colby, KS and from some modeling of over-winter water storage on irrigated fields resulted in adding a drainage routine to the lower zone soil-water routine.  That work found that after about two years that the water content in the lower parts of the rooting zone would drain down to about 70% of field capacity.  Since there are long periods of little water use from the lower zone under dryland conditions, the drainage increases the percolation from those soils when they are wet.  Also, the thickness of the lower zone that previously was a function of soil type and a maximum of 4 feet was changed to a variable input and most runs was 5 feet for a total active soil-water zone of a total of 6 feet.  This necessitated changing the data values stored in V2 to be a per foot of depth rather than a total value for the LZ.

Interception Losses

As precipitation occurs it falls first on some surface condition that ranges from the plant canopy, surface residue or bare soil in a few cases.  The surface must first be wetted sufficiently to satisfy the surface storage on the material covering the surface and overcome any evaporation from those surfaces that might be occurring during the precipitation event.  Even some water that falls on bare soil may soon evaporate directly back to the atmosphere and not get into the active soil-water storage zone in the soil.  V2 uses a theoretical reservoir at the surface that holds 0.07 inches of water regardless of the surface conditions.  The water that enters this reservoir is lost back to the atmosphere as soon as an equivalent amount of Eto sufficient to empty it is received.  If the reservoir already has some water in it when a precipitation event occurs, then the amount needed to fill it is reduced by the amount already in the reservoir.  The incoming energy used to evaporate the water in the reservoir each day is subtracted from the amount of energy that is used as input to calculate actual evapotranspiration (AET) or soil-water evaporation.  Since many of the precipitation events in the study area are small, interception is an important process in the water budget.  Around twenty percent of the total precipitation is calculated to be lost by the interception process.

Infiltration and Runoff

Rainfall that makes it to the soil surface is split into infiltration and runoff.  A RCN value for AMC II, moderately wet soil moisture condition for each HRU is needed to determine the daily runoff amount.  V2 uses estimated monthly values of RCN for the various land uses that are applicable to the study area.  The actual RCN for each day is adjusted by the actual AMC calculated for the day for based on the available soil moisture content (ASM) of the Upper Zone (UZ).  The actual RCN for the day is determined by the RCN for AMC II times a factor based upon soil wetness that adjusts the value up or down.  If the UZ ASM is less than 65% of ASM, the RCN corresponding AMC I, dry condition is used.  If the ASM is 75%, then RCN for AMC II is used.  Between ASM equal to 65 to 75%, the RCN is adjusted between 1.0 and 2.0 on a linear scale.  For wet conditions when ASM is 95% or greater, the RCN for AMC III, wet conditions, is used.  Between 75% and 95% ASM, the RCN is adjusted between 2.0 and 3.0 on a linear scale.  Figure 2 shows the corresponding calculated RCN for the range of ASM for the

Figure 2.  Relationship between actual RCN and ASM used in POTYLDR 2.0.

RCN for AMC II of 72.  This would be typical of dryland corn in a wheat-corn-fallow rotation with good residue levels on a silt-loam type soil.  V2 does have a way to adjust the RCNs globally with an adjustment factor that changes all RCNs by the value used in the input data stream.  Also, individual land uses have another adjustment that can be applied to individual land use sub-areas.  The base RCN for AMC II for all 10 land uses can be found in the Fortran Code for V2.f90 in Lines 143-155.

Once the RCN adjusted for ASM in the UZ for the day is determined, the runoff is calculated using the classic Soil Conservation Service (1972), SCS equation:

R = (P – 0.2*S)2/(P + 0.8*S),

where, R = daily runoff depth, inches (if P > 0.2*S)

P = daily precipitation, inches

S = the maximum potential abstraction from P, defined by

(1000/RCN – 10), inches

The amount of water added to the UZ as infiltration then is simply P – R less any interception losses and soil evaporation when the WUC is 0.

During freezing conditions the precipitation is accumulated as a snowpack on the surface where it is subject to evaporative losses described later.  The snowpack melts when the average air temperature for the day is above freezing.  Then, the daily melt water is treated as rainfall, P, and the amount of runoff, R, is calculated.  The snowmelt routine is a simple energy balance method that uses a degree-day method coupled with the energy in the rain that falls on the snowpack.  Since the ASM is usually high when snow melt occurs, there are some periods with runoff from snowmelt.  Snowpack water content of more than three inches has been calculated in the study area.

Irrigation water can be applied to an HRU during the growing period for various crops.  Irrigation is scheduled based upon the ASM of the soil profile and the amount added each time that the ASM falls below the user-specified value.  Typically, a value of about 60% ASM is used.  Since irrigation may occur on a day with rainfall, there may be runoff from the irrigation.

To account for this case, half of the amount of irrigation water is added to the rainfall for the day and that increased amount of water is used in all of the calculations for interception and runoff calculations.  Only half is used because irrigation is not occurring on the entire HRU at the same time and parts of an irrigated field would have ASM lower than the calculated ASM that affects the RCN for the day.

Evapotranspiration

Evapotranspiration, defined as the sum of the evaporation from the soil and water surfaces and the transpiration from vegetation, can be expressed as potential and actual evapotranspiration.  Potential evapotranspiration is the loss of water to the atmosphere from a vegetated surface at a rate unlimited by the water supply and here Eto is the value used.  If the water supply is limited by the amount of water in the soil, the vegetation’s growth state, or by residue on the soil surface, the resulting amount of AET, may be less than the full amount of AET.   Also, if part of the Eto is used for evaporating water from the interception reservoir account (I), the Eto is reduced by the amount used there.

AET is calculated each day based upon conditions at the surface,

AET = (Eto – I) *WUC*Ks

Where, AET = actual evapotranspiration, inches

Eto = grass reference evapotranspiration, inches

I = amount of Eto used to evaporate water from the interception reservoir, inches

Ks = a soil-water coefficient based on amount of ASM in the soil zone, -

Actively Growing Vegetation Condition

For conditions when plants are growing a water-use-coefficient (WUC) is important.  The WUC is a function of the type of growing plants, the time of the year, and the geographical location in the study area.  It can range from 0 to over 1.0 for irrigated corn.  A value of 1.0 is equivalent to the conditions for which Eto is calculated.  Derrel Martin (2007) developed weekly values of the WUC for three zones in the study area – west, middle, and east using ______ procedure.  The weekly average values for all ten land use conditions are in 3 separate input data files in the Subdirectory Water Use Coefficient Files.  AET is also affected by the ASM in the soil profile.  In general terms, it the ASM is above 30% in the UZ and 50% in the LZ then the ASM does not limit AET and Ks = 1.0.  If both zones have adequate ASM, 40% of the daily AET is taken from the UZ and the remainder from the LZ.  Once the UZ is less than 30% ASM the unsatisfied demand is added to the demand from the LZ.  Then, Ks for the UZ decreases linearly from 1.0 to 0 as ASM decreases from 30% to 0%.   If sufficient ASM is in the LZ, the Ks for the LZ is 1.0 and all of the remaining AET is satisfied from the LZ.  If there is not sufficient ASM in the soil profile, Ks for the LZ drops off linearly as ASM from 1.0 to 0 as ASM decreases from 50% to 0%.

Non-Growing Season

During the part of the year when the WUC is zero, the process of soil-water evaporation occurs.  When the surface is not frozen, the loss is estimated by a technique for bare soil evaporation developed by Ritchie (1972).  It takes into account the wetness of the soil, ASM, in the UZ and a reduction in the amount attributed to residue on the surface is included.  When the soil is bare and wet enough so that capillary action can transport water to the surface, water is lost as fast as energy is added to the surface at a rate equal to Eto.  This is called Stage 1 evaporation.  The amount that can be lost during Stage 1 is a function of the type of soil.  For silt-loam type soils that were most of those included in this study, the value, U, is about 0.39 inches.  Once that amount is lost and not replenished, then the process, Stage 2, slows down as the soil dries and a rate that is governed by the amount of time since the soil was wetted is used because the ability of capillary action to transport water to the surface diminishes.  In both stages, the residue factor is applied.  The residue factor, a number between 0.2 and 0.5, is applied as a reduction factor times the calculated bare soil evaporation rate.  A value of 0.5 would be applicable for conditions immediately following harvesting of wheat or corn when the crop residue is still on the surface.  Monthly values for the residue factor for each land use are built into the current V2.  The user may change the values globally by the residue factor in the input file.

Snow Cover Condition

When snow is calculated to be on the surface in a snowpack, then evaporative losses are by sublimation of the snow.  The snowpack is calculated to be present when precipitation occurs and the average daily air temperature is below freezing.  Under these conditions, the snowpack on the surface evaporated at a rate equal of 50% of the Eto times 1. minus the residue factor.  When these conditions occur other water loss processes from the surface do not occur.  Since the sublimation loss process is water that did not enter the soil in the UZ, these losses are included in the interception-portion of the water-budget account.

Soil Water Movement

The soil water system for POTYLDR is a simple, two-zone model.  The UZ is one foot thick and is the active part that affects the RCN and evaporative processes.  Water is added to the UZ by infiltration of water from precipitation and from irrigation if the HRU is one that is irrigated.  No water moves upward from the LZ.  During the period when plants are actively growing, WUC greater than 0, the plants extract 40% of the daily water needed from the UZ as described above in the evapotranspiration section.  The UZ will hold all water added to it until the ASM exceeds 100%.  Then, the excess water added by infiltration into the UZ immediately percolates to the LZ before any water is removed by plant transpiration.  At the end of each day the ASM is updated.

The water holding characteristics of the soils are assumed to fall into one of 12 irrigation group classifications for Kansas (USDA, SCS, 1975) which define the water-holding characteristics of the soil layers and the soil-water evaporation characteristics for bare soil conditions.  Characteristics of the UZ follow those values.  The LZ characteristics originally followed the same characteristics as in the 12 irrigation classifications.  In this study, the water-holding characteristics of the LZ have been defined on a per foot basis.  This allows the user to use a LZ thickness or their choice.  Since much of the land uses of interest in this study were on dryland conditions and rooting depth was judged to be greater for dryland crops, a LZ thickness of 5 feet is used.

The LZ receives percolation from the UZ as its only source of water.  Water is lost by AET during the active growing season of the various crops and the amount of daily AET extracted is as described in the evapotranspiration section.  If the ASM of the LZ exceeds 70%, then drainage or deep percolation occurs.  The amount of deep percolation is the value used as ground-water recharge in the results of this study.  The typical understanding of soil-water drainage is that if the water content is less than field capacity equivalent to 100% ASM, that drainage is low.  The previous versions of POTYLDR used a constant ASM of 90% for the threshold above which all additional water added to the LZ would immediately be lost as deep percolation.  The author’s experience with soil-water content under storage-type terraces and with some modeling work that evaluates over-winter storage of soil water on irrigated lands in the study area showed that the soils that are typical of those in this study area drain down to an ASM of about 70% over long periods, several months.  So, a relationship between drainage rate and ASM was developed and used in this study.  This is relationship for the soils in this study is,

DP = (0.33*(ASM at 100% per foot)*LZ thickness)/45,000*(ASM-66.7)^2

where,	DP = deep percolation rate, inches/day

ASM at 100% per foot = available water holding capacity of soil, inches/foot

LZ thickness = the thickness of the lower soil zone, feet

The Figure 3 below shows the daily deep percolation rate from a silt-loam soil in the study area.

Typical Pond

V2 operates the water budget for a typical pond on a daily basis.

Inflow

Total inflow the typical pond is determined by the areal amount of each of the HRUs in the simulation that are found in the drainage area or watershed of the typical pond.  The area of each HRU is multiplied by the daily runoff depth from the HRU and all of them are summed.  Since the typical pond watersheds are small, all of the runoff from the entire watershed is assumed to reach the pond on the same day.  It is assumed that the only inflow is from surface runoff and from precipitation directly on the water surface of the pond.

Dimensional Characteristics

Figure 1 shows the schematic of the pond.  The dimensional characteristics of the pond is described by its bottom dimensions in feet for length (L) and width (W) plus a constant sideslope (SS), 1:SS, and the maximum height of the retention portion (V2) of the pond.  A spreadsheet is available for the user to choose L, W, and SS to match an actual stage-storage-surface area for a pond.  If the pond is filled beyond capacity, the overflow is spilled immediately on the day it occurs.

Figure 3.  Relationship between ASM and deep percolation rate for a typical deep silt loam

in the study area that was added to POTYLDR 2.0.

Characteristics for the reservoirs in the study area that were a part of the study were quite variable.  About one-third of the reservoirs had enough information about their dimensional characteristics so that the values for a typical pond at each weather station could be calculated for it and used in V2.  For reservoirs that were unique in one or more aspects, we had to consider them as a multiple of the typical pond.  This is discussed later in this manual.

Seepage Rate

The daily seepage rate, SR, from the typical pond in V2 was changed to be a function of the relative depth of water in the pond.  The daily seepage rate is directly related to daily depth, D.  At D = 0, the seepage rate, SRmin, is the minimum value and the seepage rate, SRmax, is the maximum value when D = V2.  In between 0 and V2, the daily seepage rate increases from SRmin to SRmax by a linear relationship,

SR = SRmin + (SRmax - SRmin) * D/V2

This relationship was developed in part by Choodegowda (2009) from study of several of the small reservoirs that were monitored by participants in the study and from results of recharge studies by Eisenhauer for some reservoirs in Nebraska both of which showed that the relative seepage rate was proportional to the depth of water in the reservoirs.  Several factors affect the seepage rate, but for the conditions in the study area constant values of SRmin = 0.1 inch/day and SRmax = 1.2 inch/day were used throughout the study area.  The daily amount of seepage is the daily seepage rate times the water surface area for the pond on that day.

The amount of the seepage that becomes ground-water recharge was also studied in detail by Choodegowda (2009).  Because most the reservoirs in the study area are seldom full and the sides of the storage area are usually covered with vegetation that can be quite deep rooted.  Even in the bottom of these reservoirs they can dry up and annual weeds can grow for some periods.  When these reservoir do fill the soil under the sides of the storage area have to be wetted sufficiently with seepage water before seepage below the rooting depth of the growing vegetation can begin to recharge the local groundwater.  From Choodegowda’s studies he concluded that about ten percent of the total seepage out of the typical pond would eventually be withdrawn by the growing vegetation and be lost as evapotranspiration.  So, the POTYLDR model estimates the net ground-water recharge to be 90% of the total seepage volume and the other 10% is added to the direct evaporation from the typical pond to produce the net evaporation portion of the water budget.

Evaporation Rate

The daily amount of evaporation from the typical pond is calculated as the Eto from the daily calculated surface area of the typical pond.  This rate of evaporation was examined by Choodegowda during his work, too.  Also, average annual amounts agree reasonably well with values published by the SCS for small reservoirs in the study area (Viessman and Lewis, 1996).

Overflow

Overflow occurs whenever the retention volume of the typical pond is exceeded.  The entire amount of overflow is dumped downstream on the same day it occurs.  V2 is not a stream-routing model.

Changing Watershed Conditions With Time

POTYLDR from 1993 included the option to input the amount of the various HRUs with time and the number of typical ponds in a small watershed so that it was more possible to describe the effects of changes with time and to allow comparison with an actual streamflow record.  Also, it uses a simple transmission loss relationship that is based on a relationship between annual precipitation and annual PET.  As stated previously V2 does not include those aspects.  Remnants of it remain in the original FORTRAN code in V2, but those parts are not relevant to the current work nor can they be accessed in V2.

SPECIFIC APPLICATION INFORMATION FOR THE REPUBLICAN RIVER BASIN STUDY

POTYLDR 2.0 (V2) forms the basis for the water balance simulation modeling for the Republican River Basin Study.  For the Republican River Basin Study, however, it was necessary to add additional elements to satisfy the needed results.  So, the following sections describe the overall simulation process to produce the results needed.

- Some Background

Koelliker served as the lead computer simulation modeling for the portion of the study and developed of the water balance model and for its application to land terraces and non-Federal reservoirs in the basin.  After review of several models and their usefulness, the POTential YieLD Revised model, POTYLDR, was selected.  Two additional add-on programs were developed for simulate daily operations of land terraces.  Also, work by Choodegowda (2009) that concentrated on the water budget operations of non-Federal reservoirs was adapted to the land terraces and that work also provided additional information on the water budget operations of the non-Federal reservoirs so that the pond or reservoir part of POTYLDR could be revised to simulate the water budgets of the non-Federal reservoirs directly in POTYLDR.

- Land terraces in the Basin are located essentially exclusively on those areas subject to sufficient runoff to produce erosion by such runoff or to capture the runoff for subsequent moisture to enhance crop dryland production where to runoff could be stored in the soil profile for use by growing plants.  These conditions and where terraces have been built occur almost exclusively on silt loam type soils on areas with mild to moderate slopes that with reasonable management can be used for crop production.  Subsequently, some of these same areas have been removed from crop production by various federal programs such as the Conservation Reserve.  Too, with the development of center-pivot irrigation technology some of the areas originally in dryland crop production are now under irrigation.  Terrace construction accelerated in the 1950s and 60s and it has continues at a slow pace now.   Fields in the Basin with terraces total about 2.15 million acres or about 15% of the Basin.  Of those fields, about 65% of the total area was found to be above

- The locations of the small federal reservoirs that are included in this study are more diverse in nature.  Again, the reservoirs are mainly in parts of the Basin that were judged to have sufficient runoff to provide some stored water for uses such as livestock or to be part of watershed projects that were built for some flood control.  In these areas, too, the dominant soil characteristics are mostly of the silt loam type and where there is sufficient slope to provide suitable locations for construction of the embankment.  Most of the reservoirs were built as part of federal programs such as the Great Plains Act following the drought of the 1950s or as part of other programs that encouraged water storage and flood control.  Very few reservoirs have been built recently.  The decrease in runoff from today’s crop production practices has been a factor along with fewer incentives to install more.  The reservoirs in this study were about 700.  The size of the drainage areas ranged from less than 0.5 to several hundred square miles.  Most, however, were nearer to two square miles.  Representing all of them individually was beyond the scope of this study.  They were treated as a multiple of a “typical” reservoir at each meteorological station and those characteristics were scaled for conditions in each HUC 12.  This approach provided a total affected drainage area for these reservoirs of about 1.2 million acres or a bit less than 10% of the active drainage area the Basin from which runoff into these reservoirs would likely be affecting streamflow.

- The parts of HUC 12s considered in the simulation modeling were only portions of each one and Figure 4 shows a simple representation.  The parts modeled are the areas 1) above the lowest terrace in the terraced fields a) without terraces on them condition, and b) with terraces on them condition; and 2) the drainage areas above small federal reservoirs, a) without terraces in them condition, and b) with terraces in them condition.  Those areas inside the black lined box that were not terraced and did not drain into a small federal reservoir were not modeled.  Those areas below the lowest terrace in the field and that were in the drainage area of a small reservoir were considered the same as the unterraced condition, but only the portions of that condition that were in the drainage area of a small federal reservoir was a part of the modeling process.

The study called for this study to produce results for three different scenarios to estimate the effects on streamflow at a sub-basin outlet and on the change in ground-water recharge within a sub-basin.  The three scenarios are

- Non-Federal reservoirs alone

- Land terraces alone

- Non-Federal reservoirs and terraces combined

The overall approach was to develop the results for 1) and 2) directly, but 3) required that the terraces in the drainage areas of reservoirs to be treated differently.  Those terraces reduce the inflow into the reservoir which in turn reduces the overflow from the reservoir somewhat and also reduces the amount of ground-water recharge from the reservoir.  So, the effect of terraces located above reservoirs on streamflow is less than for those that were not above reservoirs.  This necessitated a fourth condition or scenario to be simulated, 4) reservoirs with terraces in their drainage areas.  The location of terraces in the HUC 12 has no effect the amount of ground-water recharge from the terraces only.  It only affects the amount of ground-water recharge from the reservoirs.  So, to determine the results for 3) above the results from 2) above were scaled back to take just the runoff from the area of terraced lands estimated to not be above a reservoir and then add that amount to the amount of overflow from the reservoirs with terraces in their drainage area.  This accounted for the remainder of terraces in the drainage areas of the reservoirs to get the total runoff within the HUC 12.  The total ground-water recharge was the sum of the amount from all terraced lands in 2) plus the amount from 4) for the reservoirs with terraces in their drainage area.

Not terraced and does not drain into a small reservoir

Not terraced and does drain into a small reservoir

Unterraced land below lowest terrace in   terraced fields

Terraced land above lowest terrace in   terraced fields

- Figure 4.  Idealized HUC 12 showing those portions represented in the water-budget modeling process.

- SPECIFICS OF APPLYING THE MODELING TO THE REPUBLICAN RIVER BASIN STUDY

- Before presenting the details of the modeling process the user should review of Figure 5a and b to examine how all of the parts of the fit together.  Then, the details of the individual parts follow.

- STEP 1 -- V2 serves as the water-balance model.  Its main purposes are to

- Simulate a daily water balance for each of the important Hydrologic Response Units (HRUs) in the Basin that are important to predict the effects of land terracing on streamflow and ground-water recharge

- Produce daily output values that can be used to predict the effects of land terracing on runoff and ground-water recharge e effectively than by using just the runoff curve number method in the regular POTYLDR program

- Simulate the operation of a typical small federal reservoir without and with terraces in its drainage area to determine the effects of the reservoirs on streamflow and ground-water recharge at the reservoir location.

The study called for this study to produce results for three different scenarios to estimate the effects on streamflow at a sub-basin outlet and on the change in ground-water recharge within a sub-basin.  The three scenarios are

- Non-Federal reservoirs alone

- Land terraces alone

- Non-Federal reservoirs and terraces combined

The overall approach was to develop the results for 1) and 2) directly, but 3) required that the terraces in the drainage areas of reservoirs to be treated differently.  Those terraces reduce the inflow into the reservoir which in turn reduces the overflow from the reservoir somewhat and also reduces the amount of ground-water recharge from the reservoir.  So, the effect of terraces located above reservoirs on streamflow is less than for those that were not above reservoirs.  This necessitated a fourth condition or scenario to be simulated, 4) reservoirs with terraces in their drainage areas.  The location of terraces in the HUC 12 has no effect the amount of ground-water recharge from the terraces only.  It only affects the amount of ground-water recharge from the reservoirs.  So, to determine the results for 3) above the results from 2) above were scaled back to take just the runoff from the area of terraced lands estimated to not be above a reservoir and then add that amount to the amount of overflow from the reservoirs with terraces in their drainage area.  This accounted for the remainder of terraces in the drainage areas of the reservoirs to get the total runoff within the HUC 12.  The total ground-water recharge was the sum of the amount from all terraced lands in 2) plus the amount from 4) for the reservoirs with terraces in their drainage area.

- Figure 5a.  Complete operation to simulate effects of land terraces and small federal reservoirs on runoff and ground-water recharge in the Republican River Basin.

- Figure 5b. Complete operation to simulate effects of land terraces and small federal reservoirs on runoff and ground-water recharge in the Republican River Basin.

So, in total it required 192 (32X3X2) total runs of the V2 model for a complete simulation to get output for the entire basin for further processing.  All results from the water-balancing modeling simulations were reported in units of acre-feet/square mile so that they could be scaled up for each HUC 12 by using the amounts of conditions for each scenario in each HUC 12.  Note:  53.33 acre-feet per square mile is equal to a depth of 1.00 inches.

The length of the daily record for 31 of the meteorological stations was 59 years, 1950-2008, except for Sedgwick, NE which had the same information for 1952-2008.  The input information for each HRU and for each reservoir is described in detail later.  The simulations from V2 provided output files that were used initially to populate partially an Excel spreadsheet with results that has 96 rows, one row for each of the three soil types at each of the 32 meteorological stations.  Direct results from V2 provided values of long-term average annual amounts of evapotranspiration, runoff, and ground-water recharge from each land use without terraces and long-term average annual amounts of inflow, overflow, net evaporation, and ground-water recharge for the non-Federal reservoir with no terraces in its drainage area.

Within the V2 simulation model the operation of a small reservoir was done.  It used daily outputs from the HRUs that were representative of the type of and amount of area of the land use conditions in the watershed for the inflow to the reservoir.  The characteristics of the reservoir were represented by a stage-storage-area-discharge relationship, an estimated seepage loss rate under its daily water surface area that was a function of the depth of water in the reservoir, evaporation from the water surface area at the Eto rate, and precipitation onto the water surface.  The previous work by Choodegowda (2009) was used to determine the seepage rate function and that work also determined that about 90% of the seepage from the reservoir would become ground-water recharge and the other 10% was lost as evapotranspiration from the areas of the reservoir that were periodically inundated.  Additional discussion of the reservoir modeling aspects can found in in Sensitivity Analyses for the Republican River Basin Study, Reservoir Analyses in the final study report.

The detailed operation of the V2 water balance model has been discussed previously in this manual.  The following is a presentation of the input information needed for a run of V2 and the output files that it produces.

NOTE:  All files presented here are also on the media included contains this manual and all files produced by a run of V2.  The contents of media for this model are listed in Appendix A.

The user may find a simple editor such as Notepad ++ that is available at no cost best for reading and editing files for V2.  It can be found at http://notepad-plus-plus.org .

Example Input File:  'cambridge5nt.dat'

'Cambridge, NE Soil 5 + Reservoir w/No Terraces'

'cambridgepcp.dat' 'cambridgetmp.dat' 'cambridgeEToPM.dat'

'cambridge5NT.OUT' 'WUCmiddle' 'cambridge5NT' '251415 05'

0.10 1.20 15.2 25. 25. 21.0 10000. 05

251415 251415 251415 1950 2008

20 02 0.93 1.00 1.00 5.0

Explanation of Input Items in the Above File: NOTE:  Items in italics may be selected by the user.  Those not in parentheses are specific input file names for datasets.

Input Filename:  cambridge5nt.dat Name of 1 of 32 meteorological stations in the Basin.  This version is for Soil Type 5 and has no terraced areas draining into the pond.

String for Output File Heading:  Cambridge, NE Soil 5 +Reservoir w/No Terraces

Printed Output File for the Simulation: cambridge5NT.OUT

Files used for Daily Values:  Any of the files (XXXXX pcp, tmp, or EToPM) can be used with any station if desired by the user

Precipitation:  cambridgepcp.dat

Minimum and Maximum Temperature:  cambridgetmp.dat

Reference ETo:  cambridgeEToPM.dat

Output Filename for output from the simulation in tabular form:  cambridge5NT.OUT

Input Filename for Weekly Land-use Water-use Coefficient:  (WUCmiddle)

Choice of 3 files for east, middle, and west parts of the Basin

Output Filename Used to Identify Other Output Files from Original POTYLDR:

cambridge5NT Output file contents are presented in the Output File Contents Section.

Output files produced are for surface runoff, pond (reservoir), percolation, and irrigation aspects of the simulation run

Identifier for Output Filename for Station HRUs and Pond (reservoir) Average Annual Values:  251415 05  The 6-digit identifier for the meteorological station and 2-digit soil type identifier.  A W is added to the name if the output has values for the reservoir with terraces in it drainage area.  These are used in subsequent processing.

Minimum Daily Seepage Rate from Pond:  0.10, inches when pond is a 0. Depth, not empty

Maximum Daily Seepage Rate from Pond:  1.20, inches when pond is a maximum depth

Maximum Depth in Pond at Primary Spillway:  15.2, feet

Bottom Width and Length of Pond at 0. Depth:  25. 25., feet

Side slope, Run to Rise for Pond Area:  21.0

Total Area Represented by the POTYLDR Run:  10000.  acres.  Not used for Basin Study

Output Control Code for POTYLDR Output:  05 equals annual output of values, 06 is for monthly output.  These are the only two values used for the Basin Study.

Station Identifiers for the Precipitation, Temperature, and ETo Files:  251415 251415 251415  These are all for Cambridge.  They serve as a check to make sure the contents of these 3 input files agree with the Station Identifiers for the run.  If they do not match the Station ID in the particular data sets, the simulation will not run.

Year the Simulation Run Starts and Ends:  1950 2008  Any period can be used if it is contained within the input data files.

Number of HRUs in the Run:  20  Can be up to 24.

Number of HRUs Irrigated in the Run:  2  Can be up to 24.

Coefficient for Adjust Built-in Monthly Runoff Curve Numbers for Condition II for the Run from the Standard Monthly Values:  0.93 This value adjusts all HRUs.  Usual values are 0.93, 0.92, and 0.90 for Soil Types 5, 7, and 11, respectively.

Coefficient to Adjust Built-in Monthly Residue Factors for the Run from the Standard Monthly Values for all HRUs:  1.00

Coefficient to Adjust Built-in Weekly Water-Use Coefficients from the Values in Input File, WUCxxxxx for all HRUs for the Run:  1.00

Thickness of the Soil in the Lower Zone for the Run:  5.0, feet  Same value used for all soils in the Republican Study.

POTYLDR 2.0 (V2) Input for Each Individual HRU

Example:  03 05 1 2 3 1 2 3 02. 2 1.00 One line for each HRU.

No. 1-20  03 20 HRUS are used in the Basin Study

Soil Type:  05 1 thru 12 – Only Soils 5, 7, or 11 are used in Republican Basin Study

6-Year Land-use Rotation:  1 2 3 1 2 3 as 6 values, 1 thru 10

1 Wheat Plant 	2 Wheat Harvest	3 Dryland Corn	4 Sorghum

5 Soybeans	6 Hay/Forage	7 Grass/Pasture	8 Fallow

9 Continuous Wheat	10 Irrigated Corn

Area of HRU, acres:  02. defines the amount that might be in the Reservoir Drainage Area

Does HRU Drain into the Reservoir?:  1.  1= No, 2 = Yes, Does Drain in Reservoir

Factor for Runoff Curve Number Adjustment for the Particular HRU:  1. LT 1, for terrace effect on runoff, 1. = No terraces, GT 1.0 could be used for other effects on runoff if desired

Output Files Produced by a General V2 Run

The examples given below are for the Example Input File, cambridge5nt.dat

REMINDER:  All output files from this example are on the media included with this manual.

Main Output File for General POTYLDR:  cambridge5NT.OUT This is the original file information that was in mainframe format that has been reformatted to generally fit a standard page.  It echoes the input information, shows the contents of several input files, describes the characteristics of the pond, and list the input information for each HRU.  Next, it shows the annual water balance for the pond and each HRU for the entire run. A Final Summary follows that provides average annual information for the meteorological station and each HRU.  Finally it shows a summary of the frequency distribution of daily precipitation and daily runoff amounts for the first four HRUs in the Run. It is kept, but it is overwritten if the same filename is used again.

These output files are not used in the Basin Study:

XXXannlsurf.out, XXXannlpond.out, XXXannlperc.out, and XXXannlirrg.out.  Here, XXX is an identifier for the meteorological station, soil type, and terrace status in the reservoir drainage area; example cambridge5NTannlsurf.out. They are simply for those with additional interests in other aspects and are maintained from POTYLDR. These files can be output as monthly amounts, too.  They are overwritten by any run that specify output files by the same name.  They have no headings on the columns of data, but the following provide those headings:

FILENAMES:  STATIONannlsurf.out OR STATIONmnthsurf.out

The file contains information for

YEAR, (MO) PRECIP, LAKE EVAP, NET RESERVOIR EVAP, RESERVOIR OVERFLOW,

AVERAGE END-OF-MONTH % FULL FOR THE YEAR

FOLLOWED BY PRECIP EXCESS FOR EACH HRU 1 to N.

HEADINGS FOR THE OUTPUT ARE AS FOLLOWS:

YEAR (MO) PRECP LKEVP NETEVP OVRFL %FUL  RUNOFF FROM ALL HRUS WITH

UNITS OF INCHES EXCEPT FOR % FULL AND NETEVP & OVRFL ARE IN INCHES

OF DEPTH OVER THE DRAINAGE AGEA OF THE RESERVOIR

FILENAMES:  STATIONannlperc.out OR STATIONmnthperc.out

The file contains information for

YEAR, (MO) RESERVOIR SEEPAGE, DEEP PERCOLATION FROM EACH HRU.  HEADINGS ON THE OUTPUT FILE CONTENTS ARE AS FOLLOWS:

YEAR (MO) PONDSEEP IN INCHES OVER THE RESERVOIR DRAINAGE AREA FOLLOWED

BY PERCOLATION IN INCHES FROM EACH HRU.

FILENAMES:  STATIONannlirrig.out OR STATIONmnthirrig.out

The file contains information for YEAR (MO)  FOLLOWED BY MONTHLY IRRIGATION DEPTH IN INCHES FOR EACH HRU.

DAILY.OUT:  This one contains the daily water balance information for each of the HRUs for the last year of the simulation run.  It is not used directly in the Basin Study.  It contains headings for the information in the table and it is overwritten by each run of V2.

HRU1 through HRU10:  These output files, one for each of the 10 HRUs, HRU1, HRU2,…. for the with no terrace condition, contain the meteorological station ID and HRU soil type followed by a row for each day of the run.

The contents of the file have the following output headings: Year, Mo, Day, Precip, Runoff, Percolation, Water-Use Coefficient, and Residue Factor.

The last line of each one is  999,0,0,0,0,0,0,0,0,0,0 to signify the end of the record.  These are input files to the program, TERRACEPOND.

ONE and TWO:  These are temporary output files that contain the meteorological station ID, Soil Type, and average annual water balance values -- evapotranspiration, runoff, and percolation in acre-feet per square mile – for further processing.  Each run of V2 overwrites these files.  The contents of these files are used in subsequent batch file processing by CNSLD before the next run of V2 for the next set of conditions is executed.  ONE is for no terraces in the pond drainage are and TWO is for terraces in the pond drainage area.

The contents of the files are a string of values that are as follows:

STATION ID, Soil No., Precipitation, Ref ET, followed by 20 sets of

values in acre-feet per square mile for ET, Runoff, and Percolation

for each HRU.  Finally, the values for the typical pond are shown as

follows: drainage area in square miles followed by inflow, overflow,

net seepage, and net evaporation all in acre-feet/square mile, then

square miles of terraces in the drainage area of the pond and finally

the average percent full for the pond.

251415 05:  The same as ONE, but by a unique name and contains values that might be of interest for other purposes for with no terraces in the drainage area of the pond.

251415 05W:  The same as TWO, but by a unique name and contains values that might be of interest for other purposes for with terraces in the drainage area of the pond.

Once V2 has been executed for a run for a station, the necessary files needed by TERRACEPOND will be available in the same directory in which V2 was executed.

STEP 2 – TERRACEPOND (Identified as PB)

Background Information

The POTYLDR program has always simulated the effect of land terraces on runoff and percolation by using reduced runoff curve numbers for terraced lands compared to unterraced lands.  This method has been satisfactory for general results that are most concerned about their effects on runoff and streamflow.  It this study, however, an important aspect is to estimate the effects of terraces as small reservoirs so that both the effect on runoff and percolation is treated more thoroughly and realistically.  This is of particular importance in the Basin because many of the terraces are the storage types that retain runoff for subsequent loss by evaporation, evapotranspiration, and ground-water recharge.

The work done by Choodegowda (2009) to model the daily operation of the small federal reservoirs in the study area provided a basic framework to treat a terrace as a reservoir or pond.  In Choodegowda’s work, the reservoir was modeled as a series of level sections and each level was operated depending on the water level or stage in the reservoir.  If the stage was above the principal spillway level or overflow the excess was lost downstream.  If the water level was a lower stage, evaporation from the surface was estimated by the ET0 for the day and infiltration for the day occurred into all of the levels that were inundated.  In that work, different infiltration rates were used based upon location in the reservoir and the hydraulic head on the level section.  Each day the water level was updated to start the next day’s calculations. Thus, the amount of water infiltrating into each level section that was inundated was determined.  Also, the amount of the infiltration that moved below the rooting depth for each level section was estimated by simulating each level as a land-based HRU that was periodically flooded.  That part of the terrace simulation is discussed in the next section.

The terrace channel was assumed to be similar in characteristics to produce runoff as the field and was a part of the HRU if there was no water in the terrace channel.  If there was water stored in the channel at the time of a runoff event, the area of the water was subtracted from the total field to calculate the runoff into the pond.  All of the precipitation that fell on the water surface was added directly to the total volume added into the storage volume. The volume was converted into a depth in the channel.  If the depth was greater than the overflow level, all of the excess was lost as runoff on the day it occurred.  All infiltration into the terrace channel on each layer was assumed to move only vertically in the soil beneath the level.  Previous field work and sampling indicated this was a good assumption for the soils and conditions in the study area.

Water that remained in the channel was lost by evaporation at the reference ET rate, ET0, from the surface and by infiltration into each level that was inundated.  If a level was inundated, there was not AET from it. The amount lost by infiltration from each level was 0.5 inches per day from Levels 1 and 2 and 1.0 inches per day for all of the other levels for both types of terraces.  For those levels that became inundated on a day of a runoff event, the total infiltration into them was the amount of infiltration from the precipitation event plus the daily amount of infiltration of the particular level.  If the water depth over a level after evaporation was subtracted was less than a full day’s infiltration amount, the amount of infiltration for the day was limited to amount that was there to infiltrate.

Specifics for Terraces

Figure 6 below shows the characteristics of the typical broadbase terrace cross-section (not to scale) used to represent all of that type in the Basin.  The information to develop this cross-section and for the flat-channel terrace cross-section came from Eisenhauer based upon the terraced land field survey.  Figure 7 below shows the characteristics of the flat-channel terrace cross-section (not to scale) used to represent all of that type in the Basin.

Figure 6.  Terrace cross-section (not to scale) used to represent broadbase terraces.

Figure 7.  Terrace cross-section (not to scale) used to represent flat-channel terraces.

To input the information about the terrace type, storage volume, and infiltration rate used in each level required an input file for each of the five terrace types simulated.   The input for file for a terrace type, closed-end broadbase shown here, was a simple ASCII file of this type,

filename, LCBBTERRACEDATA.DAT.

Table 1.  Example input file, LCBBTERRACEDATA.DAT, to represent the level, closed-end broadbase type.

08  	Level No. at overflow

1333 	Terrace length, feet

165 	Terrace spacing, feet

1   0.0     0.1     0.5  Level No.; height from bottom, inches: surface width, feet; and

2   3.0     1.0     0.5  	    infiltration rate, inches/day

3   5.0     3.0     1.0

4   7.0     5.0     1.0

5   9.0    10.0     1.0

6  10.5    15.0     1.0

7  11.8    19.0     1.0

8  13.2    24.0     1.0

9  15.0    30.0     1.0

10 18.0    38.0     1.0

11 21.0    45.0     1.0

12 24.0    55.0     1.0

13 27.0    65.0     1.0

14 30.0    85.0     1.0

15 33.0    95.0     1.0

16 36.0   105.0     1.0

All the data files for all five terrace types must be available in the same file directory as the executable file, CB.

All three types of broadbase terraces used this same cross-section shape.  The only difference was that the level, closed-end type had the overflow set at Level 8, the level, open-end had the overflow set at Level 6, and the graded one had the overflow at Level 4.  For the flat-channel type of terraces, these same characteristics are shown below under the heading Flat-channel Terrace Characteristics in Table 2 and in the first three columns with the exception of the infiltration rate in the various levels which were the same as for the broadbase type above.

The same cross-section shapes were used throughout the simulations because the general cross-sectional shape of terraces is mostly determined by how farming equipment will fit them.  The actual amount of storage that was used for a particular situation was set by choosing the level at which overflow from the storage would occur for all types of terraces simulated.

Based upon the results of the field survey of terrace types and conditions a distribution of the amount of storage for each type of terrace was developed and reported elsewhere by Eisenhauer.  Several simulations with the various sizes of storage capacity found in the field were done.  The results of that work concluded that the best single storage capacity for closed-end type terraces that produced essentially the same amount of runoff reduction over the long-term was 0.48 inches (Level 8) for the broad-base type and 0.99 inches (Level 10) for the flat-channel type.  Some of the closed-end types of terraces had no storage because of being breached or other reasons.  For the simulation of their effects on runoff they were assumed to be level with open ends and treated the same as described in the next paragraph.

To simulate the operation of level terraces with open ends, the amount of storage was set at 0.20 inches (Level 6) and for the flat-channel type it was set at 0.25 inches (Level 6).  These terraces require that accumulated runoff build up enough hydraulic head to cause the water to flow out and the rate of outflow is low because of the flat channel slope.  So, infiltration opportunity time is long and some surface evaporation occurs during the drain out process.  To simulate the operation of the graded broadbase type, the only type of graded terrace used, the amount of storage was set at 0.05 inches (Level 4).  Here, again, the terrace increases the opportunity time for infiltration of some runoff.  Also, the effects on the amount of runoff reduction by the graded terrace simulations were similar to the reduction in runoff that is produced when the Runoff Curve Number is reduced to account for land terracing.

The PB program is written in FORTRAN 95.  The program produces the dispensation of water added to the terrace channel that treats the channel as a small reservoir with no plant growth in the reservoir.

The program reads the five terrace characteristics files and creates the stage-storage-volume , and infiltration rate tables similar to those in Table 1.  It then reads an HRU# file previously created by

V2 to receive the inputs of year, month, day, ETo, precipitation, runoff, WUC, and residue factor.  It then uses the ETo and runoff as required input to operate the terrace channel storage similar to a small reservoir to calculate the amount of water in storage, the overflow amount, infiltration into each level, evaporation loss from the surface, and change is storage volume each day for each of the five terrace types that could have been on each of the HRUs 1 through 10..  It produces 10 output files for each terrace type, TP 11, 12, 13,…TP11 for terrace type 1  through TP 51, 52,53,….TP510

for terrace type 5.  These files contain year, month, day, ETo, precipitation, depth of infiltration into each level each day for all levels simulated, WUC, residue factor, depth of runoff, inflow volume, and overflow volume.  These 50 output files provide the subsequent input files for processing with TERRACECHANNEL to predict the dispensation of the water that was not lost by overflow and evaporation from the ponded water surface.  These files are overwritten by subsequent runs of TP so they must be read by TERRACECHANNEL before the next time TP is executed.

STEP 3 -- TERRACECHANNEL

The TERRACECHANNEL program is written in FORTRAN and is saved under the shortened name, CB, for easier use in the batch processing file sequence.

The program operates a one-dimensional daily water balance on each of the level areas of the terrace cross-section to produce the amounts of actual ET and percolation or ground-water recharge from each level, sums the results for both on each of the five terrace types for each of the 10 HRUs.  It is based on the same principles developed and used by Choodegowda (2009)) to estimate the effective recharge from small reservoirs.  It produces, and then calculates the average annual results for each

Table 2.  Characteristics of the two terrace types used to estimate the amount of overflow

from and infiltration into the levels in the terrace channel.

Broadbase Terrace Characteristics

Drainage area, acres = 5.05

Terrace spacing, feet =165

Terrace length, feet = 1,333

Lev. Depth, Width, Increment Accumul. Increment  Accumul.   Runoff storage

No.  in.    feet   area, ac  area, ac Vol. ac-in Vol., ac-in depth, in.

1   0.0    0.1     0.0031    0.0031   0.0000     0.0000     0.00

2   3.0    1.0     0.0275    0.0306   0.0092     0.0092     0.002

3   5.0    3.0     0.0612    0.0918   0.0612     0.0704     0.014

4   7.0    5.0     0.0612    0.1530   0.1836     0.2540     0.05

5   9.0   10.0     0.1530    0.3060   0.3060     0.5600     0.11

6  10.5   15.0     0.1530    0.4590   0.4590     1.0190     0.20

7  11.8   19.0     0.1224    0.5814   0.5967     1.6158     0.32

8  13.2   24.0     0.1530    0.7344   0.8140     2.4298     0.48

9  15.0   30.0     0.1836    0.9180   1.3220     3.7517     0.74

10  18.0   38.0     0.2448    1.1629   2.7541     6.5059     1.29

11  21.0   45.0     0.2142    1.3771   3.4886     9.9944     1.98

12  24.0   55.0     0.3060    1.6831   4.1312    14.1256     2.80

Flat-channel Terrace Characteristics

Drainage area, acres = 13.66

Terrace spacing, feet =293

Terrace length, feet = 2,030

Lev. Depth, Width, Increment Accumul. Increment  Accumul.   Runoff storage

No.  in.    feet   area, ac  area, ac Vol. ac-in Vol., ac-in depth, in.

1   0.0     0.1    0.0047    0.0047   0.0000     0.0000     0.00

2   1.5     4.0    0.1817    0.1864   0.0070     0.0070     0.01

3   2.7    10.0    0.2796    0.4660   0.2237     0.2307     0.02

4   3.9    18.0    0.3728    0.8388   0.5592     0.7899     0.06

5   5.1    27.0    0.4194    1.2583   1.0066     1.7965     0.13

6   6.4    34.0    0.3262    1.5845   1.6357     3.4323     0.25

7   7.6    44.0    0.4660    2.0505   1.9014     5.3336     0.39

8   8.8    47.0    0.1398    2.1903   2.4606     7.7942     0.57

9  10.0    52.0    0.2330    2.4233   2.6284    10.4226     0.76

10  11.3    58.0    0.2796    2.7029   3.1503    13.5729     0.99

11  13.0    64.0    0.2796    2.9826   4.5950    18.1679     1.33

12  15.0    74.0    0.4660    3.4486   5.9651    24.1330     1.77

13  18.0    84.0    0.4660    3.9146  10.3457    34.4788     2.53

14  24.0   104.0    0.9320    4.8466  23.4876    57.9664     4.25

of the seven land uses.  CB reads in the output files from PB that provides the basis for the daily operation of the terrace channel as a small reservoir.  It provides the ground-water recharge estimate for the terrace, AET, and change in soil water content.  It produces the estimate of the long-term average water balance for the area above a terrace and scales those to the desired units of acre-feet/square-mile per year.

The execution of CB completes the three steps to produce information for each land use, terrace operations, and reservoir operations.  To convert these simulation results to represent conditions in each HUC 12 required further processing and consolidation of these simulations into estimates.

One more intermediate step was needed to get input results for final processing for the terraces within the drainage areas of the non-federal reservoirs.  The terraces reduced the runoff from areas above terraces.  So, the V2 inputs had to be modified to account for the effects of the terraced areas on surface runoff into the reservoirs in a second run for each of the 32 stations and 3 soil types (96).

The results of the simulations for the effects of terraces with CB provided good information to develop the reduction factors.  Average results for simulations at four meteorological stations in the most heavily terraced areas in the central part of the basin for storage type terraces gave similar results for each type of terrace.  The stations were Colby and Dresden in Kansas and McCook and Cambridge in Nebraska.  A similar set of results for graded terraces using four stations in the eastern portion of the basin resulted, too.   The stations were Superior, Red Cloud, Harlan County Lake, and Holdrege all in Nebraska.  Table 3 shows these results.

The terraced areas in the reservoir drainage areas were assumed to have the same effect on ground-water recharge as those not above reservoirs.  To provide the needed input into the V2 model for the terraces in drainage areas of the typical reservoir, an adjustment factor that was applied to the runoff curve number for the terraced areas was developed that produced a similar amount of reduction in the long-term average runoff for the terraced areas for each type of five terrace types.  These values are shown in the right-most column in Table 3.  Storage type terraces have a lower value than does

Table 3.  Factors used to represent land terraces in the area above small reservoirs

to account for the effect of the terraces on surface runoff.

|  |  |  |  |  | Curve |
| --- | --- | --- | --- | --- | --- |
|  | Storage |  |  |  | Number |
|  | Capacity, | Runoff reduction, percent |  |  | Adjustment |
| Terrace type | Inches | High | Low | Average | Factor |
| Flat-channel, closed | 0.99 | 90 | 88 | 89 | 0.73 |
| Broadbase, closed | 0.48 | 77 | 75 | 76 | 0.75 |
| Flat-channel, open | 0.25 | 61 | 58 | 59 | 0.88 |
| Broadbase, open | 0.20 | 54 | 52 | 53 | 0.90 |
| Graded | 0.05 | 25 | 21 | 23 | 0.96 |

the graded terrace.  The approximate average runoff curve number for unterraced cropland in the area is 72.  The corresponding value for broadbase closed-end is 54 while for the graded terrace the value is 69.

A spreadsheet was used to calculate the best estimate of the amount of each land-use, terrace type near each meteorological station and it calculated the weighted runoff adjustment factor to be used for the terraced land uses in the drainage area of the typical reservoir at each station.  These values had to be entered manually into each input file for V2 for the with-terraces in the reservoir drainage area condition.

The V2 simulations for the with-terraces in the drainage area of the reservoirs then were able to produce the long-term average results for the water budget of the typical reservoir with terraces in its drainage area.  These results did not need to be processed through the PB and CB because the effects of those terraces on ground-water recharge had already been calculated.

Once all of the results were judged to be useable, a batch file process was developed to simulate the entire basin and to write the overall results from the water-budget simulations into a master file for post-processing.  A simple FORTRAN program, CNSLD, was used after each set of two simulations for a single soil type without and with terraces in the drainage area of the reservoir.  It read the required output from each of the two simulations and wrote a single row of output to the ASCII file, PSR, for subsequent input to the post-processing program.  The consolidated file, PSR, had a total of 96 rows of information.

For a full simulation of the length for the 59-years of meteorological data at all 32 stations and for all three soil types required about eight hours of simulation time to process.   Figures 5a and 5b shows the flow chart of the water budget simulations with POTYLDR and the add-on programs in the sequence in the batch file run.

STEP 4 – CNSLD, Consolidation of Simulation Results

Two complete sets of simulations runs of V2 are required to do a comparison of the impact of land terraces and small federal reservoirs.  One complete set of runs consists of a simulation at each of the 32 meteorological stations for each of the three soil types used for a total of 96 runs.  This first set is for the case with no terraces in the drainage area of the reservoir.  The second set of simulations is the same number of runs, 96, but with revised areas of HRUs and adjustment of the Factor for Runoff Curve Number Adjustment on those HRUs that represent the parts of the drainage area of the typical pond that are above terraces in the drainage area of reservoir.  This second set of runs requires that the user develop the Factor for Runoff Curve Number Adjustment for each location and terrace types and amounts at the meteorological station in a spreadsheet.  Then, the amount of area of each HRU in the drainage the reservoir must be entered along with the calculated Factor for Runoff Curve Number Adjustment.  This requires that all 96 individual input files to be edited by the user.

As such, these results are not in a form that can be processed easily to translate the results into the HUC 12s in the study area.  So, another FORTRAN program, CNSLD, was written to consolidate the results from each simulation of the first three steps into a single string of results each of which include the result for the simulation at a meteorological station and soil type that can be input into the final step of the process.  As shown in Figure 5b, it simply reads the output from CB and from the two simulations by V2 to extract the results for each land use on each soil type for the with and without terraces conditions along with the results for the operations of the small reservoir with and without terraces in its drainage area to produce the single line of results.  CNSLD adds these lines to the output file, PSR to complete Step 4.  All of the output values from CNSLD are provided in units of acre-feet/square-mile so that the simulated amounts in each HUC 12 can be determined based upon the amount of area of each applicable condition within the HUC 12.  The contents of PSR are as follows:

(The same information from the results of the computer simulations at each the 3 nearest meteorological sites and for each of the three different soil types at each meteorological station produces a total of 9 sets of the following info is provided in a continuous string for each of the 9 sets)

Station ID

Soil type

Five sets by terrace type – closed-end, level, broadbase; closed-end, level, flat channel; open-	end, level, broadbase; open-end, level, flat channel; and graded broadbase

Seven subsets by land use – continuous dryland corn, continuous dryland wheat, wheat-corn-fallow, wheat-fallow, hay/forage, range/pasture, and irrigated corn

One set of increase in evaporation (+ET) in the field from the terrace channels, decrease in runoff (-RNOF) at the edge of the terraced field, and increase in percolation (+PERC) under the terrace channels for each of the 35 terrace by land- use combinations, all values in average annual ac-ft/sq mile

One set of values for reservoirs without terraces – inflow, outflow, percolation, net ET, all values in average annual ac-ft/sq mile

One set of values for reservoirs with terraces – inflow, outflow, percolation, net ET, all values in average annual ac-ft/sq mile

Estimated portion of land above terraces in the reservoir drainage area, --

STEP 5 – POST, Post-Processing Results For Each HUC 12

The output file from CNSLD provides results in the file, PSR, is in a volume per unit area basis. A GIS pre-processor framework was used to define each separate geographical area (HUC 12) and to extract needed characteristics of each area from GIS coverages.  The information extracted includes the HUC 12 identifier, amount of terraced land, stream length to the outlet of the sub-basin from the centroid of the HUC 12, total final drainage area of reservoirs,  percent of three soil types under terraced lands, percent of seven land uses in terraced areas, percent of five terrace types, weighted fraction and identification number for the three nearest meteorological stations, estimated transmission loss factor and the sub-basin in which the HUC 12 is located.  All of this information was extracted and put in an Excel spreadsheet with one row for each HUC 12.  The data portion of the spreadsheet was copied into an ASCII file, HUCDAT that had one row of information for each HUC 12.  This work was done by a combination of personnel at the University of Nebraska-Lincoln and Bureau of Reclamation personnel.  This work provided much of the necessary input information for the various conditions simulated.  The process of obtaining the input is presented in the final report for the project.  The spreadsheet with the values is located in the Subdirectory Excel Spreadsheets in file, Post-processing Input Data for HUC 12s FULL SET_Updated_8_25 with NE large reservoirs.  The HUC 12 Information Required for Final Processing is as follows:

Location Info

HUC 12 Identification No. (last 6 digits only)

Station ID for the 3 nearest meteorological stations (no units)

Decimal weight for each Station ID (sums to 1.00)

Stream Travel Distance from the centroid to outlet of the sub-basin (miles)

Physical Characteristics:

Total area (acres)

Total area of terraced lands (acres)

Total area above small federal reservoirs (acres)

Percent of each of 5 terrace types (sums to 100)

Percent of each of 7 land uses on terraced fields (sums to 100)

Percent of 3 major soil types under terraced fields (sums to 100)

Transmission loss factor for the stream (% per mile of length)

Percent of terraced fields above the lowest terrace (65% for all areas)

The POST computer processing program written in FORTRAN, reads from PSR the about input items for the entire basin in ASCII strings.  It then operates on the HUC 12s one at a time.  It sorts through the PSR file contents to locate strings associated with each of the three meteorological station assigned to the HUC 12 and extracts the three strings of information for each soil type associated with each meteorological station.  It then makes calculations based upon the amount of each applicable condition weighted by the amount of the HUC 12 represented by each of the three meteorological stations to produce an output string for each of the HUC 12s in the Basin in the output file, FINALRESULTS.  Additionally, it calculates the transmission loss amount for each HUC 12 based upon its distance to the outlet of the sub-basin in which the HUC 12 is located using the Transmission Loss Factor assigned to the HUC 12.  Table 3 shows a portion of the output in this file.  The output is arranged into four separate subsets to allow for the three following scenarios to be evaluated in each HUC 12 and to be summed for particular sub-basins.  The process that the POST program uses to produce the four different subsets of results is described below.

For the Terraces Only Scenario, for each HUC 12 the 3 meteorological stations are located in the PSR file and the information for each of the 3 soil types are collected in an array that contains all of the output values from the water balance simulations that are stored in the file, PSR. The, the values for the changed in the water balance for the terraced land only are summed based upon the amount of area for each soil type by meteorological station, 9 combinations, to get the total amount of change in NET ET, RNOF, and RECHARGE for each HUC 12.  The summed value RNOF is then divided into change in SURF RUNOFF (streamflow at the outlet of the sub-basin) and TRANS LOSS (the part of RNOF that is estimated to have been lost between the field and the outlet of the sub-basin during the streamflow process) based upon the stream travel distance to the outlet of the sub-basin and the percent loss per mile of stream length for each HUC 12.

For the Reservoirs Only Scenario, the input data array is searched to collect the total amount of change in Inflow, Outflow, Percolat., and Net ET for each HUC 12. for just input values for Reservoirs with no Terraces.  The 9 sets of  values for each of the 3 meteorological stations and 3 soil types are summed to get the total amount of RECHARGE out of the reservoir, NET ET from the reservoir site, and Outflow is then divided into change in SURF RUNOFF(streamflow at the outlet of the sub-basin) and TRANS LOSS (the part of RNOF that is estimated to have been lost between the reservoir site and the outlet of the sub-basin during the streamflow process) based upon the stream travel distance to the outlet of the sub-basin and the percent loss per mile of stream length for each HUC 12.

For the Reservoirs With Terraces Scenario, again, the input data array is searched to collect the total amount of change in Inflow, Outflow, Percolat., and Net ET for each HUC 12. for just input values for Reservoirs With Terraces plus the estimated fraction of the land above terraces in the reservoir drainage area.  Here more calculations are needed and for this reason, the output file FINALRESULTS has the subset of results, For Reservoirs Only With Terraces.  The Net ET effect is the sum of the net ET effect for the Terraces Only Scenario plus the Net ET for the Reservoirs with Terraces amounts.  The Net ET from the terraces above the reservoirs is not affected by the reservoir below them.  The reservoirs, however, receive less Inflow because of the terraces above them.  So, the total Net ET is less than the sum of the Net ET from the Terraces plus the sum of the Net ET for Reservoirs Only.  Similarly, the total RECHARGE for this scenario will be affected in a similar fashion.  The effect on SURF RUNOFF and TRANS LOSS must account for the portion of terraces not above reservoirs plus the overflow from the Reservoirs with Terraces.  The sum of these two amounts are then split into SURF RUNOFF and TRANS LOSS by First, the 9 sets of values for each of the 3 meteorological stations and 3 soil types are summed to get the total amount of

Table 4.  Portion FINALRESULTS file generated by POST.

FINAL RESULTS FOR EACH HUC12 IN AVERAGE AC-FT/YR AT ITS SUB-BASIN OUTLET

VALUES UNDER EACH HEADING ARE CHANGE IN ET, STREAMFLOW, PERCOLATION AND TRANSMISSION LOSS, RESPECTIVELY

HUC12       FOR TERRACES ONLY           FOR RESERVOIRS ONLY     COMBINED TERRACES & RESERVOIRS    FOR RESERVOIRS ONLY WITH TERRACES

40101    0.0    0.0    0.0    0.0     0.0    0.0    0.0    0.0     0.0    0.0    0.0    0.0            0.0    0.0    0.0    0.0

40102    1.3   -1.0    1.1   -1.4     0.0    0.0    0.0    0.0     1.3   -1.0    1.1   -1.4            0.0    0.0    0.0    0.0

40103    9.5  -11.0   11.1   -9.5     0.0    0.0    0.0    0.0     9.5  -11.0   11.1   -9.5            0.0    0.0    0.0    0.0

40104    0.0    0.0    0.0    0.0     0.0    0.0    0.0    0.0     0.0    0.0    0.0    0.0            0.0    0.0    0.0    0.0

40105    3.7   -4.3    2.4   -1.7     0.0    0.0    0.0    0.0     3.7   -4.3    2.4   -1.7            0.0    0.0    0.0    0.0

40106    9.7  -13.0    6.1   -2.8     2.4   -7.9    7.2   -1.7    11.9  -20.3   12.8   -4.4            2.3   -7.3    6.6   -1.6

40201   15.3  -21.3    9.4   -3.4    33.0 -113.2   98.2  -18.0    45.8 -125.0   99.0  -19.8           30.6 -103.7   89.6  -16.4

40202   47.2  -49.0   37.5  -35.7    15.2  -34.9   45.2  -25.4    61.3  -81.0   78.7  -59.0           14.1  -32.0   41.2  -23.3

40203   52.1  -69.6   38.4  -20.9     0.0    0.0    0.0    0.0    52.1  -69.6   38.4  -20.9            0.0    0.0    0.0    0.0

40204   34.1  -53.9   23.9   -4.1     1.3   -5.0    4.0   -0.4    35.4  -58.5   27.5   -4.4            1.2   -4.5    3.6   -0.3

40301   59.0  -55.3   48.1  -51.8     0.0    0.0    0.0    0.0    59.0  -55.3   48.1  -51.8            0.0    0.0    0.0    0.0

40302   41.2  -41.1   36.1  -36.2     0.0    0.0    0.0    0.0    41.2  -41.1   36.1  -36.2            0.0    0.0    0.0    0.0

40303   39.8  -62.5   45.3  -22.7    14.8  -48.5   51.3  -17.6    53.6 -107.3   92.6  -38.9           13.8  -44.8   47.3  -16.3

40304   49.5  -92.5   66.6  -23.5     0.0    0.0    0.0    0.0    49.5  -92.5   66.6  -23.5            0.0    0.0    0.0    0.0

40305   59.9 -111.9   64.8  -12.8    44.8 -179.9  155.6  -20.5   101.8 -278.2  208.2  -31.8           41.8 -166.2  143.4  -19.0

40306   51.3  -99.0   62.8  -15.0     0.0    0.0    0.0    0.0    51.3  -99.0   62.8  -15.0            0.0    0.0    0.0    0.0

40307   52.1  -91.7   56.4  -16.7     0.0    0.0    0.0    0.0    52.1  -91.7   56.4  -16.7            0.0    0.0    0.0    0.0

40308   28.9  -76.5   58.3  -10.7     0.0    0.0    0.0    0.0    28.9  -76.5   58.3  -10.7            0.0    0.0    0.0    0.0

40309  145.6 -419.6  327.7  -53.7    11.2  -47.5   42.3   -6.1   155.7 -461.8  365.2  -59.1           10.1  -42.2   37.5   -5.4

40401    7.0   -7.4   20.7  -20.3     0.0    0.0    0.0    0.0     7.0   -7.4   20.7  -20.3            0.0    0.0    0.0    0.0

40402   16.8  -14.1   30.8  -33.5     0.0    0.0    0.0    0.0    16.8  -14.1   30.8  -33.5            0.0    0.0    0.0    0.0

40403   20.8  -16.5   47.9  -52.2    10.7  -12.3   40.6  -39.0    30.2  -27.2   83.0  -86.1            9.4  -10.7   35.2  -33.8

40404   38.8  -42.6   77.8  -74.0     0.0    0.0    0.0    0.0    38.8  -42.6   77.8  -74.0            0.0    0.0    0.0    0.0

40405   35.3  -51.0  113.8  -98.1   159.0 -261.2  604.3 -502.1   175.1 -278.0  637.1 -534.3          139.8 -226.9  523.3 -436.2

40406   42.6  -48.5   95.7  -89.9   132.9 -223.7  505.1 -414.3   159.5 -242.8  533.1 -449.7          116.8 -194.3  437.4 -359.9

40407   46.0  -77.2  113.9  -82.7   223.5 -518.1  849.5 -555.0   242.5 -527.2  849.5 -564.8          196.5 -450.0  735.6 -482.1

40408   60.4  -99.4  141.2 -102.2   190.1 -426.6  675.2 -438.7   249.1 -515.6  796.6 -530.2          188.7 -416.2  655.4 -427.9

40409   37.4  -56.3   52.7  -33.9   100.9 -281.8  350.3 -169.4   131.6 -316.6  375.4 -190.4           94.1 -260.3  322.7 -156.5

40410   60.6 -103.9   77.1  -33.8    18.8  -63.6   65.4  -20.7    78.2 -162.7  137.4  -52.9           17.6  -58.7   60.3  -19.1

40501  178.1 -339.3  211.3  -50.0    91.1 -361.5  323.6  -53.2   268.5 -692.0  525.4 -101.9           90.5 -352.6  314.1  -51.9

40502  109.5 -239.8  150.1  -19.8     0.0    0.0    0.0    0.0   109.5 -239.8  150.1  -19.8            0.0    0.0    0.0    0.0

40503   42.9  -79.4   40.9   -4.4    32.1 -136.1  111.5   -7.5    72.8 -205.1  143.6  -11.4           30.0 -125.7  102.7   -7.0

40601  259.2 -129.6  348.0 -477.6     0.0    0.0    0.0    0.0   259.2 -129.6  348.0 -477.6            0.0    0.0    0.0    0.0

RECHARGE out of the reservoir, NET ET from the reservoir site, and Outflow is then divided into change in SURF RUNOFF (streamflow at the outlet of the sub-basin) and TRANS LOSS (the part of RNOF that is estimated to have been lost between the reservoir site and the outlet of the sub-basin during the streamflow process) based upon the stream travel distance to the outlet of the sub-basin and the percent loss per mile of stream length for each HUC 12.

The production of the FINALRESULTS file represents the complete process of all of the simulation work developed for the study.  FORTRAN output is a less than desirable way to present the results.  So, an Excel spreadsheet was used.  The contents of FINALRESULTS, less the column headings are extracted in a simple editor and then pasted into a page on the spreadsheet, ENTIRE BASIN OUTPUT FOR 1999-2008 BASE FILE FOR COMPARISON.xls, which can be found in the directory, Excel Spreadsheets, on the USB Drive.  The copied portion of FINALRESULTS is pasted on the POST Results worksheet and the values flow to the other four worksheets for display by sub-basins, etc.

EXECUTION CHAIN for Batch Processing for Simulating the Results for the Study Area

This requires the simulations for all 32 meteorological stations be done with each of the three soils types that may be part of the area for the condition of no terraces in the drainage area of the typical reservoir.  This requires a total of 96 runs.  An additional 96 runs are required after the input files to V2 have been revised to change the amounts of land areas in the drainage area of the typical pond so that those areas with terraces are included.  So, a total of 192 runs for V2 are required.

- Create the 96 different input files to V2 for the no terraces condition in the drainage area of the pond.  This requires editing of the 96 files, one for each of three soil types for each of the 32 meteorological stations.

- Create the 96 different input files to V2 for the with terraces condition in the drainage area of the reservoir.

- Make sure all of these input files for V2 are in the same directory as V2, PB, CB, and CNSLD programs and that all other data files needed by V2 and PB in the same directory

It is recommended that the user create a new subdirectory by copying an existing directory with all of the necessary other data files in it to a new subdirectory with a relevant title for the batch file.

The following commands form a portion of an Input Batch file that can be created easily in NotePad or other editor.   On the media provided, the 1999-2008 directory contains all of the necessary files for a complete 10-year run.  The particular input batch file is, Input Batch File for 1999-2008 Simulation.txt.  That file has some introductory information in it.  The contents below the -------- line need to be copied to the MS-DOS prompt.  Use COPY to put the entire  contents to the Windows clipboard.

Next, open the MS –DOS command prompt and point it to the particular directory with all of the required information in it.  If the Command Prompt is not pointing the correct location, simply right-click it, choose properties and type in the correct directory location.  Use Paste to copy in the entire contents of Batch File at the MS-DOS prompt.  The sequence of execution of the list of commands will commence immediately provided all the correct information is in the particular directory.

All of the files along with the Batch File, Input Batch File for 1999-2008 Simulation.txt, are contained in the Directory, 1999-2008, on the media included with this manual.  Here is a sample of that batch file.  It is not recommended to use the media that is included with the manual as the location to execute the batch file. The read and write process is slow.  Using the fixed drive on your computer or a fast auxiliary drive will make the most efficient execution possible.

V2

atwood5nt.dat

V2

atwood5wt.dat

CNSLD

V2

atwood7nt.dat

PB

CB

V2

atwood7wt.dat

CNSLD

V2

atwood11nt.dat

PB

CB

V2

atwood11wt.dat

CNSLD

.

.

.

culbertson7nt.dat

PB

CB

V2

CNSLD

POST

Other Files Required in the Directory for a Run of the Batch File

It is recommended that the user copy the directory, 1999-2008, on the media included to a new directory of a pertinent name before starting a different simulation.  This assures that all of the necessary files are in the new directory.  You will see a number of other files there, too, that will be overwritten as a part of the next simulation.  Making a new directory each time also assures that none of those files from previous simulations will be lost by the next simulation.

In addition to the executable files, V2, PB, CB, and POST all of the following files are also necessary for a successful run:

V2 Input files – All 192 individual files combinations of 32 meteorological stations, three soil types, and with no terraces and with terraces in the typical reservoir drainage area.  Examples for one meteorological station are ****5nt.dat, ****5wt.dat, ****7nt.dat, ****7wt.dat, ****11nt.dat, ****11wt.dat, where **** is the name for one of the 32 stations.

All Meteorological files -- ****tmp.dat, ****pcp.dat, ****EToPM.dat. where, ****, is the name of one of the 32 stations.

Water-Use Coefficient files – WUCeast, WUCmiddle, WUCwest

Terrace Characteristics files – LCFCTERRACEDATA.DAT, LCBBTERRACEDATA.DAT,

LOFCTERRACEDATA.DAT, GRBBTERRACEDATA.DAT, LOBBTERRACEDAT.DAT

Using the MS-DOS Prompt

An example Command Prompt for a Drive H is located on the media included.  That file can be easily opened by just clicking on it.  To make it useful for your purposes, right click on it and go to Properties.  In the Start in: window you can enter the exact directory location in which you want to execute a batch file.  For example if you use a USB drive or other media on your system it will identified as a drive such as F:.  You can create a directory on the USB drive of your choice and set the Start in: to read something like, F:NewSimulation, to open the MS-DOS prompt in that directory to which you have already copied all of the required files or simply copied an existing entire directory so that all of the files are present in that directory.  Finally, use your text editor to COPY batch file commands to the Windows clipboard.  Go to the MS-DOS prompt and right click after the prompt and PASTE the contents of the batch file there.  Execution of the batch file proceeds immediately.

SOME FINAL THOUGHTS

It is important to point out that the study area was for the entire basin above Hardy, Nebraska, but the simulation modeling was done for only those portions of the basin that might be expected to have land terraces installed on them.  These were fairly deep soils that would produce some runoff and the primary ones were of the silt loam type.  Too, they had some slope that would result in water erosion from runoff or there was enough runoff to collect it in storage-type terraces to improve crop production, particularly in dry and average years.  The simulations did include these types of lands in the terraced and unterraced condition and in the drainage areas of the reservoirs the portion of the land terraced was usually less than about twenty-five percent.  For the portions of the drainage areas of the reservoirs in the study area we assumed that the unterraced portions were adequately represented by the land uses on the HRUs simulated.  To simulate all of the conditions in the entire basin to produce results for the entire basin was well beyond the scope of this study.  So, this means that all of the results for the three scenarios are determined by differences between amount from those lands affected by terraces and reservoirs with the amount produced without them being there.  Too, it is important to realize that the areas included in this study probably are on average the most productive areas for runoff in the basin. Also, these same soils are less conducive to producing ground-water recharge, too, because of their relatively fine textured.  So, an attempt to scale the results to the entire drainage area of the basin is not appropriate.  The POTYLDR model with proper input values could be used for the entire basin to simulate runoff and ground-water recharge, but that task would be a substantially greater effort and would require considerable work to include all additional HRUs to represent the additional land areas as well as much more GIS work to identify the extent of their areas.  Finally, the interactions between groundwater and streamflow along with the operation of the reservoirs, etc. would need to be addressed in a comprehensive modeling effort to represent the entire basin.

REFERENCES

Arnold, J.G., P.M. Allen and G. Bernhardt.  1993.  A comprehensive surface-groundwater flow model.  J. of Hydrology 142: 47-69.

Bureau of Reclamation.  1984.  Solomon River Basin Study.  Bureau of Reclamation, Lower Missouri River Basin Division, Denver, CO.

Choodegowda, Ravikumar B.  2009.  Modeling small reservoirs in the Great Plains to estimate overflow and ground-water recharge.  Ph.D. dissertation.  Abstract is available at: http://krex.k-state.edu/dspace/handle/2097/4610

Koelliker, J.K, J.J. Zovne, J.M. Steichen, and M.W. Berry.  1981. Study to assess water yield changes in the Solomon River Basin, Kansas.  Part I – Final Report.  Kansas Water Resources Research Institute.  Manhattan, KS. 110 pp.

Koelliker, J.K., J.J. Zovne, and M.W. Berry.  1982.  Study to assess water yield changes in the Solomon Basin, Kansas.  Part II – User’s Manual.  Kansas Water Resources Research Institute.  Manhattan, KS. 92 pp.

Koelliker, J.K. 1994.  User’s manual for POTential YieLD Model Revised.  Civil Engineering Department, Kansas State Univ., Manhattan, KS.

Koelliker, J.K.  1998.  Effects of agriculture on water yield in Kansas.  Chapter 7.  IN: Perspectives on Sustainable Development of Water Resources in Kansas, M. Sophocleous, Ed.  Kansas Geol. Sur., Bulletin 239. 239 pp.

Ramireddygari, S.R., M.A. Sophocleous, J.K. Koelliker, S.P. Perkins, R.S. Govindaraju.  2000.  Development and application of a comprehensive simulation model to evaluate impacts of watershed structures and irrigation water use on streamflow and groundwater: The case of Wet Walnut Creek Watershed, Kansas, USA. J. of Hydrology 236: 223-246.

Ritchie, J.T.  1972.  Model for predicting evaporation from a row crop with incomplete cover.  Water Resources Research 8: 1204-1213.

USDA Soil Conservation Service.  1972.  Hydrology.  National Engineering Handbook, Section 4.  Washington, DC.

USDA Soil Conservation Service.  1975.  Kansas Irrigation Guide.  Salina, KS.

Viessman, W., Jr. and G.L. Lewis.  1996.  Introduction to Hydrology, 4th Edition, Harper Collins College Publishers.  760 pp.

LIST OF ACRONYMS AND SYMBOLS USED

AET = actual evapotranspiration

AMC = antecedent soil moisture

ASM = available soil moisture

DP = deep percolation

ETo = grass reference evapotranspiration

HRU = hydrologic response unit

I = amount of incoming energy used to evaporate water from the interception reservoir

Ks = soil-water coefficient based on ASM

LZ = lower zone

P = precipitation

POTYLD = POTential YieLD model, the original version

POTYLDR = POTYLD Revised model, version developed in 1993

R = runoff

RCN = runoff curve number

S = maximum potential abstraction of rainfall

U = amount of water that is lost at the potential rate from a wet bare soil

V2 = POTYLDR 2.0, the version developed for the Republican River Basin Study

WUC = water-use coefficient

Appendix A

AUGUST 2013

FILE INFORMATION CONTENTS on the media provided for the USERS MANUAL for the COMPUTER SIMULATION MODEL to ESTIMATE the EFFECTS OF LAND TERRACING and SMALL FEDERAL RESERVIORS on STREAMFLOW and GROUND-WATER RECHARGE in the REPUBLICAN RIVER BASIN ABOVE HARDY, NEBRASKA

Each item listed below is in a separate file by the name shown in the table under the appropriate

Directory title shown.  ONLY those files that the user should need are listed below.

- Model Manual Files

Command Prompt Drive H

V2 (POTYLDR 2.0) Description Files

V2 (POTYLDR 2.0 MODEL USERS MANUAL)

Original POTYLDR Manual and File

Example Full Model Run Set - 1999-2008

Input Batch File

BATCH RUN SUMMARY for 1999-2008

FINALRESULTS

1999-2008

Input Batch File for 1999-2008 Simulation

FORTRAN 90 Code Files

V2.f90

PB.f90

CB.f90

CNSLD.f90

POST.f90

WC.f90

FORTRAN Executable Programs

V2

PB

CB

CNSLD

POST

WC

FILE INFORMATION CONTENTS (Continued)

Excel Spreadsheets

ENTIRE BASIN OUTPUT FOR 1999-2008 BASE FILE FOR COMPARISON

Post-processing Input Data for HUC 12s FULL SET_Updated_8_25 with NE large reservoirs

Terrace Characteristics Files

LCFCTERRACEDATA.DAT

LCBBTERRACEDATA.DAT

LOFCTERRACEDATA.DAT

GRBBTERRACEDATA.DAT

LOBBTERRACEDAT.DAT

Water Use Coefficients Files

WUCeast

WUCmiddle

WUCwest

Meteorological Stations Files

****pcp.dat

****tmp.dat

**** EToPM.dat