User's Manual

for

POTential YieLD Model Revised

POTYLDR

A computer simulation model to estimate

water yield on a continuous basis.

Originally developed as POTYLD by

J.K. Koelliker, J.J Zovne, J.M. Steichen and M.W. Berry

1982

Updated and revised

by

J. K. Koelliker, Professor

Civil Engineering Department

Kansas State University

October 1994

Table of Contents

Section	Page

Preface	ii

Files on the POTYLDR Distribution Diskette	1

DESCRIPTION OF INPUT INFORMATION FOR POTYLDR.EXE MODEL	3

I.	To create input file for identification of meteorological station and pond and land uses information.	3

II.	Daily meteorological data input files	8

Creating an Input Data File for POTYLDR	9

Example Problem	9

Running the Example Problem	12

Post-processing Results of POTYLDR to Estimate Sub-Basin Surface Water Yield and Ground-water Yield	13

References Cited	14

Appendix A --	Climatological Information for Calibration of Penman Equation	16

Appendix B --	Monthly Climatological Data for Various Locations	18

Appendix C -- 	Soils Information for Irrigation Classes	21

Appendix D -- 	Runoff Curve Number Information for POTYLDR	23

Appendix E --	Available Precipitation and Temperature Data Previously Prepared for use With POTYLDR	26

Appendix F --	Printouts From Example Problem From POTYLDR	27

Appendix G -- 	POTYLDR.EXE Source Code Printout	42

PREFACE

The purpose of this manual is to provide users of POTYLDR (POTential YieLD Revised), water budget simulation model a detailed description of how to use this model.  The materials presented herein assume the user has studied the description of the model and is familiar with what how it simulates water yield.

This latest version of the model represents a more comprehensive effort to update the original model (Koelliker et al., 1981, 1982) for current use and to provide more current documentation of the model in its present form for use on personal computers.  The original model, POTYLD, was converted to PC format in 1986.  It was used more or less without modification until 1993.  Recent changes and enhancements and reformatting of output have led to the current revised version, POTYLDR.  Additional enhancements include splitting meteorological data files, one for precipitation and a second for maximum and minimum daily temperatures.  Many precipitation stations do not report temperature, so the most representative station for temperature can be used in conjunction with the best precipitation station for a sub-basin simulation.  Irrigation has been added and can be simulated on any of the land uses in the sub-basin.  Irrigations with a specified depth of water are scheduled between starting and ending dates based upon percent of available soil moisture.  The output from a simulation includes ASCII files which can be used for further post-processing.  These include different files with 1) precipitation excess, 2) percolation and seepage from pond, and 3) irrigation use on each land use.  These files can be output on a monthly or annual basis.  A daily output file of water budget of the last year of the simulation run for each land use is another new addition.  Components which estimate potential evapotranspiration, runoff, interception, and snow have been modified to improve their results and operation.  Additional modifications have been made to estimate actual evapotranspiration, too, based on some adjustment of solar radiation based on daily temperature, addition of a function to estimate effects of heating and cooling at the surface, on soil moisture, and on effects of surface residue.  More enhancements/changes are discussed below.

This manual provides step-by-step instructions on how to find input information and how to enter it into computer files that will allow you to run the program.  You are provided an example problem which can be followed as a guide for your particular situation.  Also, the Distribution Diskette included with this manual contains a copy of the input files used for the example problem.  Abridged copies of the printouts of the output files are found in appendices of this manual.  Copies of the output files for the example problem are contained on the Distribution Diskette, too.  A  complete listing of the variable names used, flowcharts, and program listing are provided in appendices, also.

To determine the watershed surface yield for the original procedure, another program, DEPLETE was used to process outputs from POTYLD.  In POTYLDR, a post-processing section has been added so that it is done automatically during the execution of the main program.  The transmission loss factor has been important to the calculation of actual runoff.  The original concept from Sharp et al. (1966) has been revised so that it is based upon moisture deficit rather than Thornthwaite's PET.  This allows for easier calculations and bases losses on a factor more satisfying to the authors.  In addition to calculating surface yield, ground-water yield (percolation from land uses and seepage from ponds) is automatically calculated as part of the post-processing section.  Thus, the user now has the capability to estimate the total water yield for each period for a sub-basin as a part of making a simulation with POTYLDR.

Finally, to compare results of POTYLDR with actual streamflow still requires interpretation of  timing of flow from seepage to streamflow, if any, and effects of ground-water and surface diversions in the sub-basin on streamflow.  For the example problem, the amount per time period is calculated.  It is beyond the scope of this manual to determine these complex interactions.

I believe POTYLDR is functioning reasonably based upon our experiences with it for conditions across Kansas.  I  assume no responsibility for the results and the actions resulting therefrom.  If you find problems with the operation of POTYLDR, please contact me.  I am trying to maintain the model in good operating condition.

If you have problems or comments, contact me at

James K. Koelliker

Biological and Agricultural Engineering Department

Kansas State University

Seaton Hall

Manhattan, KS 66506-2905

Voice:  (785) 532-2904

FAX:  (785) 532-5825

E-mail:  KOELLIK@KSU.EDU

July 30, 2026

Files on the POTYLDR Distribution Diskette

The Distribution Diskette is an MS-DOS, 3.5-inch, high density (1.4Mb)

POTYLDR.EXE	FORTRAN executable code (compiled in Microsoft7 Version 5.0) -- you must have a 486 machine or math coprocessor, 640K RAM, and a fixed disk with at least 5Mb of free space

POTYLDR.FOR	FORTRAN source code in ASCII format

GTBNDANL.DAT	Input file for meteorological station description, pond, and land uses information for Example Problem in ASCII format

GTBNDMON.DAT	Input file for meteorological station description, pond, and land uses information for Example Problem in ASCII format except makes monthly output

GTBNDPCP.DAT	Daily precipitation data for Great Bend, KS, 1948-1988 for Example Problem in ASCII format.  Station Index No. 143218

GTBNDTMP.DAT	Daily temperature data for Great Bend, KS, 1948-1988 for Example Problem in ASCII format.  Station Index No. 143218

GTBNDANL.OUT	Master output file from Example Problem in ASCII format containing summary of input information, annual output for pond and land uses, and overall summary from the run

ANNLSURF.OUT	Output file from Example Problem of year, precipitation, lake evaporation, pond evaporation, overflow from the pond, and surface yield from each land use simulated on an annual basis in ASCII format

ANNLPERC.OUT	Output file from Example Problem of year, pond seepage, and percolation from each land use simulated on an annual basis in ASCII format

ANNLIRRG.OUT	Output file from Example Problem of year and amount of irrigation required by each land use simulated in ASCII format

ANNLYLD.OUT	Output file from Example Problem from post-processing routine in ASCII format

DAILY.OUT	Output file from Example Problem of daily values for each land use for the last year of the simulation run in ASCII format

PONDCALC.WK1	Spreadsheet to determine the dimensions of a typical pond for a

PONDCALC.WQ1	sub-basin for input to POTYLDR

KSCLIMAT.WK1	Spreadsheet to determine monthly average climatological

KSCLIMAT.WQ1	variables for locations in Kansas from values for adjacent 1-st order weather reporting stations

MANUAL.doc		Word file for this user's Manual for POTYLDR

Note:  The Distribution Diskette will not hold output file from a POTYLDR run.  You should make a directory on your fixed disk and copy the files from the Distribution Diskette into that directory.  The program will write files on the same drive (directory) in which you are logged.  Generally, this is the same drive (directory) in which the POTYLDR.EXE file is located.  IMPORTANT:  POTYLDR is set up to overwrite all files made from a previous run for the same input file information.  IF you have results that you want to keep, copy or move them to another directory or rename them BEFORE making a subsequent run.

The time required to make a run of POTYLDR will depend on the speed of your computer to execute the program and write the files to disk.  To run POTYLDR for the Example Problem on a 486-33 Mhz machine required 50 seconds to execute the 41 years after the three filename prompt was completed; a monthly printout of results required 55 seconds.  When 18 land uses were simulated simultaneously, annual and monthly results required 75 and 85 seconds, respectively.  A 386 machine will take about three times as long as a 486, a 286 about five times as long, and a 8088 such as an IBM-XT will take about 10 times as long as a 486.  Of course, 386, 286, and 8088 machines must have a math coprocessor.

DESCRIPTION OF INPUT INFORMATION FOR POTYLDR.EXE MODEL

I.	To create input file for identification of meteorological station and pond and land uses information.

Refer to Table 1 for an example of this file.  Follow the input formats exactly as shown on the example.  FORTRAN is rather unforgiving on format errors.  If a "." is shown on an entry, be sure your input contains one, too.  A good way to begin preparing your information would be to make a copy of the input file, GTBNDANL.DAT, from the Distribution Diskette and then edit your information into that file.  Check you editor or word-processing software on how to make a copy of an ASCII file.

Line 01:	LOCATION:  Identifier (Literal Field up to 48 characters) specified by the user.

Line 02:	Filenames for the simulation run -- FILE2, FILE3, FILE5  (must follow MS-DOS file- naming convention)

FILE2:  ASCII file with daily precipitation data in monthly blocks.   File is assembled or obtained by the user.  Format for file is described in II. following this section.

FILE3:  ASCII file with daily max. and min. temperatures, oF, in monthly blocks.  File is assembled or obtained by the user.  Format for file is described in II. following this section.

FILE5:  Filename for master output file for the simulation run specified by the user.

Line 03:	Namelist/ALPHA -- BRUNTA, BRUNTB, E, RCROP, OUTPUT, TAREA

BRUNTA, BRUNTB:  Coefficients for Penman equation based upon geographical location.  See the graphs in Appendix A for help in estimating BRUNT A (c) and BRUNT B (d).  These values may need to be adjusted by "calibration."  Select values from the graphs, make a simulation run of the model, and then make adjustments, if necessary, until estimated annual lake evaporation agrees closely with the value for the sub-basin area being simulated.  See the map in Appendix A for best estimates of annual lake evaporation.  Moisture deficit = (annual  lake evaporation - annual precipitation), inches.

E:  Wind coefficient for Penman equation for calculating potential evapotranspiration, usually 0.75.

RCROP:  Reflectance coefficient for cropped areas.  Ranges from 0.20 to 0.25, usually 0.23.

OUTPUT:  Code for type of output files.  1 = annual output values for pond and individual land use accounts on master printout and post-processing for FIXED land use conditions; annual output of precipitation, lake evaporation, pond evaporation, pond overflow, and precipitation excess from each land use (File: ANNLSURF.OUT); annual output of seepage from pond and percolation from each land use (File: ANNLPERC.OUT); annual irrigation amount for each land use (if irrigation is part of simulation) (File: ANNLIRRG.OUT); and post-processing to estimate annual water yield from the sub-basin (File: ANNLYLD.OUT).  2 = monthly output of the exact same information as for annual output in the master filename you specify plus the Files: MNTHSURF.OUT, MNTHPERC.OUT,  MNTHIRRG.OUT, and MNTHYLD.OUT with monthly information the same as for ANNLSURF.OUT, ANNLPERC,OUT, ANNLIRRG.OUT,  and ANNLYLD.OUT. 3 = same as 1, except post-processing to estimate yields for HISTORICAL land use conditions.

Table 1.	EXAMPLE FILE INPUT FOR POTYLDR.EXE FOR IDENTIFICATION OF METEOROLOGICAL DATA,

POND, AND LAND USES INFORMATION FOR SUB-BASIN

FILENAME -- GTBNDANL.DAT ( on the program on the Distribution Diskette).  The input file information is shown on the left-hand side of the page.  The explanation is on the right-hand side.  For specific explanations of inputs and where to find the values, consult the User's Manual under the particular Line No.  When program is executed, user is prompted to enter this filename for Unit 1.

Explanation of Input Line

CHEYENNE BOTTOMS SUB-BASIN W/GREAT BEND	Line 01:	Descriptive title for simulation

'GTBNDPCP.DAT' 'GTBNDTMP.DAT' 'GTBNDANL.OUT'	Line 02:	Weather data input files & output filename

0.63 0.034 0.75 0.23 3 20000.       	Line 03:	Namelist/ALPHA

0.200 12. 0. 0.0 13.0                      	Line 04:	Namelist/BETA

143218 143218 1979 1988 09 02                     	Line 05:	Namelist/OMEGA

01 64. 78. 06.50 14.1 39.4 19.6              	Line 06:	Jan. meteorological data

02 63. 79. 08.70 14.5 46.8 24.0               	Line 07:

03 66. 75. 11.30 16.3 55.5 31.3	Line 08:

04 62. 77. 14.10 16.0 67.8 44.0              	Line 09:		Lines 06-17 are fixed

05 64. 83. 15.90 14.8 77.1 54.5        	Line 10:		format.  Note the length

06 75. 81. 16.70 14.6 87.8 64.4           	Line 11:		of each entry and there

07 81. 75. 16.30 12.9 93.5 69.1         	Line 12:		is one space between

08 80. 75. 14.90 13.9 91.8 67.2           	Line 13:		each entry.

09 81. 77. 12.40 14.1 82.2 58.4           	Line 14:

10 75. 77. 09.60 13.6 71.2 47.6            	Line 15:

11 70. 75. 07.10 14.0 55.0 31.5            	Line 16:

12 68. 78. 05.90 13.6 44.4 24.2                   	Line 17:	Dec. meteorological data

01 03 1 000. 53. 72. 86. 09 25 06 15 1 1 2 0.25 	Line 18:	Land use No.1 Information

02 03 1 010. 53. 72. 86. 09 25 06 15 1 2 2 0.25 	Line 19:		Lines 18-26 are fixed

03 03 1 025. 53. 72. 86. 09 25 06 15 2 2 2 0.20 58. 76. 89.	Line 20:		format.  Note the length

04 03 1 025. 53. 72. 86. 09 25 06 15 3 2 2 0.20 58. 76. 89.	Line 21:		of each entry and there

05 03 2 010. 58. 76. 89. 05 10 09 30 1 2 2 0.05	Line 22:  	is one space between

06 03 2 000. 55. 74. 88. 05 10 09 30 1 1 2 0.20	Line 23:		each entry.  With 18 land

07 03 2 010. 55. 74. 88. 05 10 09 30 1 2 2 0.20	Line 24		these would continue to

08 03 5 040. 55. 74. 88. 04 01 10 30 1 2 1 0.10	Line 25:		Line 34.

09 03 5 040. 51. 70. 85. 04 01 10 30 1 2 1 0.20	Line 26:	Land use No. 09 Information

01 10 20 05 15 50. 2.00	Line 27:	Irrigated land uses information

06 06 15 09 05 50. 2.00                       	Line 28:	One line for each land use irrigated.

79 25 00. 20. 15. 15. 00. 00. 10. 20. 20.    	Line 29:	No. of ponds and percent of each

80 26 00. 20. 15. 15. 00. 00. 10. 20. 20. 	Line 30:		land use in sub-basin by year.

81 27 00. 20. 15. 15. 00. 00. 10. 20. 20.	Line 31:

82 28 00. 20. 15. 15. 00. 00. 10. 20. 20.	Line 32:		One line is needed for each

83 29 00. 20. 15. 15. 01. 01. 08. 20. 20.	Line 33:		year of an historic simulation

84 30 02. 18. 15. 15. 02. 01. 07. 20. 20.	Line 34:		and percent for each land use

85 30 03. 17. 15. 15. 03. 02. 05. 20. 20.	Line 35:		is needed each year. Only one line is

86 31 04. 16. 15. 15. 04. 02. 04. 20. 20.	Line 36:		needed for fixed land uses.  None is

87 31 05. 15. 15. 15. 05. 03. 02. 20. 20.	Line 37:		needed for no post-processing.

88 31 06. 14. 15. 15. 06. 03. 01. 20. 20.	Line 38:	Last line for ponds and land uses

Line 03:	(continued) 4 = same as 2, except post-processing to estimate yields for HISTORICAL land use conditions.  5 = same as 1, except NO post-processing.  6 = same as 2, except no post-processing.  All options produce file, DAILY.OUT, the water budget information on a daily basis for each land use for the last year of the simulation period.  Options 1 and 2 require one input line of land use and number of ponds information for the year of fixed conditions you are simulating.  Options 3 and 4 require one input line for EACH year of the historical period for which the simulation is being made.  The number of lines must equal YSTART-YEND + 1.  See Lines 28-39 section on how to obtain these data.

TAREA = total area in sub-basin being simulated, acres

Line 04:	Namelist/BETA -- DSEPRT, HMAX, L, W, S

DSEPRT:  Daily seepage rate for pond from beneath actual surface area calculated for the day; range from 0.0625 to 0.25 in./day.  The value is usually affected most by clay content of soil under the pond.  Higher amounts of clay result in less seepage.

HMAX:  Maximum depth in pond in feet.  Below this depth, no water can escape by overflow and above this depth all water overflows immediately.  For ponds with principal spillways, maximum depth is up to the elevation of the principal spillway.

L:  length of pond flat bottom at zero depth, feet	See PONDCALC.WK1 for help

W:  width of pond flat bottom at zero depth, feet	with choosing H, L, W, and

S:  side slope of pond (ft/ft) expressed as run:rise 	S for a typical pond.

This information may be obtained from U.S.G.S. 7.5-minute topographic maps.  Ponds existing at the time of the preparation of the map are shown.  If maps are old, consideration must be given to those added since the maps were prepared.  A number of watershed areas for ponds in the sub-basin should be measured on the maps to determine an average size.  The actual pond dimensions must determined from the maps, SCS sources, or other local authorities.  If large ponds exist, they should be considered as several typical ponds.

Line 05:	Namelist/OMEGA -- PINDX, TINDX, YSTART, YEND, NPLOTS, IPLOTS

PINDX:  Station index number (6-digit number assigned by National Climatic Center, see Climatological Data or datafile) for precipitation station used in simulation.

TINDX:  Station index number (6-digit number assigned by National Climatic Center, see Climatological Data or datafile) for temperature station used in simulation.

YSTART:  Year the simulation period starts; must be equal to or greater than beginning year of precipitation and temperature records available.

YEND:  Year the simulation period ends; must be equal to or less than the ending year of the precipitation and temperature records.

NPLOTS:  number of land uses used in simulation; must be at least 1 and no more than 18.  There must be a line for each land use beginning with Line 17.

IPLOTS:  number of land uses in simulation which are irrigated; if none use a 0.  Must be no larger than NPLOTS.  A line for each land use irrigated follows the lines for land uses.

Line 06-

Line 17:	Mean monthly meteorological data for simulation location:  MO, PSUNS, RHD, RA, WIND, MMMAXT, MMMINT (one line for each month, 1-12).  These data, except RA, are available for five, 1st-order stations in each state.  Climates of the States (Water Information Center, 1974) contains information for all 50 states.  Use triangulation from the three most adjacent 1st-order stations for a particular location.  Where topographic or other influences affect variation in local climate triangulation may not be best.  Use judgment or local data.  BE SURE TO FOLLOW THE INPUT FORMAT EXACTLY AS SHOWN ON TABLE 1.  Information for the five, 1st-order stations in Kansas is shown in Appendix B.  Also, the spreadsheet, KSCLIMAT.WK1 has been developed to aid in estimating these values for a particular location in Kansas.  You need to know the distance to your location from the closest two or three 1-st order reporting stations.

MO:  Number of month

PSUNS:  long-term average percentage sunshine at location (%)

RHD:  long-term average relative humidity for early in the morning (6-8 AM) at location

RA:  mid-monthly intensity of clear day solar radiation, mm of evaporable water; available in most hydrology texts (if expressed as langleys or cal/cm2 per day, divide by 56 to convert to mm of water).  This value is a function of latitude of the particular location.  These values are readily available and usually reference Criddle (1958), see Appendix B.

WIND:  long-term average wind speed (miles/hr) for particular location; assumed to be measured at 30 feet above ground.

MMMAXT:  long-term average maximum air temperature for particular location, "F

MMMINT:  long-term average minimum air temperature for particular location, "F

Line 18-

Line 26:	Land use area parameters:  NO, ISOIL, ICROP, AREA, RCNI, RCNII, RCNIII, MGSBP, DGSBP, MGSEP, DGSEP, ROTATE, POND, TERR, MUL, FLRCN1, FLRCN2, FLRCN3

NO:  Land use area number

ISOIL:  SCS irrigation soil class, see Appendix C for Kansas values and/or SCS Irrigation Guide for the particular state for soil classifications.

ICROP:  Crop code:  1=wheat, 2=sorghum, 3=corn, 4=soybeans, 5=pasture, 6=alfalfa, 7=fallow.

AREA:  area of pond watershed represented by land use (acres).  If land use is not in pond watershed, enter 000.

RCNI, RCNII, RCNIII:  Runoff curve numbers for land uses for AMC I, II, and III, respectively.  See Appendix D or SCS publications, hydrology textbooks, etc for RCNII values.  See Appendix D, if needed, for conversion from AMC II to I and III.

MGSBP:  Month growing seasons begins             	These must be determined

DGSBP:  Day of month growing seasons begins    	for local conditions.

MGSEP:  Month growing season ends		      	Consult County Extension,

DGSEP:  Day of month growing season ends	      	SCS or other sources.

ROTATE:  Indicator for wheat/fallow rotation -- 1=no rotation, 2=fallow in first year, 3=wheat in first year.  Wheat-fallow is the only rotation implemented in the model at this time.

POND:  Indicator for land use which represents part of pond watershed --1=land use area does not drain into pond, 2=land use area does drain into pond

TERR:  Indicator type of terrace on land use -- 1=no terraces, 2=graded, 3=level, 4=level, closed-end

MUL:  reduction factor to apply when crop coefficient is 0. to account for effect of residue on suppression of evaporation from bare soil -- 0.0=no residue or bare soil to maximum of about 0.30 for complete residue cover with part standing to create excellent vapor barrier.

FLRCN1, FLRCN2, FLRCN3:  Runoff curve numbers for fallow areas for AMC I, II, and III.  See Appendix D or SCS publication, hydrology textbooks, etc for appropriate values.  See Appendix D, if needed, for conversion from AMC II to I and III.

Line 27-

Line 28:	Irrigated land uses parameters (one line for each land use irrigated):  IRGPLT, IMOBEG, IDYBEG, IMOEND, IDYEND, PSCH, ARATE

IRGPLT:  Land use number that is irrigated

IMOBEG:  Month of year irrigation begins	Consult County Extension,

IDYBEG:  Day of month irrigation begins	SCS or local experts for

IMOEND:  Month of year irrigation ends	best information.

IDYEND:  Day of month irrigation ends

PSCH:  Threshold of minimum percent of available soil moisture in upper or upper and lower zones together at which irrigation will be added.  Upper zone is checked first.  If upper zone is below PSCH, then irrigation is simulated.  If not, the combination of upper and lower zones are checked to see if the total is below PSCH.  If so, irrigation is simulated.  Value for most agricultural field crops is 50%.  For alfalfa and grass 40% is adequate.

ARATE:  Amount of simulated irrigation added to upper zone when irrigation is needed, inches.  Amount varies with the type of irrigation.  Generally, a value of less than 1.0 inches is not used.  A maximum value would be about 4 inches per application.  In no case should the amount of irrigation more than fill the upper and lower zones to more than 100% of field capacity.  See Appendix C for soils information, pay attention to the amount of available water various soil can hold.

Line 29-

Line 38:	Number of ponds and percent of each land use simulated for post-processing to calculate estimated water yield.  YEAR, NPD, PLU(1 TO NPLOTS)

For OUTPUT = 1 or 2 (fixed land-use conditions), a single line is needed for the year of land-use conditions being simulated.  These values will be used for the entire simulation period.

For OUTPUT = 3 or 4 (historical land-use conditions), a line is needed for each year of the period being simulated.  The number of lines must equal YSTART-YEND+1.

These lines are not needed if OUTPUT = 5 or 6.  They will be ignored by the program.

YEAR:  Year (19XX) for pond and land use information (first year must correspond to first year of the simulation).

NPD:  number of typical ponds in sub-basin.  If large ponds exist, they should be considered as several typical ponds by determining how many typical ponds each one represents.

PLU(1 to NPLOTS):  percent of each land use below ponds in the sub-basin (must be in the same order of the land uses simulated); there must be a value for each land use for each year.  Be sure the total for all land use adds up to 100 percent.  If land uses below ponds are substantially different than those above ponds, then land uses should be adjusted as appropriate to reflect actual conditions.  Land use information may be obtained from Census of Agriculture, State Board of Agriculture publications, SCS offices, etc.

II.	Daily meteorological data input files

These files have been assembled for many stations in Kansas for various uses.  A list of stations and years available are located in Appendix E.  Stations which have been simulated previously with POTYLDR also have an ASCII file: statnCLM.DAT,  which contains the meteorological and climate data for the station developed for the location and from the calibration runs.

Information in the precipitation and temperature files are blocked into months.  Each monthly block contains the following information:

SPINDX or STINDX:  6-digit station index number (same as PINDX and TINDX as described above)

YEAR:  	year (last two digits)

MONTH:	month (1 to 12)

followed by,

FOR PRECIPITATION FILES ONLY:

PREC:                   31 values of daily precipitation, inches

FOR TEMPERATURE FILES ONLY:

TMAX and TMIN:   31 pairs of values of maximum  & minimum daily temperature, "F

In all files, missing values or days beyond the end of a month contain a 999.  All monthly blocks must contain 31 values in the precipitation files and 31 pairs of values in the temperature files.

Examples of two months of meteorological files (line is wrapped to display on page format) are shown below.  Each succeeding month is in exactly the same format.  These files are for Great Bend, KS (143218) for January 1948 (48 1) and February 1948 (48 2).

Note:  Data in these files do not have to start and end in the same year.  They must, however, both include the entire period of simulation between the starting and ending years specified.

GTBNDPCP.DAT

143218 1948  1   0.17  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.01  0.00  0.02  0.00  0.00  0.01  0.07  0.00

143218 1948  2   0.00  0.00  0.08  0.00  0.08  0.00  0.00  0.00  0.00  0.02  0.12  0.07  0.05  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.03  0.00  0.00  2.29  0.00  0.00   999   999

GTBNDTMP.DAT

143218 1948  1   26  12  43   7  55  22  49  26  49  26  48  22  70  30  66  31  50  32  48  36  59  35  47  31  40  15  54  11  48  15  32   9  40   7  35  21  41   7  52  20  54  23  47  16  23   1  24  13  28  11  24   9  12   1  22  -8  25   1  29   1  30  21

143218 1948  2   31  11  26  13  26  15  37  15  30  17  33  10  35  22  25   1  38   9  31  17  29   0  15   7  28 -15  38  11  45  28  64  27  69  31  67  29  52  18  30  20  32  12  39  20  40  30  39  28  53  29  58  38  53  40  51  30  42  27 999 999 999 999

All data values in the files must be separated by a space or a return and must include enough 999 entries to have 31 values in a block in the precipitation files and 31 pairs of values in the temperature files.

Creating an Input Data File for POTYLDR

An important key to using POTYLDR is to gather and convert information for the sub-basin of interest into an input file that adequately represents the conditions in the area for the period of the simulation.  Here, the user must examine the area and then refer to the references or sources described herein for guidance in selecting values.  An example problem follows which was the basis for the input information placed in file, GTBNDANL.DAT, and for the example files included on the Distribution Diskette and printed in this manual.

Example Problem:

The Cheyenne Bottoms Wildlife Refuge is an important ecological resource for Kansas and North America.  It is located in central Kansas about eight miles northeast of Great Bend in northern Barton County.  It's main natural tributary watershed is Blood Creek.  A portion of Blood Creek (61 mi2) was gauged by the USGS from 1962-1980 and reported as Blood Creek near Boyd.  The watershed of Blood Creek can be divided into three sub-basins each of about 20,000 acres and each represented by a different precipitation station.  One of those sub-basins can best be modeled using Great Bend as the precipitation station.  Other applicable precipitation stations are Claflin and Bison.  Great Bend, however, is the best station to represent temperature for the entire basin.  This example represents a 10-year (1979-1988)  simulation of annual values for this sub-basin using land uses and watershed information selected to illustrate some changes in land use conditions in the sub-basin that might be plausible.  These are not intended to be representative of actual conditions, however.

The conditions are simplified for this example.  The user can obtain land use conditions from Farm Facts (Kansas State Board of Agriculture), local Soil Conservation Service (USDA-SCS) District Offices or Agricultural Stabilization and Conservation Service (USDA-ASCS) Offices or other sources for Barton County.  Soils information is available from the Barton County Soil Survey (USDA-SCS) and the Kansas Irrigation Guide (USDA-SCS) for soils hydrologic group and irrigation class, respectively.  Local County Extension Service personnel are often excellent sources of information on cropping practices, planting dates, etc.  If conditions in the sub-basin being considered are substantially different than for the entire county, judgment is needed to determine appropriate information.  Conservation practices including residue management, terraces, and ponds information is available from local, area, or state SCS offices.  U. S. Geological Survey 7.5-minute topographic survey maps are useful to identify ponds and to determine watershed areas for ponds.  These maps are dated and are usually 5 to 25 years old.  Care must be taken to get the latest estimates on ponds, however, few have been constructed recently, unless they are a part of organized watershed districts.   Large ponds such as watershed dams can be represented by multiples of the typical pond.  Care should be taken to see that the ratio of watershed area to water surface area, etc are adequately represented by the multiples of the typical pond approach.  Color slides which show about two square miles each are available for recent dates from the local ASCS office.  These will show ponds, terraces, and recent conservation practices.  Slides can be purchased through the ASCS.

The information used in this example problem are as follows:

Meteorological Station:		Great Bend, KS, 143218, period of record 1948-1988

Precipitation available in the datafile:  GTBNDPCP.DAT

Temperature available in the datafile:  GTBNDTMP.DAT

Penman equation inputs:	Brunt A = 0.63, Brunt B = 0.034, E = 0.75, RCROP = 0.23

Estimated annual lake evaporation = 61.5 inches

Annual precipitation = 25.5 inches

See Appendix A for help with these values.

Geographical location:		Longitude 98.5o W, Latitude 38.5o N

Monthly meteorological data:	Obtained by triangulation of information from Wichita, Dodge City and Concordia 1st-order stations from Climates of the States (Water Information Center, 1974).  Applicable section for Kansas is found in Appendix B.  RA was obtained from Criddle (1958) for latitude 38.5o N by interpolation, see Appendix B.

MO PSUNS RHD   RA  WIND MMMAXT MMMINT

01  64   78  06.50 14.1 39.4   19.6

02  63   79  08.70 14.5 46.8   24.0

03  66   75  11.30 16.3 55.5   31.3

04  62   77  14.10 16.0 67.8   44.0

05  64   83  15.90 14.8 77.1   54.5

06  75   81  16.70 14.6 87.8   64.4

07  81   75  16.30 12.9 93.5   69.1

08  80   75  14.90 13.9 91.8   67.2

09  81   77  12.40 14.1 82.2   58.4

10  75   77  09.60 13.6 71.2   47.6

11  70   75  07.10 14.0 55.0   31.5

12  68   78  05.90 13.6 44.4   24.2

Land use: 	Land uses are approximately as expected in the area.  Irrigation was assumed to develop during the period on continuous wheat and grain sorghum by conversion of dryland cropping to irrigated.  Land use by year is shown later.

Soil type:		Harney silt loam, Irrigation Class 3, Hydrologic Group B/C (See Appendices  )

Conservation	30% of dryland cropland terraced with graded type terraces (interview with

Practices:		 local SCS personnel and personal observations) residue management on 75% of cropland (interview with local SCS personnel and personal observations)

Ponds:		Typical pond has average watershed area of 160 acres, maximum depth of 12 feet, bottom length and width of 0 feet and side slope of 13:1.  Obtained from study of USGS maps.

Land uses were separated into nine different ones for this example.  It was assumed that the land uses above the ponds were exactly the same as for areas of the sub-basin not above ponds, except there is no irrigated land above the typical pond.

The nine land uses and information for each are as follows:

Note:	All soils are Irrigation Class 3 and Hydrologic Group B/C (use a RCNII value half way between those published for B and for C for all RCNII and FLRCN2 values for all land uses)

Land use 1:	Continuous wheat, irrigated 0% terraced and good residue management (MUL=0.25), RCN (AMC I, II, III):  53, 72, 86

Planting date:  September 25 and harvest date:  June 15

0 acres drain into typical pond (most is in lowland areas below ponds)

Land use 2:	Continuous wheat, 30% terraced and 50% residue management (MUL=0.25), RCN (AMC I, II, III):  53, 72, 86

Planting date:  September 25 and harvest date:  June 15

10 acres drain into typical pond

Land use 3:	Wheat-fallow, 30% terraced and 50% with residue management (MUL=0.20), RCN (AMC I, II, III): 53, 72, 86, for fallow RCN (AMC I, II, III):  58, 76, 89

Planting date:  September 25 and harvest date:  June 15

25 acres drain into typical pond

Land use 4:	Fallow-wheat, 30% terraced and 50% with residue management (MUL=0.20),  RCN (AMC I, II, III): 53, 72, 86, for fallow RCN (AMC I, II, III):  58, 76, 89

Planting date:  September 25 and harvest date:  June 15

25 acres drain into typical pond

Alternating wheat-fallow and fallow-wheat is necessary to account for half of each land use every year and to more nearly simulate actual sub-basin conditions.

Land use 5:	Continuous grain sorghum in conventional tillage,  30% terraced without residue management (MUL=0.05), RCN (AMC I, II, III): 58, 76, 89

Planting date:  May 10 and harvest date:  September 30

10 acres drain into typical pond

Land use 6:	Continuous grain sorghum, irrigated, in conservation tillage,  0% terraced with residue management (MUL=0.20), RCN (AMC I, II, III): 55, 74, 88

Planting date:  May 10 and harvest date:  September 30

0 acres drain into typical pond (most irrigated land is in lowlands below ponds)

Land use 7:	Continuous grain sorghum with conservation tillage, 30% terraced with residue management (MUL=0.20), RCN (AMC I, II, III): 55,74, 88

Planting date:  May 10 and harvest date:  September 30

10 acres drain into typical pond

Land use 8:	Pasture and range in fair condition, no terracing or residue management (MUL=0.10), RCN (AMC I, II, III): 55, 74, 88

Growing dates:  Beginning date:   April 01 and end date:  October 30

40 acres drain into typical pond

Land use 9:	Pasture and range in good condition, no terracing or residue management (MUL=0.20), RCN (AMC I, II, III): 51, 70, 85

Growing dates:  Beginning date:   April 01 and end date:  October 30

40 acres drain into typical pond

Irrigation is practiced on Land Uses No. 01 and 06.

Land Use No. 01 is continuous wheat.  Irrigation could begin on October 20 and continue through May 15 whenever available soil moisture in the upper zone or in the entire upper plus lower zones falls below 50% at which time 2.00 inches of water net is added as infiltration into the upper zone.

Land Use No. 06 is continuous grain sorghum. Irrigation could begin on June 15 and continue through September 05 whenever available soil moisture in the upper zone or in the entire upper plus lower zones falls below 50% at which time 2.00 inches of water net is added as infiltration into the upper zone.

Historical information on number of ponds and percent of each land use is shown below.  These land uses are those BELOW ponds.  If land uses are substantially different below ponds, then the land uses should be adjusted as appropriate.

Year Ponds LU1 LU2 LU3 LU4 LU5 LU6 LU7 LU8 LU9

1979   25  00. 20. 15. 15. 00. 00. 10. 20. 20.

1980   26  00. 20. 15. 15. 00. 00. 10. 20. 20.

1981   27  00. 20. 15. 15. 00. 00. 10. 20. 20.

1982   28  00. 20. 15. 15. 00. 00. 10. 20. 20.

1983   29  00. 20. 15. 15. 01. 01. 08. 20. 20.

1984   30  02. 18. 15. 15. 02. 01. 07. 20. 20.

1985   30  03. 17. 15. 15. 03. 02. 05. 20. 20.			        1986   31  04. 16. 15. 15. 04. 02. 04. 20. 20.

1987   31  05. 15. 15. 15. 05. 03. 02. 20. 20.

1988   31  06. 14. 15. 15. 06. 03. 01. 20. 20.

Once the input file is completed, it must be saved in ASCII format.  The simulation can be run using this file along with the daily precipitation and temperature data input files.

Running the Example Problem

BE SURE YOU HAVE RENAMED, MOVED, OR COPIED ANY FILES FROM PREVIOUS RUNS FROM THE SUBDIRECTORY YOU WANT TO SAVE BEFORE STARTING A NEW RUN.  THESE FILES WILL BE OVERWRITTEN BY THE NEXT SIMULATION.

Be sure you are in the directory that contains the POTYLDR.EXE, GTBNDANL.DAT, GTBNDPCP.DAT, and GTBNDTMP.DAT files.  At the DOS prompt,

Type POTYLDR <Enter>

Type GTBNDANL.DAT <Enter> when prompted for Unit 1

The program should now run and will print messages to the screen to show you how the simulation is progressing.  When the DOS prompt is returned, you can use any text editor such as EDIT on DOS 5 or 6, Wordstar, WordPerfect, etc that will keep files as ASCII and browse through them.  Files produced will depend on the output code selected and the name you gave to the master output file.  Be sure to review the beginning of the master output file to verify that the input data was read correctly by the program.  If you want a printout of this file, open it as a text file in your editor and format it into pages.  Be sure to use a fixed font such as Courier when printing it.  It is in 80-column format.  So, Courier 12  or other 12-pitch or elite font allows it to have reasonable margins in portrait layout.  A printout of GTBNDANL.OUT which shows the beginning, first year, and final summary is contained in Appendix F.  This entire file is on the Distribution Diskette.

The ANNLSURF.OUT, ANNLPERC.OUT, and ANNLIRRG.OUT files do not contain any headings.  They are of marginal value to try to interpret the results of the run.  They are in a form that makes them efficient to import into spreadsheets, etc for further processing.  Be sure that you do not convert these files out of ASCII format when you open them if you intend to use them later.   Printouts of ANNLSURF.OUT,  ANNLPERC.OUT, and ANNLIRRG.OUT with headings added for ease of interpretation are contained in Appendix F.

Post-processing Results of POTYLDR to Estimate Sub-Basin Surface Water Yield and Ground-water Yield

If OUTPUT = 1, 2, 3, or 4, POTYLDR will take the appropriate *.OUT files and the pond and land uses information contained in the input file and calculate water yield for the sub-basin and produces another file, ANNLYLD.OUT.   If OUTPUT = 1 or 3, annual output is produced; with 2 or 4, the file MNTHYLD.OUT is produced.  This ASCII file summarizes the results for the sub-basin.  This file contains information about the sub-basin and descriptive headings for the information in the file.  It includes number of ponds and the seepage and overflow from them along with the percent of each land use and their precipitation excess and percolation for each period.  The post-processing calculates transmission losses for each period for surface yield based upon annual moisture deficit and adjusts the surface yield accordingly.   A printout of ANNLYLD.OUT which has been rearranged to fit on a page is shown in Table 2.

The job of simulation is now complete.  The correctness and reasonableness of results cannot be determined.  The procedure that has been followed with previous studies is to judge the quality of the results as to how well they appear to be simulating conditions in the area.  If they are considered OK, then repeat the process for the remaining sub-basins in the study area.  Next, make estimates or obtain information about effects of withdrawals and/or additions to streamflow.  Sum up the water yield for each period from each sub-basin and add or subtract other effects on streamflow.  Finally, compare the results with an actual gauge record for the sub-basin, if available.  In areas where percolation is an important aspect of streamflow, further consideration of the delay of percolation and the portion of it which actually reaches the outlet of the basin must be determined or estimated in order to make a direct comparison with gauged streamflow.

As with any tool, it is up to the user to operate it and interpret the final results.  The results will be no better than the inputs.  The results will be no better than the ultimate interpretations made from the results.  Of course, the tool must function properly, too.  I believe it is working properly.

References:

Blaney, H.F. and W.D. Criddle, 1962.  Determining consumptive use and irrigation water requirements.  USDA Tech. Bull. 1275.  U.S. Government Printing Office, Washington, DC.

Brunt, D., 1944.  Physical and Dynamical Meteorology.  Cambridge University Press.

Criddle, W.D.  1958.  Methods of computing consumptive use of water.  J. of Irrig. and Drainage Div., Proc. of the American Society of Civil Engineers Vol. 84(IR1): 1-27.

Fenster, C.R., H.I. Owens, and R.H. Follett, 1977.  Conservation tillage for wheat in the Great Plains.  USDA Extension Pamphlet 1190.  U.S. Government Printing Office, Washington, D.C.

Gray, D. M., 1973.  Principles of Hydrology.  Water Information Center, Inc.  Port Washington, NY.

Koelliker, J.K., J.J. Zovne, J.M. Steichen, and M.W. Berry, 1981.  Study to assess water yield changes in the Solomon Basin, Kansas.  Part I - Final Report.  Kansas Water Resources Research Institute.  Manhattan, KS.  123 pp.

Koelliker, J.K., J.J. Zovne, J.M. Steichen, and M.W. Berry, 1982.  Study to assess water yield changes in the Solomon Basin, Kansas.  Part II - User's Manual.  Kansas Water Resources Research Institute.  Manhattan, KS.

Koelliker, J.K., M.J. Brown, and J.J. Zovne, 1983.  Assessment of changes in the precipitation regime of the Republican River Basin.  Working paper for the Bureau of Reclamation, Denver, CO.  Civil Engineering Dept., Kansas State Univ., Manhattan, KS.

Koelliker, J.K., 1984.  An historical perspective on soil and water conservation:  Its effect on surface water supplies.  Paper presented at the 39th Annual Meeting of the Soil and Water Conservation Society of America, Oklahoma City, OK, August 1.  Civil Engineering Dept., Kansas State Univ., Manhattan, KS.

Koelliker, J.K., 1991.  Future water supply for Cheyenne Bottoms Wildlife Refuge, Kansas.  Final report to Howard Needles, Tammen and Bergendoff.  Civil Engineering Dept., Kansas State Univ., Manhattan, KS.

Kutz, R., 1990.  Bureau of Reclamation reservoirs in Kansas.  Presented to the Kansas Academy of Science, March 1990.  Bureau of Reclamation, McCook, NE.

Rawls, W.J., C.A. Onstad, and H.H. Richardson, 1980.  Residue and tillage effects on SCS runoff curve numbers.  Trans. of the ASAE 23: 357-361.

Ritchie, J.T., 1972.  Model for predicting evaporation from a row crop with incomplete cover, pp. 1204-1213..  USDA Soil & Water Conservation Research Division, Blackland Conservation Research Center, Temple, TX.

Sauer, S.P. and F.D. Masch, 1969.  Effects of small structures on water yield in Texas.  In: Effects of Watershed Changes on Streamflow, pp. 118-135.  W.L. Moore and C.W. Morgan, Eds.  Univ. of Texas Press, Austin, TX.  289 pp.

Sharp, A.L., A.E. Gibbs, and W.J. Owens, 1966.  Development of a procedure for estimating the effects of land and watershed treatment on streamflow.  USDA Tech. Bull. 1352.  U.S. Government Printing Office, Washington, DC.

Steichen, J.M., 1983.  Field verification of runoff curve numbers for fallow rotations.  J. of Soil and Water Conservation 38: 496-499.

USDA Soil Conservation Service, 1972.  Hydrology.  National Engg. Handbook, Section 4. Washington, DC.

USDA Soil Conservation Service.  1975+, Kansas Irrigation Guide.  Salina, KS.

Viessman, W. and G.L. Lewis.  1996.  Introduction to Hydrology.  HarperCollins College Publishers, New York, NY.

Water Information Center, Inc., 1974.  Climates of the States, Volume 2.  Port Washington, NY.

Zovne, J.J., T.A. Bean, J.K. Koelliker, and J.A. Anschutz, 1977.  Model to evaluate feedlot runoff control systems.  J. of the Irrig. and Drainage Div., ASCE, 103: 79-92.

Zovne, J.J. and J.K. Koelliker, 1979.  Application of continuous watershed modeling to feedlot runoff management and control.  Report No. EPA-600/2-79-065.  National Technical Information Service, Springfield, VA.

APPENDIX A -- Climatological Information for Calibration of Penman Equation

In Viessman and Lewis (1996), p. 83.

Appendix B -- Monthly Climatological Data for Various Locations

Mean-Monthly Extra-Terrestrial Radiation on a Horizontal Plane

(mm water/day), Northern Hemisphere

(L 560 cal/g),  after Criddle (1958)

-------------------------------- North latitude ---------------------------------

Month	900	800	700	600	500	400	300	200	100	  00

Jan.	0.0	0.0	0.0	1.3	3.6	6.0	8.5	10.8	12.8	14.5

Feb.	0.0	0.0	1.1	3.5	5.9	8.3	10.5	12.3	13.9	15.0

Mar.	0.0	1.8	4.3	6.8	9.1	11.0	12.7	13.9	14.8	15.2

Apr.	7.9	7.8	9.1	11.1	12.7	13.9	14.8	15.2	15.2	14.7

May	14.9	14.6	13.6	14.6	15.4	15.9	16.0	15.7	15.0	13.9

June	18.1	17.8	17.0	16.5	16.7	16.7	16.5	15.8	14.8	13.4

July	16.8	16.5	15.8	15.7	16.1	16.3	16.2	15.7	14.8	13.5

Aug.	11.2	10.6	11.4	12.7	13.9	14.8	15.3	15.3	15.0	14.2

Sept.	2.6	4.0	6.8	8.5	10.5	12.2	13.5	14.4	14.9	14.9

Oct.	0.0	0.2	2.4	4.7	7.1	9.3	11.3	12.9	14.1	15.0

Nov.	0.0	0.0	0.1	1.9	4.3	6.1	9.1	11.2	13.1	14.6

Dec.	0.0	0.0	0.0	0.9	3.0	5.5	7.9	10.3	12.4	14.3

_________________________________________________________

KANSAS FIRST-ORDER STATIONS MEAN ANNUAL VALUES NEEDED FOR PENMAN EQUATION IN POTYLDR

taken from Water Information Center, Inc (1974)

Topeka, KS

Longitude   Latitude

95.63 W  39.07 N

Lake Evap, in.  50.0

Precip., in.    33.1

Moist Def., in. 17.0

Rel Humidity Reported at 6:00 am

Avg. %  Avg. Rel. Solar Rad. Avg. wind  Avg. air temp, deg F

Mo  Sunshine  Hum.,%   mm, water  speed, mph Max.     Min.

1     50       77       6.2       10.4      38.8     18.6

2     52       80       8.5       10.7      43.9     22.8

3     55       79      11.1       12.9      54.0     31.1

4     54       79      14.0       12.9      66.1     42.8

5     57       85      15.9       11.3      73.0     52.5

6     65       84      16.7       11.5      84.7     62.9

7     66       86      16.3        9.3      90.7     67.7

8     68       86      14.8        9.6      89.2     66.0

9     74       82      12.3       10.0      81.2     56.8

10     67       82       9.5       10.3      70.6     43.1

11     57       78       6.4       10.1      54.2     31.0

12     52       79       5.7       11.0      42.5     21.9

Concordia, KS

Longitude   Latitude

97.67 W  39.57 N

Lake Evap, in.  55.0

Precip., in.    25.4

Moist Def., in. 29.6

Rel Humidity Reported6:00 am

Avg. %  Avg. Rel. Solar Rad. Avg. wind  Avg. air temp, deg F

Mo  Sunshine  Hum.,%   mm, water  speed, mph Max.     Min.

1     60       78       6.0        7.7      37.6     18.6

2     60       78       8.3        8.1      42.9     22.9

3     63       77      11.0        9.3      52.2     31.0

4     63       77      13.9        9.8      64.8     43.1

5     65       77      15.9        8.6      73.6     52.8

6     72       79      16.7        8.0      84.5     63.2

7     79       76      16.3        7.1      91.2     68.7

8     76       79      14.8        6.9      89.6     67.0

9     72       78      12.2        7.7      80.8     58.1

10     70       78       9.3        7.6      67.2     46.2

11     64       78       6.2        7.8      52.4     31.1

12     58       79       5.5        7.5      41.5     22.6

Wichita, KS

Longitude   Latitude

97.42 W  37.65 N

Lake Evap, in.  58.0

Precip., in.    30.7

Moist Def., in. 27.3

Rel Humidity Reported at 6:00 am

Avg. %  Avg. Rel. Solar Rad. Avg. wind  Avg. air temp, deg F

Mo  Sunshine  Hum.,%   mm, water  speed, mph Max.     Min.

1     59       78       7.4       13.5      41.3     22.6

2     63       81       9.5       13.7      47.8     26.5

3     66       74      11.9       15.4      56.4     34.1

4     63       78      14.4       15.3      67.0     45.3

5     61       84      16.0       13.9      75.3     54.5

6     75       84      16.6       13.6      86.0     64.6

7     84       74      16.2       11.9      92.3     69.4

8     82       72      15.1       12.6      91.7     68.0

9     84       71      12.9       12.7      82.8     60.5

10     65       79      10.4       13.7      70.8     49.5

11     71       74       7.8       13.2      55.1     35.0

12     67       77       6.8       13.0      44.6     26.3

Dodge City, KS

Longitude   Latitude

99.93 W   37.8 N

Lake Evap, in.  64.0

Precip., in.    20.6

Moist Def., in. 43.5

Rel Humidity Reported at 6:00 am

Avg. %  Avg. Rel. Solar Rad. Avg. wind  Avg. air temp, deg F

Mo  Sunshine  Hum.,%   mm, water  speed, mph Max.     Min.

1     68       78       7.2       14.6      42.0     18.6

2     63       78       9.4       15.2      47.2     22.7

3     67       76      11.8       17.3      55.6     29.6

4     61       76      14.3       16.7      66.8     40.8

5     66       83      15.9       15.8      75.2     50.8

6     75       79      16.6       15.7      86.1     60.9

7     78       76      16.3       13.9      92.9     66.9

8     78       78      15.0       13.6      91.3     65.8

9     78       74      12.8       15.1      82.5     57.2

10     75       75      10.3       14.5      70.6     44.9

11     70       75       7.6       14.9      55.3     30.1

12     68       78       6.7       14.2      44.5     22.2

Goodland, KS

Longitude   Latitude

101.7 W  39.37 N

Lake Evap, in.  55.5

Precip., in.    18.2

Moist Def., in. 37.5

Rel Humidity Reported at 5:00 am MDT

Avg. %  Avg. Rel. Solar Rad. Avg. wind  Avg. air temp, deg F

Mo  Sunshine  Hum.,%   mm, water  speed, mph Max.     Min.

1     68       76       6.1       12.4      39.3     12.3

2     63       79       8.4       12.5      44.1     16.3

3     66       78      11.1       14.4      50.5     22.5

4     64       78      13.9       14.1      61.0     33.3

5     66       83      15.9       13.6      72.1     44.0

6     75       79      16.7       12.9      82.9     54.1

7     78       80      16.3       11.9      90.9     60.2

8     78       82      14.8       11.5      89.1     58.6

9     77       74      12.3       11.9      80.6     48.5

10     75       75       9.4       11.6      67.7     36.5

11     70       76       6.2       11.9      52.1     22.8

12     68       76       5.6       11.3      41.7     15.5

APPENDIX C -- Soils Information for Irrigation Classes

IRRIGATION DESIGN CLASS DESCRIPTIONS FOR SOILS IN POTYLDR

From Kansas SCS Irrigation Guide (1975)

============================================================================

Irri-

gation  Profile

soil     depth

class     fta  				Soil class description

____________________________________________________________________________

1	3.0	Deep soils with silt loam or silty clay loam surface layers and slowly to very slowly permeable heavy clay and claypan subsoils.

2	3.0	Deep soils with silty clay or clay textures throughout. Surface infiltration and subsoil permeability are very slow when the soil is moist.  Shrinkage from drying causes extensive cracking, resulting in high infiltration rates until swelling occurs.

3	5.0	Deep soils with silt loam, loam, clay loam, or silty clay loam surface layers and clay loam, silty clay loam, or silty clay subsoils.  Subsoil permeability is slow to moderately slow.  Shrinkage cracks resulting from drying in the soils with more clayey subsoil textures give a relatively high initial infiltration rate.

4	2.5	Moderately deep soils with silt loam, clay loam, or silty clay loam surface layers and clay loam or silty clay subsoils with predominantly moderately slow permeability.

5	5.0	Deep soils with silt loam, loam, clay loam, or silty clay loam surface layers and subsoils.  Subsoil permeability is moderate to moderately slow.

6	3.0	Moderately deep soils with silt loam or loam surface layers and loam, clay loam, or silty clay subsoils with moderateto moderately slow permeability.

7	5.0	Deep soils with silt loam, loam or very fine sandy loam surface layers and moderately permeable, medium textured subsoils.

8	2.5	Moderately deep soils with silt loam, loam or very fine sandy loam surface layers and moderately permeable clay loam, loam, or silt loam subsoils.

9	5.0	Deep soils with fine sandy and loam surface layers and subsoils that have moderately rapid permeability.  Available water capacity is moderate to low.

10	5.0	Soils are moderately deep over sand with sandy loam to loam surface layers and moderately rapid to rapidly permeable subsoils with low available water capacity.

11	5.0	Deep soils with loamy fine sand or loamy sand surface layers and moderately rapid to rapidly permeable subsoils.

12	5.0	Deep rapidly permeable soils with sand or fine sand textures throughout.

============================================================================

aSoil profile depth used in POTYLDR is as shown or 4.0 feet whichever is less.

SOIL MOISTURE PROPERTIES FOR IRRIGATION SOIL CLASSES USED IN POTYLDR

=============================================================================================================

(1)	(2)	(3)	(4)	(5)	(6)	(7)	(8)	(9)

Permanent	Soil moisture	Upper

Available water	Field capacity	wilting point	at saturation	limit	Empir-

Irri-		Soil	inches	inches	inches	inches	stage 2a	ical

gation SCS	profile					evapor.	coeff.

soil	soil	depth	Uppera  Lowerb	Upper    Lower	Upper    Lower	Upper   Lower	U	c'

class group  ft 	zone     zone 	zone     zone	zone     zone	zone    zone	in.	in. day-2

_____________________________________________________________________________________________________________

1	D	3.0 	2.6 	2.7 	4.6	9.4 	2.0 	6.7 	5.8 	11.8	0.47	0.20

2	D	3.0	l.5 	2.9 	4.4 	9.4 	2.9 	6.5 	6.2 	11.5	0.47	0.20

3	C	5.0 	2.5 	5.7 	4.5 	14.2 	2.0 	8.5 	5.7 	16.3	0.39	0.18

4	C	2.5	2.4 	2.5 	4.6 	7.0 	2.2 	4.5 	5.7 	8.2	0.39	0.18

5	B	5.0 	2.5 	6.7 	4.5 	13.9 	2.0 	7.2 	5.7 	15.8	0.39	0.18

6	 B	3.0	2.6 	4.2 	4.3 	9.1 	1.7 	4.9 	5.5 	10.4	0.39	0.18

7	B	5.0 	2.4 	6.6 	4.0 	13.7 	1.6 	7.1 	5.4 	15.4	0.35	0.16

8	B	2.5 	2.4 	3.3 	4.0 	6.8 	1.6 	3.5 	5.4 	7.7	0.35	0.16

9	B	5.0 	2.4 	5.2 	3.8 	9.2 	1.4 	4.0 	5.3 	13.9	0.31	0.14

10	B	5.0 	2.2 	4.1 	3.5 	7.0 	1.3 	2.9 	5.2 	13.2	0.31	0.14

11	B	5.0 	1.5 	4.1 	2.3 	7.0 	0.8 	2.9 	4.8 	13.2	0.28	0.13

12	A	5.0 	1.0 	2.5 	1.7 	4.3 	0.7 	1.8 	4.8 	12.9	0.24	0.13

=============================================================================================================

aUpper zone thickness is 1.0 feet.

bLower zone thickness is 3.0 feet except where soil profile depth limitations occur.

APPENDIX D -- Runoff Curve Number Information for POTYLDR

Cultivation Practices, Crop Types, and Runoff Curve Numbers for Antecedent Moisture Condition II on Group B and C Soils.

(original taken from Koelliker et al., 1981)

NOTE:	These values were used in the South Fork Solomon River Basin Above Webster Reservoir Study. Subsequently, these values were used in the Cheyenne Bottoms Wildlife Area, Kansas Water Supply Study (Koelliker, 1991) where most soils were a Group B/C.  "Mulched" in the original table has been changed to the more current term of "with residue".

Runoff Curve Numbera

Description 									Group B	Group C

_____________________________________________________________________________________

pasture 	70 	80

wheat 	74	82

wheat, graded terrace	70	77

wheat, level terrace with open ends	68	75

wheat, level terrace with closed ends	56	 *

wheat, with residue	70	78

wheat, graded terrace, with residue 	66	73

wheat, level terrace with open ends, with residue 	64	71

wheat, level terrace with closed ends, with residue 	52	 *

row crop 	78	85

row crop, graded terrace	71	78

row crop, level terrace with closed ends 	69	76

row crop, level terrace with closed ends	61	 *

fallow		83	88

fallow, graded terrace	77	83

fallow, level terrace with open ends	75	81

fallow, level terrace with closed ends	64	 *

fallow, with residue	79	84

fallow, graded terrace, with residue	73	79

fallow, level terrace with open ends, with residue	71	77

fallow, level terrace with closed ends, with residue	60	 *

===========================================

aUser should recognize that hydrologic condition and amount of residue can affect the curve   number by about  3.

* This practice is not typically encountered in the watershed.

Sources:	USDA, Soil Conservation Service.  1972.  Hydrology.  National Engineering Handbook, Section 4.  Washington, DC.

Holste, Authur H.  1980.  Telephone interview by J. K. Koelliker.  Soil Conservation Service, Hays, KS.

Runoff Curve Numbers for Selected Areas For Antecedent Moisture Condition II

(for Ia=0.2*S)  Selected Conditions Only -- Reproduced from Tr-55a.

Fully developed urban areas (vegetation established)

Cover Description

Average %		Curve numbers for

Cover Type						Impervious	         hydrologic soil group

Hydrologic Condition				Area			A	B	C	D

Open space (lawns, parks,golf courses, cemeteries, etc)

Poor condition (grass cover < 50%)					68	79	86	89

Fair condition (grass cover 50 to 75%)					49	69	79	84

Good condition (grass cover > 75%)					39	61	74	80

Impervious areas:

Paved parking lots, roofs, driveways, etc.

(excluding right-of-way)							98	98	98	98

Streets and roads:

Paved: curbs and storm sewers (excluding right-of-way)			98	98	98	98

Paved: open ditches (including right-of-way)				83	89	92	93

Gravel (including right-of-way)						76	85	89	91

Dirt (including right-of-way)						72	82	87	89

Urban districts:

Commercial and business					85		96	96	96	96

Industrial							72		81	88	91	93

Residential districts by average lot size:

1/8-acre or less (townhouses)				65		77	85	90	92

1/4-acre						38		61	75	83	87

1/3-acre						30		57	72	81	86

1/2-acre						25		54	70	80	85

1-acre							20		51	68	79	84

2-acre							12		46	65	77	82

Rural areas		Cover

Hydrologic	Hydrologic Soil Group

Land Use			Treatment or Practice		Condition	A	B	C	D

Fallow				Straight row				------		77	86	91	94

Row crops			Straight row				poor		72	81	88	91

Straight row				good		67	78	85	89

Contoured				poor		70	79	84	88

Contoured				good		65	75	82	86

Contoured and terraced		poor		66	74	80	82

Contoured and terraced		good		62	71	78	81

Small grain			Straight row				poor		65	76	84	88

Straight row				good		63	75	83	87

Contoured				poor		63	74	82	85

Contoured				good		61	73	81	84

Contoured and terraced		poor		61	72	79	82

Contoured and terraced		good		59	70	78	81

Pasture or range						poor		68	79	86	89

fair		49	69	79	84

good		39	61	74	80

Meadow								good		30	58	71	78

Woods									poor		45	66	77	83

fair		46	60	73	79

good		25	55	70	77

Farmsteads									59	74	82	86

Roads 	(dirt, including right-of-way)				----		72	82	87	89

(hard surface, including right of way)			----		74	84	90	92

aUSDA, SCS.  1986.  Urban Hydrology for Small Watersheds. Technical Release 55.

Table to convert from one runoff curve number to the corresponding one with different antecedent moisture conditions.

Source: USDA, SCS, National Engineering Handbook (1972)

CN for 	CN for 	CN for

AMC II	AMC I 	AMC III

100	100	100

99	97	100

98	94	99

97	91	99

96	89	99

95	87	98

94	85	98

93	83	98

92	81	97

91	80	97

90	78	96

89	76	96

88	75	95

87	73	95

86	72	94

85	70	94

84	68	93

83	67	93

82	66	92

81	64	92

80	63	91

79	62	91

78	60	90

77	59	89

76	58	89

75	57	88

74	55	88

73	54	87

72	53	86

71	52	86

70	51	85

69	50	84

68	48	84

67	47	83

66	46	82

65	45	82

64	44	81

63	43	80

62	42	79

61	41	78

60	40	78

59	39	77

58	38	76

57	37	75

56	36	75

55	35	74

APPENDIX E --	Climate, Precipitation, and Temperature Data Files Prepared  Previously for use With POTYLDR (included on weather and climate data diskette)

Index	Station	Filename -	Years	Applicable

Station	Name 	(1st 5 letters)	Available	Temperature

141699	Colby, KS	COLBY	1920-1980	Same

142980 	Garden City	GRDNC	1920-1988	Same

143218	Great Bend, KS	GTBND	1948-1993	Same

143810	Horton, KS	HORTN	1900-1975	Same

144399	Manhattan, KS	MNHTN	1958-1986	Same

144399	Konza Prairie	KONZA	1979-1988	Same

APPENDIX F:	PRINTOUTS FROM EXAMPLE PROBLEM FROM POTYLDR

ABRIDGED COPY OF FILE:  GTBNDANL.OUT

ASCII FILENAME: GTBNDANL.OUT   SIMULATION RUN MADE: 10-25-1993 at 10:40

Location of sub-basin: CHEYENNE BOTTOMS SUB-BASIN W/GREAT BEND

ECHO OF INPUT VALUES FOR METEOROLOGICAL STATION, POND, AND LAND USES

INPUTS FOR PET EQUATION

brunt a= .630  brunt b= .034  e= .750  rcrop= .230

TYPICAL POND CHARACTERISTICS

maximum height = 12.0 ft., bottom length =   0. ft. & width =   0. ft.

ss= 13.:1, and seepage rate= .200 in./day

STATIONS FOR USED FOR THIS SUB-BASIN

precipitation: filename= GTBNDPCP.DAT, station index= 143218

temperature: filename =  GTBNDTMP.DAT, station index= 143218

starting year= 1979 and ending year=  1988

OTHER INFORMATION

Total No. of land uses=  9; No. of land uses irrigated=  2

Sub-basin total area=  20000. acres

output code=  3

MONTHLY CLIMATIC INFORMATION

mo  psun,%   rhd,%   rad,mm  wind,mph mmmaxt,F mmmint,F

1    64.     78.      6.50    14.1     39.4     19.6

2    63.     79.      8.70    14.5     46.8     24.0

3    66.     75.     11.30    16.3     55.5     31.3

4    62.     77.     14.10    16.0     67.8     44.0

5    64.     83.     15.90    14.8     77.1     54.5

6    75.     81.     16.70    14.6     87.8     64.4

7    81.     75.     16.30    12.9     93.5     69.1

8    80.     75.     14.90    13.9     91.8     67.2

9    81.     77.     12.40    14.1     82.2     58.4

10    75.     77.      9.60    13.6     71.2     47.6

11    70.     75.      7.10    14.0     55.0     31.5

12    68.     78.      5.90    13.6     44.4     24.2

LAND USES INFORMATION (acres= amount of each land use in drainage area of pond)

no soil crop acres cnI II  III st.mo dy ed.mo dy  r p t  mul cnI II  III

1   3   1     0.  53. 72. 86.   10  5     6 15   1 1 2  .25  0.  0.  0.

2   3   1    10.  53. 72. 86.   10  5     6 15   1 2 2  .25  0.  0.  0.

3   3   1    25.  53. 72. 86.   10  5     6 15   2 2 2  .20 58. 76. 89.

4   3   1    25.  53. 72. 86.   10  5     6 15   3 2 2  .20 58. 76. 89.

5   3   2    10.  58. 76. 89.    5 15     9 30   1 2 2  .05  0.  0.  0.

6   3   2     0.  55. 74. 88.    5 15     9 30   1 1 2  .20  0.  0.  0.

7   3   2    10.  55. 74. 88.    5 15     9 30   1 2 2  .20  0.  0.  0.

8   3   5    40.  55. 74. 88.    4  1    10 30   1 2 1  .15  0.  0.  0.

9   3   5    40.  51. 70. 85.    4  1    10 30   1 2 1  .20  0.  0.  0.

IRRIGATED LAND USE INFORMATION

no st.mo dy ed.mo dy  %asm inches

1    10 20     5 15   50.  2.00

6     6 15     9  5   50.  2.00

POND AND LAND USE INFORMATION YEAR-BY-YEAR IN SUB-BASIN

no.  annual amount of each land use in percent of area in sub-basin

year pnds  01   02   03   04   05   06   07   08   09   10   11   12   13  14

1979  25   .0 20.0 15.0 15.0   .0   .0 10.0 20.0 20.0

1980  26   .0 20.0 15.0 15.0   .0   .0 10.0 20.0 20.0

1981  27   .0 20.0 15.0 15.0   .0   .0 10.0 20.0 20.0

1982  28   .0 20.0 15.0 15.0   .0   .0 10.0 20.0 20.0

1983  29   .0 20.0 15.0 15.0  1.0  1.0  8.0 20.0 20.0

1984  30  2.0 18.0 15.0 15.0  2.0  1.0  7.0 20.0 20.0

1985  30  3.0 17.0 15.0 15.0  3.0  2.0  5.0 20.0 20.0

1986  31  4.0 16.0 15.0 15.0  4.0  2.0  4.0 20.0 20.0

1987  31  5.0 15.0 15.0 15.0  5.0  3.0  2.0 20.0 20.0

1988  31  6.0 14.0 15.0 15.0  6.0  3.0  1.0 20.0 20.0

Calculated Crop Coefficients for Crops Included on one or More Land Uses

Crop/Month 1    2    3    4    5    6    7    8    9   10   11   12

1      .43  .42  .58  .79  .80  .34  .00  .00  .00  .94  .65  .44

2      .00  .00  .00  .00  .15  .67 1.00 1.00  .57  .00  .00  .00

3      .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00

4      .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00

5      .00  .00  .00  .56  .74  .92 1.00  .97  .78  .55  .00  .00

6      .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00

7      .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00  .00

STATION:  CHEYENNE BOTTOMS SUB-BASIN W/GREAT BEND           1979 TO 1988

TYPICAL POND VARIABLES:

(A) BASE DIMENSIONS--    0.FEET BY   0. FEET

(B) SIDE SLOPE--  RUN:RISE = 13. :1

(C) MAXIMUM DEPTH--  12.0 FEET

(D) MAXIMUM POND VOLUME--    107.27 ACRE-INCHES

(E) MAXIMUM POND SURFACE AREA--    2.23 ACRES

(F) SEEPAGE RATE--   .200 INCHES/DAY

(G) DRAINAGE AREA-- 160.00 ACRES

LAND USES AREA VARIABLES:

LAND USE  1

(A) AREA IN POND WATERSHED--     .00 ACRES

(B) CROP--  WHEAT

(C) SOIL TYPE--  3 (SCS IRRIGATION CLASS)

(D) CONSERVATION PRACTICES-- NO FLOW INTO POND

TERRACES-- GRADED              RESIDUE FACTOR = .25

(E) RUNOFF CURVE NUMBERS: AMC I   = 53.

AMC II  = 72.

AMC III = 86.

(F) IRRIGATED UPPER ZONE AVAILABLE SOIL MOISTURE BELOW: 50.%

WITH 2.00 INCHES BEGINNING 10-20 AND ENDING  5-15

LAND USE  2

(A) AREA IN POND WATERSHED--   10.00 ACRES

(B) CROP--  WHEAT

(C) SOIL TYPE--  3 (SCS IRRIGATION CLASS)

(D) CONSERVATION PRACTICES--    FLOW INTO POND

TERRACES-- GRADED              RESIDUE FACTOR = .25

(E) RUNOFF CURVE NUMBERS: AMC I   = 53.

AMC II  = 72.

AMC III = 86.

LAND USE  3

(A) AREA IN POND WATERSHED--   25.00 ACRES

(B) CROP--  WHEAT

(C) SOIL TYPE--  3 (SCS IRRIGATION CLASS)

(D) CONSERVATION PRACTICES--    FLOW INTO POND

TERRACES-- GRADED              RESIDUE FACTOR = .20

(E) RUNOFF CURVE NUMBERS: AMC I   = 53.

AMC II  = 72.

AMC III = 86.

(G) CROP ROTATION WITH -- FALLOW

FALLOW RUNOFF CURVE NUMBERS: AMC I   = 58.

AMC II  = 76.

AMC III = 89.

LAND USE  4

(A) AREA IN POND WATERSHED--   25.00 ACRES

(B) CROP--  WHEAT

(C) SOIL TYPE--  3 (SCS IRRIGATION CLASS)

(D) CONSERVATION PRACTICES--    FLOW INTO POND

TERRACES-- GRADED              RESIDUE FACTOR = .20

(E) RUNOFF CURVE NUMBERS: AMC I   = 53.

AMC II  = 72.

AMC III = 86.

(G) CROP ROTATION WITH -- FALLOW

FALLOW RUNOFF CURVE NUMBERS: AMC I   = 58.

AMC II  = 76.

AMC III = 89.

LAND USE  5

(A) AREA IN POND WATERSHED--   10.00 ACRES

(B) CROP--  SORGHUM

(C) SOIL TYPE--  3 (SCS IRRIGATION CLASS)

(D) CONSERVATION PRACTICES--    FLOW INTO POND

TERRACES-- GRADED              RESIDUE FACTOR = .05

(E) RUNOFF CURVE NUMBERS: AMC I   = 58.

AMC II  = 76.

AMC III = 89.

LAND USE  6

(A) AREA IN POND WATERSHED--     .00 ACRES

(B) CROP--  SORGHUM

(C) SOIL TYPE--  3 (SCS IRRIGATION CLASS)

(D) CONSERVATION PRACTICES-- NO FLOW INTO POND

TERRACES-- GRADED              RESIDUE FACTOR = .20

(E) RUNOFF CURVE NUMBERS: AMC I   = 55.

AMC II  = 74.

AMC III = 88.

(F) IRRIGATED UPPER ZONE AVAILABLE SOIL MOISTURE BELOW: 50.%

WITH 2.00 INCHES BEGINNING  6-15 AND ENDING  9- 5

LAND USE  7

(A) AREA IN POND WATERSHED--   10.00 ACRES

(B) CROP--  SORGHUM

(C) SOIL TYPE--  3 (SCS IRRIGATION CLASS)

(D) CONSERVATION PRACTICES--    FLOW INTO POND

TERRACES-- GRADED              RESIDUE FACTOR = .20

(E) RUNOFF CURVE NUMBERS: AMC I   = 55.

AMC II  = 74.

AMC III = 88.

LAND USE  8

(A) AREA IN POND WATERSHED--   40.00 ACRES

(B) CROP--  PASTURE

(C) SOIL TYPE--  3 (SCS IRRIGATION CLASS)

(D) CONSERVATION PRACTICES--    FLOW INTO POND

TERRACES-- NONE                RESIDUE FACTOR = .15

(E) RUNOFF CURVE NUMBERS: AMC I   = 55.

AMC II  = 74.

AMC III = 88.

LAND USE  9

(A) AREA IN POND WATERSHED--   40.00 ACRES

(B) CROP--  PASTURE

(C) SOIL TYPE--  3 (SCS IRRIGATION CLASS)

(D) CONSERVATION PRACTICES--    FLOW INTO POND

TERRACES-- NONE                RESIDUE FACTOR = .20

(E) RUNOFF CURVE NUMBERS: AMC I   = 51.

AMC II  = 70.

AMC III = 85.

****** ANNUAL SUMMARY ******

WATER ACCOUNT FOR THE POND IN ACRE-INCHES FOR 1979

-------------------------------------------------------------------------------

INFLOWS                         OUTFLOWS

----------------------  -------------------------------------- DELTA   ENDING

PRECIP. PRECIP. EXCESS  SURFACE EVAP.   SEEPAGE     DISCHARGE  VOLUME  DEPTH,FT

16.7      269.3           59.8         70.1          77.2     79.0    10.86

LAND USE NO. 1 SNOW PACK MOISTURE STORAGE INFORMATION (INCHES):

MOISTURE STORED IN SNOW PACK ON DEC. 31---  .49

CHANGE IN STORAGE DURING 1979 =  .49

CHANGE IN SOIL MOISTURE = (INPUTS) - (OUTPUTS) +/- (CHANGE IN SNOW STORAGE)

LAND USE NO.  1  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

23.86   8.00       4.89        3.31      .00   21.30      1.81    16.41

LAND USE NO.  2  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

23.86    .00       4.67        2.60      .00   15.83       .21    14.81

LAND USE NO.  3  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

23.86    .00       4.87        3.31      .41   14.07       .63    15.23

LAND USE NO.  4  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

23.86    .00       4.59        2.44      .11   14.01      2.17    16.77

LAND USE NO.  5  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

23.86    .00       4.66        1.32      .00   18.25      -.93    13.67

LAND USE NO.  6  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

23.86  14.00       4.56        2.76      .00   29.38       .59    15.19

LAND USE NO.  7  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

23.86    .00       4.56        1.12      .00   18.15      -.54    14.06

LAND USE NO.  8  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

23.86    .00       4.52        1.13      .00   18.54      -.91    13.69

LAND USE NO.  9  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

23.86    .00       4.48         .74      .10   18.68      -.71    13.89

****** ANNUAL SUMMARY ******

WATER ACCOUNT FOR THE POND IN ACRE-INCHES FOR 1980

-------------------------------------------------------------------------------

INFLOWS                         OUTFLOWS

----------------------  -------------------------------------- DELTA   ENDING

PRECIP. PRECIP. EXCESS  SURFACE EVAP.   SEEPAGE     DISCHARGE  VOLUME  DEPTH,FT

24.6      116.4           87.7         98.6          18.9    -64.2     6.22

LAND USE NO. 1 SNOW PACK MOISTURE STORAGE INFORMATION (INCHES):

MOISTURE STORED IN SNOW PACK ON DEC. 31---  .00

CHANGE IN STORAGE DURING 1980 = -.49

CHANGE IN SOIL MOISTURE = (INPUTS) - (OUTPUTS) +/- (CHANGE IN SNOW STORAGE)

LAND USE NO.  1  WATER BALANCE (INCHES) FOR LAND USE FOR 1980

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

18.26  10.00       4.21         .64     1.41   22.93      -.39    16.02

LAND USE NO.  2  WATER BALANCE (INCHES) FOR LAND USE FOR 1980

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

18.26    .00       3.84         .64      .12   16.25     -2.04    12.77

LAND USE NO.  3  WATER BALANCE (INCHES) FOR LAND USE FOR 1980

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

18.26    .00       3.72         .67      .70   14.11      -.37    14.86

LAND USE NO.  4  WATER BALANCE (INCHES) FOR LAND USE FOR 1980

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

18.26    .00       4.12        1.21     3.37   13.31     -3.21    13.56

LAND USE NO.  5  WATER BALANCE (INCHES) FOR LAND USE FOR 1980

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

18.26    .00       3.84         .78      .00   16.23     -2.04    11.62

LAND USE NO.  6  WATER BALANCE (INCHES) FOR LAND USE FOR 1980

-------------------------------------------------------------------------------

INPUTS                       OUTPUTS                    SOIL MOISTURE

-------------    ---------------------------------------  ---------------

PRECIP. IRRIG.    INTRCPTN PRECIP.EXCESS PERC.   AET       CHANGE  TOTAL

18.26  18.00       3.63        1.95      .65   31.61     -1.01    14.18

ANNUAL OUTPUT FOR POND AND LAND USES CONTINUE THROUGH 1988.

***** FINAL SUMMARY *****

SUB-BASIN:  CHEYENNE BOTTOMS SUB-BASIN W/GREAT BEND

PERIOD SIMULATED:  1979 TO 1988 = 10 YEARS

METEOROLOGICAL SUMMARY

AVERAGE ANNUAL LAKE EVAPORATION= 62.30 INCHES

AVERAGE ANNUAL PRECIPITATION= 24.71 INCHES

PRECIPITATION RANGE= 22.72 INCHES

(FROM A LOW OF 14.27 INCHES TO A HIGH OF 36.99 INCHES)

AVERAGE MOISTURE DEFICIT= 37.59 INCHES

AVERAGE POTENTIAL EVAPOTRANSPIRATION= 57.08 INCHES

SUMMARY OF POND OPERATIONS (ALL VOLUMES ARE IN ACRE-INCHES)

NUMBER OF YEARS HAVING AN OVERFLOW =  6

AVERAGE NUMBER OF OVERFLOWS PER YEAR HAVING AN OVERFLOW =  4.00

AVERAGE DAILY OVERFLOW VOLUME =   50.23

MAXIMUM DAILY OVERFLOW VOLUME =  280.75

AVERAGE OVERFLOW VOLUME PER YEAR HAVING AN OVERFLOW =    200.90

TOTAL OVERFLOW VOLUME=   1205.41

AVERAGE ANNUAL DIRECT PRECIPITATION VOLUME=   29.84

AVERAGE ANNUAL PRECIPITATION EXCESS VOLUME FLOWING INTO POND=   253.46

AVERAGE ANNUAL EVAPORATION VOLUME=   77.30

AVERAGE ANNUAL SEEPAGE VOLUME=   85.09

AVERAGE ANNUAL OVERFLOW VOLUME =    120.54

SUMMARY OF AVERAGE ANNUAL WATER BALANCE FOR EACH LAND USE

LAND USE NO.  1

IRRIGATION ADDED =  8.60 INCHES

PRECIPITATION EXCESS =  2.15 INCHES

PERCOLATION = 2.76 INCHES

INTERCEPTION = 5.18 INCHES

EVAPOTRANSPIRATION = 23.05 INCHES

CHANGE IN SOIL MOISTURE =  .16 INCHES

LAND USE NO.  2

IRRIGATION ADDED =   .00 INCHES

PRECIPITATION EXCESS =  1.35 INCHES

PERCOLATION =  .85 INCHES

INTERCEPTION = 4.96 INCHES

EVAPOTRANSPIRATION = 17.88 INCHES

CHANGE IN SOIL MOISTURE = -.34 INCHES

LAND USE NO.  3

IRRIGATION ADDED =   .00 INCHES

PRECIPITATION EXCESS =  2.37 INCHES

PERCOLATION = 1.97 INCHES

INTERCEPTION = 5.01 INCHES

EVAPOTRANSPIRATION = 15.49 INCHES

CHANGE IN SOIL MOISTURE = -.14 INCHES

LAND USE NO.  4

IRRIGATION ADDED =   .00 INCHES

PRECIPITATION EXCESS =  1.78 INCHES

PERCOLATION = 2.22 INCHES

INTERCEPTION = 5.00 INCHES

EVAPOTRANSPIRATION = 15.88 INCHES

CHANGE IN SOIL MOISTURE = -.18 INCHES

LAND USE NO.  5

IRRIGATION ADDED =   .00 INCHES

PRECIPITATION EXCESS =  1.97 INCHES

PERCOLATION =  .13 INCHES

INTERCEPTION = 4.96 INCHES

EVAPOTRANSPIRATION = 18.06 INCHES

CHANGE IN SOIL MOISTURE = -.41 INCHES

LAND USE NO.  6

IRRIGATION ADDED = 14.20 INCHES

PRECIPITATION EXCESS =  2.82 INCHES

PERCOLATION = 1.70 INCHES

INTERCEPTION = 4.83 INCHES

EVAPOTRANSPIRATION = 29.74 INCHES

CHANGE IN SOIL MOISTURE = -.20 INCHES

LAND USE NO.  7

IRRIGATION ADDED =   .00 INCHES

PRECIPITATION EXCESS =  1.82 INCHES

PERCOLATION =  .37 INCHES

INTERCEPTION = 4.75 INCHES

EVAPOTRANSPIRATION = 18.15 INCHES

CHANGE IN SOIL MOISTURE = -.40 INCHES

LAND USE NO.  8

IRRIGATION ADDED =   .00 INCHES

PRECIPITATION EXCESS =  1.43 INCHES

PERCOLATION =  .02 INCHES

INTERCEPTION = 4.71 INCHES

EVAPOTRANSPIRATION = 18.93 INCHES

CHANGE IN SOIL MOISTURE = -.41 INCHES

LAND USE NO.  9

IRRIGATION ADDED =   .00 INCHES

PRECIPITATION EXCESS =  1.02 INCHES

PERCOLATION =  .12 INCHES

INTERCEPTION = 4.69 INCHES

EVAPOTRANSPIRATION = 19.27 INCHES

CHANGE IN SOIL MOISTURE = -.41 INCHES

SUMMARY OF STATISTICAL DATA

PRECIPITATION FREQUENCY DATA

INTENSITY     FREQUENCY      FREQUENCY    RUNOFF FREQ.

(IN.)        ( %  )         (DAYS)         (DAYS)

>0.0         100.00         697.00         171.00

>0.1          62.27         434.00         171.00

>0.2          46.34         323.00         171.00

>0.3          35.29         246.00         159.00

>0.4          27.55         192.00         142.00

>0.5          20.37         142.00         118.00

>0.6          16.79         117.00         101.00

>0.7          12.91          90.00          82.00

>0.8          10.62          74.00          70.00

>0.9           8.61          60.00          56.00

>1.0           7.03          49.00          47.00

>1.1           5.74          40.00          39.00

>1.2           5.31          37.00          37.00

>1.3           4.73          33.00          33.00

>1.4           4.02          28.00          28.00

>1.5           3.59          25.00          25.00

>1.6           3.30          23.00          23.00

>1.7           2.87          20.00          20.00

>1.8           2.73          19.00          19.00

>1.9           2.30          16.00          16.00

>2.0           2.01          14.00          14.00

>3.0            .72           5.00           5.00

>4.0            .14           1.00           1.00

>5.0            .14           1.00           1.00

>10.            .00            .00            .00

RUNOFF FREQUENCY DATA

INTENSITY        FREQUENCY ( % )                      FREQUENCY (DAYS)

(IN.)  L USE 1 L USE 2 L USE 3 L USE 4     L USE 1 L USE 2 L USE 3 L USE 4

>0.0    100.00  100.00  100.00  100.00      144.00  101.00  134.00  121.00

>0.1     31.94   32.67   28.36   34.71       46.00   33.00   38.00   42.00

>0.2     15.97   15.84   17.16   25.62       23.00   16.00   23.00   31.00

>0.3     11.81    9.90   11.94   15.70       17.00   10.00   16.00   19.00

>0.4      9.72    7.92    9.70    9.92       14.00    8.00   13.00   12.00

>0.5      9.03    7.92    8.21    9.09       13.00    8.00   11.00   11.00

>0.6      6.94    6.93    8.21    4.96       10.00    7.00   11.00    6.00

>0.7      6.25    4.95    6.72    4.13        9.00    5.00    9.00    5.00

>0.8      4.86    2.97    5.97    2.48        7.00    3.00    8.00    3.00

>0.9      4.17    1.98    5.97    1.65        6.00    2.00    8.00    2.00

>1.0      3.47    1.98    5.22    1.65        5.00    2.00    7.00    2.00

>1.1      2.78    1.98    3.73    1.65        4.00    2.00    5.00    2.00

>1.2      2.78    1.98    3.73    1.65        4.00    2.00    5.00    2.00

>1.3      2.78    1.98    3.73    1.65        4.00    2.00    5.00    2.00

>1.4      2.08    1.98    3.73    1.65        3.00    2.00    5.00    2.00

>1.5      2.08    1.98    3.73    1.65        3.00    2.00    5.00    2.00

>1.6      1.39     .99    2.99     .83        2.00    1.00    4.00    1.00

>1.7       .69     .99    2.99     .00        1.00    1.00    4.00     .00

>1.8       .69     .99    2.99     .00        1.00    1.00    4.00     .00

>1.9       .69     .00    2.24     .00        1.00     .00    3.00     .00

>2.0       .69     .00    2.24     .00        1.00     .00    3.00     .00

>3.0       .00     .00     .00     .00         .00     .00     .00     .00

>4.0       .00     .00     .00     .00         .00     .00     .00     .00

>5.0       .00     .00     .00     .00         .00     .00     .00     .00

>10.       .00     .00     .00     .00         .00     .00     .00     .00

ANNUAL OUTPUT FILES, ANNLSURF.OUT, ANNLPERC.OUT, AND ANNLIRRG.OUT

FROM POTYLDR.EXE

Headings added for clarity; they are not printed on the files.  Monthly output files are the same except there is a line for each month of each year.

ANNLSURF.OUT

--INCHES--  -------------------- ALL DEPTHS ARE IN INCHES -------------------

LAKE  POND  POND   ------ VALUES FOR EACH LAND USE IN SIMULATION ------

YEAR PRECP EVAP  EVAP* OVRFLW No.1  No.2  No.3  No.4  No.5  No.6  No.7  No.8  No.9

1979 23.86 60.43   .27   .48  3.31  2.60  3.31  2.44  1.32  2.76  1.12  1.13   .74

1980 18.26 64.56   .39   .12   .64   .64   .67  1.21   .78  1.95   .75   .73   .46

1981 36.99 63.40   .24  2.78  4.58  3.36  5.92  3.66  4.88  4.57  4.33  3.98  3.11

1982 21.95 60.77   .28   .00   .50   .26   .43   .83  1.33  1.34  1.25   .06   .01

1983 23.75 60.25   .27   .00   .77   .41  1.32   .90  1.13  1.49   .98   .74   .44

1984 23.57 62.77   .33   .42  1.95  1.08  1.11  1.75  1.42  1.44  1.28  1.63  1.04

1985 33.53 60.76   .33  2.72  4.84  2.43  6.81  2.34  5.74  7.47  5.31  3.76  2.98

1986 23.10 63.59   .32   .00  1.11   .40   .58  1.52   .62  2.08   .54   .33   .05

1987 27.78 62.25   .38  1.01  3.27  2.09  3.32  2.00  2.43  3.83  2.52  1.89  1.37

1988 14.27 64.18   .17   .00   .49   .27   .27  1.18   .04  1.31   .13   .03   .02

ANNLPERC.OUT

-------- ALL DEPTHS ARE IN INCHES OVER WATERSHED ---------

POND  ------ VALUES FOR EACH LAND USE IN SIMULATION ------

YEAR  SEEP* No.1  No.2  No.3  No.4  No.5  No.6  No.7  No.8  No.9

1979   .44   .00   .00   .41   .11   .00   .00   .00   .00   .10

1980   .62  1.41   .12   .70  3.37   .00   .65   .00   .00   .00

1981   .64  8.84  3.49  5.12  5.28  1.32  4.17  2.16   .00   .00

1982   .51   .86   .00   .00  2.12   .00  1.08   .00   .00   .00

1983   .42  2.15   .00  1.24   .00   .00  1.29   .00   .00   .00

1984   .56  3.01   .71  2.87  2.57   .00  2.69   .73   .23  1.09

1985   .73  7.51  4.20  6.07  5.31   .00  4.13   .73   .00   .00

1986   .53  1.83   .00   .00  2.78   .00  1.06   .00   .00   .00

1987   .64  2.02   .00  3.27   .32   .00  1.94   .11   .00   .00

1988   .25   .00   .00   .00   .34   .00   .00   .00   .00   .00

ANNLIRRIG.OUT

------------- ALL DEPTHS ARE IN INCHES -------------

------ VALUES FOR EACH LAND USE IN SIMULATION ------

YEAR  No.1  No.2  No.3  No.4  No.5  No.6  No.7  No.8  No.9

1979  8.00   .00   .00   .00   .00 14.00   .00   .00   .00

1980 10.00   .00   .00   .00   .00 18.00   .00   .00   .00

1981  8.00   .00   .00   .00   .00  8.00   .00   .00   .00

1982 10.00   .00   .00   .00   .00 12.00   .00   .00   .00

1983  4.00   .00   .00   .00   .00 18.00   .00   .00   .00

1984  6.00   .00   .00   .00   .00 18.00   .00   .00   .00

1985  8.00   .00   .00   .00   .00 14.00   .00   .00   .00

1986 10.00   .00   .00   .00   .00 12.00   .00   .00   .00

1987  8.00   .00   .00   .00   .00 12.00   .00   .00   .00

1988 14.00   .00   .00   .00   .00 16.00   .00   .00   .00

_________________

*Pond depths are in inches over the drainage area of the typical pond

DAILY.OUT FILE FOR FIRST FOUR LAND USES

(ABRIDGED for brevity; file contained on Distribution Diskette)

Sub-basin simulated: CHEYENNE BOTTOMS SUB-BASIN W/GREAT BEND

Daily information for 1988, the last year of the simulation period.

-------- LAND USE 1 --------   -------- LANDUSE 2 --------   -------- LAND USE 3 ---------   -------- LAND USE 4 --------

MO DY TMX TMN PREC. PET IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO.

6  1  80  59  .79  .23  .00  .10  .00  .00  .04 15.69  .00  .10  .00  .00  .01 12.00  .00  .10  .00  .00  .01 12.11  .00  .10  .00  .00  .03 16.29

6  2  80  62  .00  .21  .00  .00  .00  .00  .07 15.62  .00  .00  .00  .00  .05 11.95  .00  .00  .00  .00  .05 12.06  .00  .00  .00  .00  .18 16.11

6  3  83  56  .05  .26  .00  .05  .00  .00  .07 15.55  .00  .05  .00  .00  .04 11.91  .00  .05  .00  .00  .04 12.02  .00  .05  .00  .00  .09 16.02

6  4  84  60  .00  .25  .00  .00  .00  .00  .08 15.46  .00  .00  .00  .00  .05 11.86  .00  .00  .00  .00  .05 11.97  .00  .00  .00  .00  .14 15.88

6  5  83  56  .00  .26  .00  .00  .00  .00  .09 15.38  .00  .00  .00  .00  .05 11.81  .00  .00  .00  .00  .05 11.92  .00  .00  .00  .00  .06 15.82

6  6  85  57  .00  .27  .00  .00  .00  .00  .09 15.29  .00  .00  .00  .00  .05 11.76  .00  .00  .00  .00  .05 11.87  .00  .00  .00  .00  .05 15.78

6  7  95  67  .00  .30  .00  .00  .00  .00  .10 15.19  .00  .00  .00  .00  .05 11.71  .00  .00  .00  .00  .05 11.82  .00  .00  .00  .00  .04 15.74

6  8 103  68  .00  .33  .00  .00  .00  .00  .11 15.08  .00  .00  .00  .00  .05 11.66  .00  .00  .00  .00  .05 11.76  .00  .00  .00  .00  .03 15.71

6  9 100  63  .00  .32  .00  .00  .00  .00  .10 14.98  .00  .00  .00  .00  .05 11.61  .00  .00  .00  .00  .05 11.71  .00  .00  .00  .00  .03 15.68

6 10  85  58  .00  .27  .00  .00  .00  .00  .09 14.89  .00  .00  .00  .00  .04 11.57  .00  .00  .00  .00  .04 11.68  .00  .00  .00  .00  .03 15.65

6 11  88  61  .00  .28  .00  .00  .00  .00  .09 14.80  .00  .00  .00  .00  .04 11.54  .00  .00  .00  .00  .04 11.64  .00  .00  .00  .00  .03 15.62

6 12  90  67  .00  .27  .00  .00  .00  .00  .09 14.71  .00  .00  .00  .00  .03 11.50  .00  .00  .00  .00  .04 11.60  .00  .00  .00  .00  .02 15.60

6 13  90  68  .00  .26  .00  .00  .00  .00  .09 14.62  .00  .00  .00  .00  .03 11.47  .00  .00  .00  .00  .03 11.57  .00  .00  .00  .00  .02 15.57

6 14  89  64  .00  .27  .00  .00  .00  .00  .09 14.54  .00  .00  .00  .00  .03 11.44  .00  .00  .00  .00  .03 11.53  .00  .00  .00  .00  .02 15.55

6 15  88  67  .00  .25  .00  .00  .00  .00  .08 14.45  .00  .00  .00  .00  .03 11.41  .00  .00  .00  .00  .03 11.50  .00  .00  .00  .00  .02 15.53

6 16  90  61  .00  .28  .00  .00  .00  .00  .09 14.36  .00  .00  .00  .00  .03 11.38  .00  .00  .00  .00  .03 11.47  .00  .00  .00  .00  .02 15.51

6 17  94  65  .00  .30  .00  .00  .00  .00  .10 14.27  .00  .00  .00  .00  .03 11.35  .00  .00  .00  .00  .03 11.44  .00  .00  .00  .00  .02 15.49

6 18  98  70  .00  .31  .00  .00  .00  .00  .10 14.17  .00  .00  .00  .00  .03 11.32  .00  .00  .00  .00  .03 11.41  .00  .00  .00  .00  .02 15.47

6 19 101  72  .00  .32  .00  .00  .00  .00  .10 14.07  .00  .00  .00  .00  .03 11.29  .00  .00  .00  .00  .03 11.38  .00  .00  .00  .00  .02 15.46

6 20 108  70  .00  .34  .00  .00  .00  .00  .10 13.97  .00  .00  .00  .00  .03 11.26  .00  .00  .00  .00  .03 11.35  .00  .00  .00  .00  .02 15.44

6 21 105  74  .00  .33  .00  .00  .00  .00  .09 13.87  .00  .00  .00  .00  .03 11.23  .00  .00  .00  .00  .03 11.32  .00  .00  .00  .00  .02 15.42

6 22 102  75  .00  .32  .00  .00  .00  .00  .09 13.79  .00  .00  .00  .00  .03 11.20  .00  .00  .00  .00  .03 11.29  .00  .00  .00  .00  .02 15.41

6 23 100  72  .00  .31  .00  .00  .00  .00  .08 13.70  .00  .00  .00  .00  .02 11.18  .00  .00  .00  .00  .02 11.27  .00  .00  .00  .00  .02 15.39

6 24  98  71  .00  .31  .00  .00  .00  .00  .08 13.63  .00  .00  .00  .00  .02 11.16  .00  .00  .00  .00  .02 11.25  .00  .00  .00  .00  .02 15.37

6 25 100  72  .00  .31  .00  .00  .00  .00  .07 13.55  .00  .00  .00  .00  .02 11.14  .00  .00  .00  .00  .02 11.22  .00  .00  .00  .00  .02 15.36

6 26 100  73  .00  .31  .00  .00  .00  .00  .07 13.48  .00  .00  .00  .00  .02 11.12  .00  .00  .00  .00  .02 11.20  .00  .00  .00  .00  .01 15.34

6 27  94  69  .04  .29  .00  .04  .00  .00  .05 13.43  .00  .04  .00  .00  .02 11.10  .00  .04  .00  .00  .02 11.19  .00  .04  .00  .00  .01 15.33

6 28  94  72  .00  .27  .00  .00  .00  .00  .06 13.37  .00  .00  .00  .00  .02 11.09  .00  .00  .00  .00  .02 11.17  .00  .00  .00  .00  .01 15.31

6 29  97  71  .00  .30  .00  .00  .00  .00  .06 13.31  .00  .00  .00  .00  .02 11.07  .00  .00  .00  .00  .02 11.15  .00  .00  .00  .00  .01 15.30

6 30  97  69  .50  .30  .00  .10  .00  .00  .04 13.67  .00  .10  .00  .00  .01 11.46  .00  .10  .00  .00  .01 11.54  .00  .10  .00  .00  .01 15.69

7  1  80  60  .00  .23  .00  .00  .00  .00  .13 13.54  .00  .00  .00  .00  .13 11.33  .00  .00  .00  .00  .14 11.40  .00  .00  .00  .00  .01 15.67

7  2  83  59  .00  .25  .00  .00  .00  .00  .05 13.48  .00  .00  .00  .00  .05 11.27  .00  .00  .00  .00  .06 11.34  .00  .00  .00  .00  .01 15.66

7  3  90  60  .00  .28  .00  .00  .00  .00  .04 13.44  .00  .00  .00  .00  .04 11.23  .00  .00  .00  .00  .05 11.29  .00  .00  .00  .00  .01 15.65

7  4  98  69  .00  .30  .00  .00  .00  .00  .04 13.40  .00  .00  .00  .00  .04 11.19  .00  .00  .00  .00  .04 11.26  .00  .00  .00  .00  .01 15.63

7  5  97  75  .00  .28  .00  .00  .00  .00  .03 13.37  .00  .00  .00  .00  .03 11.16  .00  .00  .00  .00  .03 11.22  .00  .00  .00  .00  .01 15.62

7  6  97  75  .00  .28  .00  .00  .00  .00  .03 13.34  .00  .00  .00  .00  .03 11.13  .00  .00  .00  .00  .03 11.19  .00  .00  .00  .00  .01 15.61

7  7  95  72  .00  .28  .00  .00  .00  .00  .03 13.32  .00  .00  .00  .00  .03 11.11  .00  .00  .00  .00  .03 11.17  .00  .00  .00  .00  .01 15.60

7  8  92  69  .00  .27  .00  .00  .00  .00  .02 13.29  .00  .00  .00  .00  .02 11.08  .00  .00  .00  .00  .03 11.14  .00  .00  .00  .00  .01 15.58

7  9  91  68  .13  .27  .00  .10  .00  .00  .02 13.30  .00  .10  .00  .00  .02 11.09  .00  .10  .00  .00  .02 11.15  .00  .10  .00  .00  .01 15.60

7 10  90  65  .07  .28  .00  .07  .00  .00  .02 13.28  .00  .07  .00  .00  .02 11.07  .00  .07  .00  .00  .02 11.12  .00  .07  .00  .00  .01 15.59

7 11  91  64  .00  .28  .00  .00  .00  .00  .02 13.26  .00  .00  .00  .00  .02 11.05  .00  .00  .00  .00  .02 11.10  .00  .00  .00  .00  .01 15.58

7 12  96  67  .00  .29  .00  .00  .00  .00  .02 13.24  .00  .00  .00  .00  .02 11.03  .00  .00  .00  .00  .02 11.08  .00  .00  .00  .00  .01 15.57

7 13 101  67  .82  .31  .00  .10  .00  .00  .02 13.94  .00  .10  .00  .00  .02 11.73  .00  .10  .00  .00  .02 11.78  .00  .10  .00  .00  .01 16.28

7 14 101  76  .00  .31  .00  .00  .00  .00  .02 13.92  .00  .00  .00  .00  .02 11.71  .00  .00  .00  .00  .02 11.76  .00  .00  .00  .00  .01 16.27

7 15 101  72  .48  .31  .00  .10  .00  .00  .02 14.29  .00  .10  .00  .00  .02 12.07  .00  .10  .00  .00  .02 12.12  .00  .10  .04  .00  .17 16.44

OUTPUT FOR SUB-BASIN FROM POTYLDR.EXE IN FILE, ANNLYLD.OUT

Output on a monthly basis is similar except there is a line for each month of the simulation period.  The output has been split horizontally to fit onto this page and the YEAR Column is repeated.

SUMMARY OF SURFACE AND PERCOLATION/GROUNDWATER YIELD FROM

CHEYENNE BOTTOMS SUB-BASIN W/GREAT BEND

FOR HISTORICAL LAND USES WITH TIME

TOTAL SUB-BASIN AREA =  20000. ACRES AND TYPICAL POND WATERSHED =  160. ACRES

ALL VALUES UNLESS SPECIFIED ARE IN INCHES

ESTIMATED,ACRE-FEET TRLS        LAKE MOIST. NO. OVR WTRSHD  LAND USE 01    LAND USE 02    LAND USE 03

YEAR  SURF   PERC   TOTAL FCTR PRECIP EVAP DEFCT PNDS SURF SEEP  -%- SURF PERC  -%- SURF PERC  -%- SURF PERC

1979  2050.   310.  2360. 1.29 23.86 60.43 36.57   25  .48  .44   .0 3.31  .00 20.0 2.60  .00 15.0 3.31  .41

1980   624.  1272.  1896. 1.60 18.26 64.56 46.30   26  .12  .62   .0  .64 1.41 20.0  .64  .12 15.0  .67  .70

1981  5561.  4354.  9914. 1.11 36.99 63.40 26.41   27 2.78  .64   .0 4.58 8.84 20.0 3.36 3.49 15.0 5.92 5.12

1982   364.   720.  1084. 1.35 21.95 60.77 38.82   28  .00  .51   .0  .50  .86 20.0  .26  .00 15.0  .43  .00

1983   749.   494.  1243. 1.29 23.75 60.25 36.50   29  .00  .42   .0  .77 2.15 20.0  .41  .00 15.0 1.32 1.24

1984  1360.  2467.  3827. 1.36 23.57 62.77 39.20   30  .42  .56  2.0 1.95 3.01 18.0 1.08  .71 15.0 1.11 2.87

1985  5338.  4901. 10239. 1.12 33.53 60.76 27.23   30 2.72  .73  3.0 4.84 7.51 17.0 2.43 4.20 15.0 6.81 6.07

1986   527.  1071.  1598. 1.40 23.10 63.59 40.49   31  .00  .53  4.0 1.11 1.83 16.0  .40  .00 15.0  .58  .00

1987  2562.  1431.  3993. 1.25 27.78 62.25 34.47   31 1.01  .64  5.0 3.27 2.02 15.0 2.09  .00 15.0 3.32 3.27

1988   239.   188.   427. 1.77 14.27 64.18 49.91   31  .00  .25  6.0  .49  .00 14.0  .27  .00 15.0  .27  .00

AVG.  1937.  1721.  3658. 1.21

LAND USE 04    LAND USE 05    LAND USE 06    LAND USE 07    LAND USE 08    LANDUSE 09

YEAR   -%- SURF PERC  -%- SURF PERC  -%- SURF PERC  -%- SURF PERC  -%- SURF PERC  -%- SURF PERC

1979  15.0 2.44  .11   .0 1.32  .00   .0 2.76  .00 10.0 1.12  .00 20.0 1.13  .00 20.0  .74  .10

1980  15.0 1.21 3.37   .0  .78  .00   .0 1.95  .65 10.0  .75  .00 20.0  .73  .00 20.0  .46  .00

1981  15.0 3.66 5.28   .0 4.88 1.32   .0 4.57 4.17 10.0 4.33 2.16 20.0 3.98  .00 20.0 3.11  .00

1982  15.0  .83 2.12   .0 1.33  .00   .0 1.34 1.08 10.0 1.25  .00 20.0  .06  .00 20.0  .01  .00

1983  15.0  .90  .00  1.0 1.13  .00  1.0 1.49 1.29  8.0  .98  .00 20.0  .74  .00 20.0  .44  .00

1984  15.0 1.75 2.57  2.0 1.42  .00  1.0 1.44 2.69  7.0 1.28  .73 20.0 1.63  .23 20.0 1.04 1.09

1985  15.0 2.34 5.31  3.0 5.74  .00  2.0 7.47 4.13  5.0 5.31  .73 20.0 3.76  .00 20.0 2.98  .00

1986  15.0 1.52 2.78  4.0  .62  .00  2.0 2.08 1.06  4.0  .54  .00 20.0  .33  .00 20.0  .05  .00

1987  15.0 2.00  .32  5.0 2.43  .00  3.0 3.83 1.94  2.0 2.52  .11 20.0 1.89  .00 20.0 1.37  .00

1988  15.0 1.18  .34  6.0  .04  .00  3.0 1.31  .00  1.0  .13  .00 20.0  .03  .00 20.0  .02  .00

ABRIDGED OUTPUT FILE GTBNDMON.OUT FROM POTYLDR.EXE

NOTE:  Only portion of file different than GTBNDANL.OUT is shown for one year.

****** ANNUAL SUMMARY ******

WATER ACCOUNT FOR THE POND IN ACRE-INCHES FOR 1979

-------------------------------------------------------------------------------

INFLOWS                     OUTFLOWS

----------------------  -------------------------------  DELTA   ENDING

MONTH PRECIP. PRECIP. EXCESS  SURFACE EVAP. SEEPAGE DISCHARGE  VOLUME  DEPTH,FT

JAN.    .0           .0            .0          .0       .0        .0      .00

FEB.    .0          1.7            .1          .1       .0       1.5     2.94

MAR.   1.2         23.3           1.8         2.8       .0      19.8     7.03

APR.    .9           .0           3.7         4.1       .0      -6.9     6.19

MAY     .9           .2           3.6         3.2       .0      -5.7     5.24

JUNE    .7           .0           3.2         2.1       .0      -4.5     4.13

JULY   2.4        145.9           5.6         3.8     41.6      97.3    11.80

AUG.    .9           .0          17.1        12.1       .0     -28.2    10.61

SEPT    .3           .0          10.8         9.4       .0     -20.0     9.54

OCT.   5.7         97.9           6.2         8.0     35.5      53.9     9.38

NOV.   1.6           .3           4.5        12.7       .0     -15.5    11.41

DEC.   2.2           .0           3.2        11.8       .0     -12.8    10.86

TOT.  16.7        269.3          59.8        70.1     77.2      79.0    10.86

LAND USE NO. 1 SNOW PACK MOISTURE STORAGE INFORMATION (INCHES):

MOISTURE STORED IN SNOW PACK ON DEC. 31---  .49

CHANGE IN STORAGE DURING 1979 =  .49

CHANGE IN SOIL MOISTURE = (INPUTS) - (OUTPUTS) +/- (CHANGE IN SNOW STORAGE)

LAND USE NO.  1  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                   OUTPUTS                    SOIL MOISTURE

--------------  ------------------------------------   ---------------

MONTH PRECIP. IRRIG.  INTRCPTN PRECIP.EXCESS PERC.  AET       CHANGE  TOTAL

JAN.   1.74     .00      .15        .00       .00    .00        .00   14.60

FEB.    .10     .00      .28        .00       .00    .14       1.26   15.86

MAR.   3.36     .00      .71        .23       .00   1.79        .63   16.50

APR.   1.29    2.00      .60        .00       .00   3.52       -.83   15.67

MAY    1.62    4.00      .58        .00       .00   4.67        .37   16.04

JUNE   1.77     .00      .66        .03       .00   2.56      -1.48   14.55

JULY   6.80     .00      .83       2.01       .00   2.26       1.70   16.25

AUG.    .45     .00      .24        .00       .00    .57       -.36   15.89

SEPT    .18     .00      .18        .00       .00    .29       -.29   15.60

OCT.   4.57    2.00      .34       1.04       .00   3.50       1.63   17.23

NOV.    .75     .00      .23        .00       .00   1.26       -.67   16.55

DEC.   1.23     .00      .09        .00       .00    .74       -.14   16.41

TOT.  23.86    8.00     4.89       3.31       .00  21.30       1.81   16.41

LAND USE NO.  2  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                   OUTPUTS                    SOIL MOISTURE

--------------  ------------------------------------   ---------------

MONTH PRECIP. IRRIG.  INTRCPTN PRECIP.EXCESS PERC.  AET       CHANGE  TOTAL

JAN.   1.74     .00      .08        .00       .00    .00        .00   14.60

FEB.    .10     .00      .14        .00       .00    .14       1.48   16.08

MAR.   3.36     .00      .71        .23       .00   1.79        .63   16.71

APR.   1.29     .00      .60        .00       .00   3.52      -2.83   13.88

MAY    1.62     .00      .58        .00       .00   2.55      -1.51   12.36

JUNE   1.77     .00      .66        .00       .00   1.27       -.16   12.20

JULY   6.80     .00      .83       1.83       .00   1.50       2.65   14.85

AUG.    .45     .00      .24        .00       .00    .57       -.36   14.48

SEPT    .18     .00      .18        .00       .00    .29       -.29   14.19

OCT.   4.57     .00      .34        .54       .00   2.31       1.30   15.50

NOV.    .75     .00      .23        .00       .00   1.19       -.60   14.89

DEC.   1.23     .00      .07        .00       .00    .68       -.08   14.81

TOT.  23.86     .00     4.67       2.60       .00  15.83        .21   14.81

LAND USE NO.  3  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                   OUTPUTS                    SOIL MOISTURE       --------------  ------------------------------------   ---------------

MONTH PRECIP. IRRIG.  INTRCPTN PRECIP.EXCESS PERC.  AET       CHANGE  TOTAL

JAN.   1.74     .00      .15        .00       .00    .00        .00   14.60

FEB.    .10     .00      .28        .00       .00    .28       1.12   15.72

MAR.   3.36     .00      .71        .18       .00   2.01        .46   16.18

APR.   1.29     .00      .60        .00       .00   1.09       -.40   15.78

MAY    1.62     .00      .58        .00       .00   1.06       -.02   15.76

JUNE   1.77     .00      .66        .00       .00   1.26       -.15   15.61

JULY   6.80     .00      .83       2.37       .41   1.89       1.29   16.90

AUG.    .45     .00      .24        .00       .00    .97       -.76   16.14

SEPT    .18     .00      .18        .00       .00    .32       -.32   15.82

OCT.   4.57     .00      .34        .74       .00   3.21        .21   16.03

NOV.    .75     .00      .23        .01       .00   1.26       -.68   15.35

DEC.   1.23     .00      .07        .00       .00    .72       -.12   15.23

TOT.  23.86     .00     4.87       3.31       .41  14.07        .63   15.23

LAND USE NO.  4  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                   OUTPUTS                    SOIL MOISTURE

--------------  ------------------------------------   ---------------

MONTH PRECIP. IRRIG.  INTRCPTN PRECIP.EXCESS PERC.  AET       CHANGE  TOTAL

JAN.   1.74     .00      .04        .00       .00    .00        .00   14.60

FEB.    .10     .00      .09        .01       .00    .12       1.58   16.18

MAR.   3.36     .00      .71        .23       .09   1.79        .54   16.72

APR.   1.29     .00      .60        .00       .00   3.52      -2.83   13.89

MAY    1.62     .00      .58        .00       .00   2.56      -1.52   12.37

JUNE   1.77     .00      .66        .00       .00   1.28       -.17   12.21

JULY   6.80     .00      .83       1.63       .00   1.51       2.84   15.04

AUG.    .45     .00      .24        .00       .00    .97       -.76   14.28

SEPT    .18     .00      .18        .00       .00    .32       -.32   13.96

OCT.   4.57     .00      .33        .58       .01    .29       3.28   17.25

NOV.    .75     .00      .24        .00       .00   1.15       -.56   16.69

DEC.   1.23     .00      .09        .00       .00    .52        .08   16.77

TOT.  23.86     .00     4.59       2.44       .11  14.01       2.17   16.77

LAND USE NO.  5  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                   OUTPUTS                    SOIL MOISTURE

--------------  ------------------------------------   ---------------

MONTH PRECIP. IRRIG.  INTRCPTN PRECIP.EXCESS PERC.  AET       CHANGE  TOTAL

JAN.   1.74     .00      .08        .00       .00    .00        .00   14.60

FEB.    .10     .00      .14        .01       .00    .33       1.27   15.87

MAR.   3.36     .00      .71        .13       .00   2.34        .18   16.06

APR.   1.29     .00      .60        .00       .00    .62        .07   16.13

MAY    1.62     .00      .58        .02       .00    .90        .11   16.24

JUNE   1.77     .00      .66        .00       .00   4.36      -3.26   12.98

JULY   6.80     .00      .83        .41       .00   4.07       1.49   14.47

AUG.    .45     .00      .24        .00       .00   3.17      -2.96   11.51

SEPT    .18     .00      .18        .00       .00    .33       -.33   11.18

OCT.   4.57     .00      .33        .74       .00    .24       3.18   14.36

NOV.    .75     .00      .24        .00       .00   1.29       -.70   13.66

DEC.   1.23     .00      .07        .00       .00    .59        .01   13.67

TOT.  23.86     .00     4.66       1.32       .00  18.25       -.93   13.67

LAND USE NO.  6  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                   OUTPUTS                    SOIL MOISTURE

--------------  ------------------------------------   ---------------

MONTH PRECIP. IRRIG.  INTRCPTN PRECIP.EXCESS PERC.  AET       CHANGE  TOTAL

JAN.   1.74     .00      .04        .00       .00    .00        .00   14.60

FEB.    .10     .00      .09        .02       .00    .23       1.45   16.05

MAR.   3.36     .00      .71        .13       .00   2.01        .50   16.56

APR.   1.29     .00      .60        .00       .00   1.09       -.40   16.16

MAY    1.62     .00      .58        .00       .00    .90        .14   16.29

JUNE   1.77    4.00      .66        .00       .00   5.08        .03   16.33

JULY   6.80    2.00      .83       1.95       .00   7.43      -1.41   14.92

AUG.    .45    8.00      .24        .00       .00   7.60        .61   15.53

SEPT    .18     .00      .18        .00       .00   2.78      -2.78   12.75

OCT.   4.57     .00      .33        .65       .00    .58       2.93   15.67

NOV.    .75     .00      .24        .00       .00   1.13       -.54   15.13

DEC.   1.23     .00      .06        .00       .00    .54        .06   15.19

TOT.  23.86   14.00     4.56       2.76       .00  29.38        .59   15.19

LAND USE NO.  7  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                   OUTPUTS                    SOIL MOISTURE

--------------  ------------------------------------   ---------------

MONTH PRECIP. IRRIG.  INTRCPTN PRECIP.EXCESS PERC.  AET       CHANGE  TOTAL

JAN.   1.74     .00      .02        .00       .00    .00        .00   14.60

FEB.    .10     .00      .11        .02       .00    .12       1.57   16.17

MAR.   3.36     .00      .71        .13       .00   2.01        .50   16.67

APR.   1.29     .00      .60        .00       .00   1.09       -.40   16.27

MAY    1.62     .00      .58        .00       .00    .90        .14   16.41

JUNE   1.77     .00      .66        .00       .00   4.38      -3.27   13.14

JULY   6.80     .00      .83        .31       .00   4.14       1.52   14.65

AUG.    .45     .00      .24        .00       .00   3.27      -3.06   11.59

SEPT    .18     .00      .18        .00       .00    .36       -.36   11.24

OCT.   4.57     .00      .33        .65       .00    .20       3.31   14.54

NOV.    .75     .00      .24        .00       .00   1.13       -.54   14.00

DEC.   1.23     .00      .06        .00       .00    .54        .06   14.06

TOT.  23.86     .00     4.56       1.12       .00  18.15       -.54   14.06

LAND USE NO.  8  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                   OUTPUTS                    SOIL MOISTURE

--------------  ------------------------------------   ---------------

MONTH PRECIP. IRRIG.  INTRCPTN PRECIP.EXCESS PERC.  AET       CHANGE  TOTAL

JAN.   1.74     .00      .01        .00       .00    .00        .00   14.60

FEB.    .10     .00      .08        .02       .00    .13       1.60   16.20

MAR.   3.36     .00      .71        .15       .00   2.11        .39   16.58

APR.   1.29     .00      .60        .00       .00   2.48      -1.79   14.79

MAY    1.62     .00      .58        .00       .00   3.03      -1.99   12.80

JUNE   1.77     .00      .66        .00       .00   2.37      -1.26   11.55

JULY   6.80     .00      .83        .31       .00   3.16       2.50   14.04

AUG.    .45     .00      .24        .00       .00   2.90      -2.69   11.35

SEPT    .18     .00      .18        .00       .00    .37       -.37   10.99

OCT.   4.57     .00      .34        .65       .00    .30       3.21   14.20

NOV.    .75     .00      .23        .00       .00   1.10       -.51   13.69

DEC.   1.23     .00      .06        .00       .00    .60        .00   13.69

TOT.  23.86     .00     4.52       1.13       .00  18.54       -.91   13.69

LAND USE NO.  9  WATER BALANCE (INCHES) FOR LAND USE FOR 1979

-------------------------------------------------------------------------------

INPUTS                   OUTPUTS                    SOIL MOISTURE

--------------  ------------------------------------   ---------------

MONTH PRECIP. IRRIG.  INTRCPTN PRECIP.EXCESS PERC.  AET       CHANGE  TOTAL

JAN.   1.74     .00      .00        .00       .00    .00        .00   14.60

FEB.    .10     .00      .05        .01       .00    .12       1.66   16.26

MAR.   3.36     .00      .71        .05       .10   2.01        .48   16.74

APR.   1.29     .00      .60        .00       .00   2.48      -1.79   14.95

MAY    1.62     .00      .58        .00       .00   3.13      -2.09   12.87

JUNE   1.77     .00      .66        .00       .00   2.40      -1.29   11.57

JULY   6.80     .00      .83        .20       .00   3.19       2.58   14.15

AUG.    .45     .00      .24        .00       .00   2.96      -2.75   11.40

SEPT    .18     .00      .18        .00       .00    .39       -.39   11.02

OCT.   4.57     .00      .34        .49       .00    .31       3.37   14.39

NOV.    .75     .00      .23        .00       .00   1.15       -.56   13.83

DEC.   1.23     .00      .06        .00       .00    .54        .06   13.89

TOT.  23.86     .00     4.48        .74       .10  18.68       -.71   13.89

OUTPUT FROM THE PROGRAM CONTINUES FOR EACH YEAR OF THE

SIMULATION.  THEN, THE FINAL OUTPUT IS THE SAME AS FOR

ANNUAL IN GTBNDANL.OUT.

APPENDIX G -- POTYLDR.FOR Source Code Printout

C     *****   POTENTIAL YIELD MODEL REVISED (POTYLDR)

C     *****

C     *****   KANSAS STATE UNIVERSITY     CIVIL ENGINEERING DEPT.

C     *****

C     *****   JUNE 1, 1987  plus revisions through October 20, 1993

C     *****

C     *****   complied in Microsoft FORTRAN 5.0 (1989)

$NOFLOATCALLS

$STORAGE:2

C     *****

INTEGER CROP,DGSB,DGSBP,DGSE,DGSEP,FROZE,OUTPUT

INTEGER POND,PREVYR,ROTATE,SOIL,T,TERR

INTEGER YEAR,YEARS,YEND,YSTART

INTEGER SMDI,PERS,SPER,COUNT

REAL IA,IAET,INRCN1,INRCN2,INRCN3,KCROP,KROPKO,MD,LAKEP,LAKEVT

REAL L,LAKEVP,LKEVPT,MMAT,MMMAXT,MMMINT,MUL,NIA,NDPERC,NRNOF

REAL IASTOR

DIMENSION AAETRS(18), ACHSOM(18), AETL(18), AETU(18)

DIMENSION AINTER(18), AMONTH(13), AREA(18), ASTAT(25)

DIMENSION AVLFCL(12), AVLFCU(12), C(12), CTP(25)

DIMENSION CTPR(25), CTRDAY(04), DGSBP(18)

DIMENSION DGSEP(18), DSPERC(18), DSRNFF(18), EO(18)

DIMENSION FCL(12), FCU(12), FLRCN1(18), FLRCN2(18)

DIMENSION FLRCN3(18), FREQ(25), IAET(18), ICROP(18)

DIMENSION INCROP(18), INRCN1(18), INRCN2(18), INRCN3(18)

DIMENSION IMOBEG(18), IDYBEG(18), IMOEND(18),IDYEND(18)

DIMENSION PSCH(18), ARATE(18), AIRRIG(18),TIRRG(18)

DIMENSION ISOIL(18), PERCLU(18), PXCLU(18),PACK(18)

DIMENSION PRYLD(18), SYLDLU(18), PXCLUM(18,12)

DIMENSION MGSBP(18), MGSEP(18), MMAT(12), MMMAXT(12), MMMINT(12)

DIMENSION MO(12), MUL(18), NDIM(12), NDPERC(18), NIA(18)

DIMENSION NPLOT(18), NRNOF(18), PCROP(18), PEACCT(18)

DIMENSION POND(18),PREC(31), PSUNS(12), PWPLZ(12), PWPUZ(12)

DIMENSION PRP(12), LAKEP(12), PNDOT(12), IASTOR(18)

DIMENSION RA(12), RCNI(18), RCNII(18), RCNIII(18), RHD(12)

DIMENSION ROTATE(18), SM(18), SMGWZ(18), SMLZ(18)

DIMENSION SMPD(18), SMSATU(12), SMUZ(18), T(18)

DIMENSION TAVG(31), TERR(18), TMAX(31), TMIN(31), TOTLKE(12)

DIMENSION U(12), WIND(12), PLU(18), PETACT(13), DVAL(18,6)

DIMENSION CTR(25,4), KCROP(7,12), PDACCT(13,8), PRECAC(25,4)

DIMENSION RUNACC(25,25), SMACCT(13,9,18), krop(7), NTERR(4)

INTEGER*4 PINDX, TINDX, PSTIND, TSTIND

CHARACTER*12 FILE2,FILE3,FILE5

character*24 krop

CHARACTER*20 NTERR

CHARACTER*34 HIST, FIXED, STRNG

character*48 location

character*4 amonth,astat,plit

CHARACTER*2 NO

data blnk/'  '/

data no/'NO'/

data krop/'WHEAT','SORGHUM','CORN','SOYBEANS','PASTURE',

1'ALFALFA','FALLOW'/

C     *****

C     *****************************************************************

C     *****  TERRACES TYPES CHANGED TO LITERALS SEPTEMBER 15, 1993 JKK

C     *****

C     *****************************************************************

C     *****

DATA NTERR/'NONE','GRADED','LEVEL','LEVEL, CLOSED-END'/

DATA AMONTH/'JAN.','FEB.','MAR.','APR.','MAY ','JUNE','JULY','AUG.

1','SEPT','OCT.','NOV.','DEC.','TOT.'/

DATA HIST/'FOR HISTORICAL LAND USES WITH TIME'/

DATA FIXED/'FOR LAND USES FIXED WITH TIME'/

DATA ASTAT/'>0.0','>0.1','>0.2','>0.3','>0.4','>0.5','>0.6','>0.7'

1,'>0.8','>0.9','>1.0','>1.1','>1.2','>1.3','>1.4','>1.5','>1.6','>

21.7','>1.8','>1.9','>2.0','>3.0','>4.0','>5.0','>10.'/

DATA AVLFCL/2.7,2.9,5.7,2.5,6.7,4.2,6.6,3.3,5.2,4.1,4.1,2.5/

DATA AVLFCU/2.6,1.5,2.5,2.4,2.5,2.6,2.4,2.4,2.4,2.2,1.5,1.0/

DATA C/0.2,0.2,0.177,0.177,0.177,0.177,0.159,0.159,0.138,0.138,0.1

134,0.131/

DATA FCL/9.4,9.4,14.2,7.0,13.9,9.1,13.7,6.8,9.2,7.0,7.0,4.3/

DATA FCU/4.6,4.4,4.5,4.6,4.5,4.3,4.0,4.0,3.8,3.5,2.3,1.7/

DATA FREQ/0.0,0.1,0.2,0.3,0.4,0.5,0.6,0.7,0.8,0.9,1.0,1.1,1.2,1.3,

11.4,1.5,1.6,1.7,1.8,1.9,2.0,3.0,4.0,5.0,10.0/

DATA NDIM/31,28,31,30,31,30,31,31,30,31,30,31/

DATA PWPLZ/6.7,6.5,8.5,4.5,7.2,4.9,7.1,3.5,4.0,2.9,2.9,1.8/

DATA PWPUZ/2.0,2.9,2.0,2.2,2.0,1.7,1.6,1.6,1.4,1.3,0.8,0.7/

DATA SMSATU/5.8,6.2,5.7,5.7,5.7,5.5,5.4,5.4,5.3,5.2,4.8,4.8/

DATA U/0.47,0.47,0.39,0.39,0.39,0.39,0.35,0.35,0.31,0.31,0.28,0.24

1/

DATA CONTRL,EPRIM,PDT,SMMAXL/1.0,0.5,37.5,2.2/

DATA COUNT,CROP,FROZE,NODSCH,NYDSCH,SOIL/6*0/

DATA AET,AETLZ,AETUZ,AVAILL,AVAILU,CTP/30*0.0/

DATA CTPDAY,CTPR,CTR,CTRDAY,DA,DPERC/132*0.0/

DATA DSCVOL,EVAPLK,EVAPT,EXCESS,EXFILT,H/6*0.0/

DATA IA,PACK,PACKPY,PDVOL,PEAK,PEINT/23*0.0/

DATA PERC,PETACT,PETTAC,PONVOL,PRCPT,SM,TPAREA/36*0.0/

DATA WATER,WET/2*0.0/

DATA TPREC/0.0/

DATA IMOBEG,IDYBEG,IMOEND,IDYEND/72*0/

DATA PSCH,ARATE,AIRRIG,TIRRG/72*0.0/

DATA TSUYLD,TPRYLD,TBSUYL,TTSUYL,TSYLDA,TPYLDA,PRCT,LAKEVT/8*0.0/

C     ******************************************************************

C     *****

C     *****   FOLLOWING LINES REVISED JUNE 1987, April 1991 and Sept. 1993

C     *****     ***** INPUTS *****

C     *****

write(*,2)

2 format(' Enter the name of master the file for this run.',/'    Th

1is file must reside on the disk. ')

open(1,file=' ',status='old')

10 read (1,15) LOCATION

15 format (a48)

read (1,*) file2, file3, file5

open(2,file=FILE2,status='old')

open(3,file=FILE3,status='old')

open(5,file=FILE5,status='UNKNOWN')

C     *****   READ CLIMATOLOGICAL INFO, LAND USE AREAS AND POND PARAMETERS

read (1,*)brunta,bruntb,e,rcrop,output,tarea

read (1,*)dseprt,hmax,l,w,s

read (1,*)pindx,tindx,ystart,yend,nplots,iplots

C     *****

C     ***** NEXT LINE IS PART OF A DROUGHT INDEX ROUTINE NOT CURRENTLY

C     ***** IMPLEMENTED IN POTYLDR.

C     *****

C      read (1,*)jcrop,mbegin,mend,mpond,skplot

CALL GETDAT (IYR,IMON,IDAY)

CALL GETTIM (IHR,IMIN,ISEC,I100TH)

write(5,30) FILE5, IMON,IDAY,IYR,IHR,IMIN,LOCATION

30 FORMAT ('ASCII FILENAME: ',A12,'  SIMULATION RUN MADE: 'I2,'-',I2,

1'-',I4,' at ',I2,':',I2,//,'Location of sub-basin: ',A48,//,'ECHO

2OF INPUT VALUES FOR METEOROLOGICAL STATION, POND, AND LAND USES',

3/)

C     *****   ECHO PRINT NAMELIST VALUES

write (5,31)brunta,bruntb,e,rcrop

write (5,32)hmax,l,w,s,dseprt

write (5,33)file2,pindx,file3,tindx,ystart,yend,nplots,IPLOTS,

1tarea,output

31 format('INPUTS FOR PET EQUATION',/,'brunt a=',f5.3,2X,'brunt b=',

1f5.3,2X,'e=',f5.3,2X,'rcrop='f5.3,/)

32 format('TYPICAL POND CHARACTERISTICS',/,'maximum height =',f5.1,

1' ft., bottom length =',f5.0,' ft. & width =',f5.0,' ft.',/,'ss=',

2f4.0,':1, and seepage rate=',f5.3,' in./day',/)

33 format('STATIONS FOR USED FOR THIS SUB-BASIN',/,'precipitation: fi

1lename= ',A12,', station index= 'I6,/,'temperature: filename =  ',

2A12,', station index= ',I6,/,'starting year=',i5,' and ending year

3=',I6,//,'OTHER INFORMATION',/,'Total No. of land uses= ',I2,'; No

4. of land uses irrigated= ',I2,/,'Sub-basin total area= ',F7.0,

5' acres',/,'output code= ',I2,//)

PREVYR = YSTART

YEARS = YEND-YSTART+1

C     *****   READ THE MONTHLY AVERAGE CLIMATOLOGICAL DATA

READ (1,50) (MO(I),PSUNS(I),RHD(I),RA(I),WIND(I),MMMAXT(I),

1MMMINT(I),I=1,12)

write(5,51)

50 format(i2,2f4.0,f6.2,3f5.1)

51 format('MONTHLY CLIMATIC INFORMATION',/,'mo  psun,%   rhd,%   rad,

1mm  wind,mph mmmaxt,F mmmint,F')

WRITE (5,52) (MO(I),PSUNS(I),RHD(I),RA(I),WIND(I),MMMAXT(I),

1MMMINT(I),I=1,12)

52 FORMAT (I2,F7.0,F8.0,F10.2,F8.1,2F9.1)

DO 53 I=1,12

MMAT(I)=(MMMAXT(I)+MMMINT(I))/2

53 CONTINUE

GOTO (55,56,55,56,55,56) OUTPUT

C     ******************************************************************

C     *****

55 open(4,file='annlsurf.out',status='UNKNOWN')

C     *****   added Nov. 1989 file for annual surface excesses

C     *****

C     ******************************************************************

C     *****

C     *****   added Sep 03, 1993 files for deep percolation amounts

C     *****    including pond seepage.

C     *****

open(7,file='annlperc.out',status='UNKNOWN')

C     *****

C     ******************************************************************

GOTO 60

C     ******************************************************************

C     *****

C     *****   added November 30, 1989 file for monthly surface excesses,

C     *****    revised September 03, 1993 to add NET pond evaporation.

C     *****

56 open(6,file='mnthsurf.out',status='UNKNOWN')

C     *****

C     *****   added Sep 03, 1993 files for deep percolation amounts

C     *****    including pond seepage.

open(8,file='mnthperc.out',status='UNKNOWN')

C     *****

C     ******************************************************************

C     *****

C     *****   READ LAND USES AREA PARAMETERS

C     *****        NPLOT=NUMBER OF THE PLOT

C     *****        ISOIL=SCS IRRIGATION SOIL CLASS

C     *****        ICROP=CROP TYPE

C     *****        AREA=AREA IN ACRES

C     *****        RCNI=RCN FOR AMC I

C     *****        RCNII=RCN FOR AMC II

C     *****        RCNIII=RCN FOR AMC III

C     *****        MGSBP=MONTH GROWING SEASON BEGINS

C     *****        DGSBP=DAY OF MONTH GROWING SEASON BEGINS

C     *****        MGSEP=MONTH GROWING SEASON ENDS

C     *****        DGSEP=DAY OF MONTH GROWING SEASON ENDS

C     *****        ROTATE=WHEAT/FALLOW ROTATION INDICATOR

C     *****        POND=INDICATOR FOR LAND USE FLOWING INTO POND

C     *****         1=DOES NOT FLOW INTO POND,  2=DOES FLOW INTO POND

C     *****        TERR=INDICATOR FOR TERRACE TYPE

C     *****         1=NONE

C     *****         2=GRADED

C     *****         3=LEVEL

C     *****         4=LEVEL, CLOSED-END

C     ***** MUL REDEFINED TO ACCOUNT FOR EFFECT OF RESIDUE ON SOIL WATER

C     *****  EVAPORATION WHEN CROP IS NOT GROWING  SEPTEMBER 14, 1993  JKK

C     *****  MUL= DECIMAL REDUCTION OF EVAPORATION FROM LAND USE FOR RESI-

C     *****  DUE COMPARED TO BARE SOIL WHEN CROP IS NOT GROWING, TYPICAL

C     *****  RANGE = 0.05 FOR CONVENTIONAL TILLAGE TO

C     *****  0.30 WITH 6,000 LB/AC STANDING RESIDUE

C     *****        FLRCN1=FALLOW RCN FOR AMC I (IF NEEDED)

C     *****        FLRCN2=FALLOW RCN FOR AMC II (IF NEEDED)

C     *****        FLRCN3=FALLOW RCN FOR AMC III (IF NEEDED)

C     *****

C     ******************************************************************

C     *****

60 WRITE (5,62)

62 FORMAT (/,'LAND USES INFORMATION (acres= amount of each land use i

1n drainage area of pond)',/,'no soil crop acres cnI II  III st.mo

2dy ed.mo dy  r p t  mul cnI II  III')

DO 72 I=1,NPLOTS

READ (1,70) NPLOT(I),ISOIL(I),ICROP(I),AREA(I),RCNI(I),RCNII(I),

1RCNIII(I),MGSBP(I),DGSBP(I),MGSEP(I),DGSEP(I),ROTATE(I),POND(I),

2TERR(I),MUL(I),FLRCN1(I),FLRCN2(I),FLRCN3(I)

70 FORMAT (I2,I3,I2,F5.0,3F4.0,4I3,3I2,F5.2,3F4.0)

C     *****   ECHO PRINT INDIVIDUAL LAND USE INPUT PARAMETERS

WRITE (5,71) NPLOT(I),ISOIL(I),ICROP(I),AREA(I),RCNI(I),RCNII(I),

1RCNIII(I),MGSBP(I),DGSBP(I),MGSEP(I),DGSEP(I),ROTATE(I),POND(I),

2TERR(I),MUL(I),FLRCN1(I),FLRCN2(I),FLRCN3(I)

71 FORMAT (I2,2I4,F7.0,1X,3F4.0,2X,2I3,3X,2I3,2X,3I2,F5.2,3F4.0)

72 CONTINUE

C     *****

C     ******************************************************************

C     *****

C     ***** ECHO IRRIGATED LAND USES INFORMATION, ADDED SEPTEMBER 15,1993

C     *****  IPLOTS = TOTAL NO. OF LAND USES IRRIGATED

C     *****  IRGPLT = NO. OF LAND USE IRRIGATED

C     *****  IMOBEG = MO IRRIGATION BEGINS

C     *****  IDYBEG = DY IRRIGATION BEGINS

C     *****  IMOEND = MO IRRIGATION ENDS

C     *****  IDYEND = DY IRRIGATION ENDS

C     *****  PSCH = % AVAILABLE SOIL MOISTURE IN UPPER ZONE WHEN IRRIG.

C     *****  ARATE = DEPTH OF IRRIGATION ADDED EACH TIME, INCHES

C     *****

IF (IPLOTS.EQ.0) GOTO 83

C     *****

C     *****  OPEN OUTPUT FILE FOR IRRIGATION WATER USE, IF ONE OR MORE

C     *****  LAND USE IS IRRIGATED.

C     *****

GOTO (76,73,76,73,76,73) OUTPUT

73 open(9,file='mnthirrg.out',status='UNKNOWN')

GOTO 78

76 open(10,file='annlirrg.out',status='UNKNOWN')

78 WRITE (5,79)

79 FORMAT (/'IRRIGATED LAND USE INFORMATION',/,'no st.mo dy ed.mo dy

1 %asm inches')

DO 82 IR=1,IPLOTS

READ (1,80) IRGPLT, IMOBEG(IRGPLT), IDYBEG(IRGPLT),IMOEND(IRGPLT),

1IDYEND(IRGPLT), PSCH(IRGPLT), ARATE(IRGPLT)

80 FORMAT (I2,4I3,F4.0,F5.2)

WRITE (5,81) IRGPLT,IMOBEG(IRGPLT),IDYBEG(IRGPLT),IMOEND(IRGPLT),

1IDYEND(IRGPLT), PSCH(IRGPLT), ARATE(IRGPLT)

81 FORMAT(I2,I6,I3,I6,I3,F6.0,F6.2)

82 CONTINUE

C     *****************************************************************

C     *****

C     *****  READ IN HISTORICAL OR ASSUMED LAND USE FOR POSTPROCESSING

C     *****  TO ESTIMATE SURFACE AND PERCOLATION/GROUNDWATER YIELD.

C     *****  NEW SECTION ADDED SEPTEMBER 21, 1993  PLU=% LAND USE

C     *****

83 GOTO (90,90,84,84,97,97) OUTPUT

84 WRITE (5,85)

85 FORMAT (/,'POND AND LAND USE INFORMATION YEAR-BY-YEAR IN SUB-BASIN

1',/,'     no.  annual amount of each land use in percent of area i

2n sub-basin')

WRITE (5,86)

86 FORMAT ('year pnds  01   02   03   04   05   06   07   08   09   1

10   11   12   13   14   15   16   17   18')

LYEAR=999

open(12,file='TEMP',status='UNKNOWN')

DO  89 J=1,YEARS

READ (1,*) YEAR, NPD, (PLU(I),I=1,NPLOTS)

WRITE (12,87) YEAR, NPD,(PLU(I),I=1,NPLOTS)

87 FORMAT (I4,I3,18F5.1)

WRITE (5,88) YEAR, NPD,(PLU(I),I=1,NPLOTS)

88 FORMAT(I4,I4,18F5.1)

89 CONTINUE

GOTO 96

90 open(12,file='TEMP',status='UNKNOWN')

WRITE (5,91)

91 FORMAT (/,'POND AND LAND USE INFORMATION IN SUB-BASIN',/,

1'     no.         amount of each land use in percent of area in su

2b-basin')

WRITE (5,92)

92 FORMAT ('year pnds  01   02   03   04   05   06   07   08   09   1

10   11   12   13   14   15   16   17   18')

READ (1,*) YEAR, NPD, (PLU(I),I=1,NPLOTS)

WRITE (5,95) YEAR,NPD,(PLU(I),I=1,NPLOTS)

LYEAR=YEAR

DO 94 J=1,YEARS

YEAR = YSTART-1+J

WRITE (12,93) YEAR, NPD,(PLU(I),I=1,NPLOTS)

93 FORMAT (I4,I4,18F5.1)

94 CONTINUE

95 FORMAT(I4,I4,18F5.1)

96 CLOSE (12,STATUS='KEEP')

C     *****

C     ******************************************************************

C     *****

C     *****   SUBROUTINE VOLRT CALCULATES THE DIMENSIONS OF THE

C     *****    THE POND USING THE GIVEN INPUT INFORMATION.

97 CALL VOLRT (A1,A2,A3,A4,A5,L,PSAREA,S,VOLMAX,W,HMAX)

C     *****   CALCULATE THE CROP COEFFICIENTS

DO 98 K=1,NPLOTS

MGSB = MGSBP(K)

DGSB = DGSBP(K)

MGSE = MGSEP(K)

DGSE = DGSEP(K)

CROP = ICROP(K)

C     *****   SUBROUTINE CROPCO CALCULATES THE MONTHLY CROP

C     *****    COEFFICIENTS FOR EACH LAND USE.

CALL CROPCO (CROP,MGSB,DGSB,MGSE,DGSE,KCROP,NDIM,MMAT)

INCROP(K) = ICROP(K)

INRCN1(K) = RCNI(K)

INRCN2(K) = RCNII(K)

INRCN3(K) = RCNIII(K)

98 CONTINUE

C     *****   INITIALIZE VARIABLES

DO 110 I=1,25

DO 100 J=2,9

IF (J.LE.4) PRECAC(I,J) = 0.0

100 RUNACC(I,J) = 0.0

110 CONTINUE

DO 120 J=1,12

120 KCROP(7,J) = 0.0

C     *****

C     *****************************************************************

C     *****

C     *****  CROP COEFFICIENTS SET TO MAXIMUM OF 1.00 SEPT 1993 JKK

WRITE (5,122)

122 FORMAT(/,'Calculated Crop Coefficients for Crops Included on one o

1r More Land Uses',/,'Crop/Month 1    2    3    4    5    6    7

2 8    9   10   11   12')

DO 128 IJ=1,7

DO 126 KJ=1,12

IF (KCROP(IJ,KJ).GT.1.00) KCROP(IJ,KJ) = 1.00

126 CONTINUE

WRITE (5,127) IJ,(KCROP(IJ,KL),KL=1,12)

127 FORMAT(I3,4X,12F5.2)

128 CONTINUE

C     *****

DO 130 II=1,NPLOTS

IF (POND(II).NE.1) DA = DA+AREA(II)

TPAREA = TPAREA+AREA(II)

T(II) = 0.0

EO(II) = 0.0

IAET(II) = 0.0

DSRNFF(II) = 0.0

AINTER(II) = 0.0

AAETRS(II) = 0.0

ACHSOM(II) = 0.0

DSPERC(II) = 0.0

C     *****  THE FOLLOWING LINE WAS ADDED IN SEPTEMBER, 1982

SOIL = ISOIL(II)

C     *****   FOLLOWING 2 LINES REVISED SEPTEMBER, 1982

SMLZ(II) = 0.5*AVLFCL(SOIL) + PWPLZ(SOIL)

SMUZ(II) = 0.5*AVLFCU(SOIL) + PWPUZ(SOIL)

SMGWZ(II) = 6.30

SMPD(II) = SMLZ(II)+SMUZ(II)

C     *****   ESTABLISH FALLOW LAND USES FOR BEGINNING OF SIMULATION

130 IF (ROTATE(II).EQ.2) ICROP(II) = 7

C     *****

C     *****   PRINT INPUT PARAMETERS

C     *****

WRITE (5,150) LOCATION,YSTART,YEND,L,W,S,HMAX,

1VOLMAX,PSAREA,DSEPRT,DA

150 FORMAT (///,'STATION:  ',A48,2X,I4,' TO ',I4/,

1/5X,'TYPICAL POND VARIABLES:',/10X,'(A) BASE DIMENSIONS-- ',F5.0,'

2FEET BY',F5.0,' FEET',/10X,'(B) SIDE SLOPE--  RUN:RISE = ',F3.0,'

3:1',/,10X,'(C) MAXIMUM DEPTH-- ',F5.1,' FEET',/10X,'(D) MAXIMUM PO

4ND VOLUME-- ',F9.2,' ACRE-INCHES',/10X,'(E) MAXIMUM POND SURFACE A

5REA--',F8.2,' ACRES',/10X,'(F) SEEPAGE RATE--',F7.3,' INCHES/DAY',

6/10X,'(G) DRAINAGE AREA--',F7.2,' ACRES'//,'LAND USES AREA VARIABL

7ES:')

DO 180 J=1,NPLOTS

PLAREA = AREA(J)

CROP = INCROP(J)

TERT = TERR(J)

SOIL = ISOIL(J)

PLIT = NO

IF (POND(J).GT.1) PLIT = BLNK

WRITE (5,160) J,PLAREA,KROP(CROP),SOIL,PLIT,NTERR(TERT),MUL(J),RCN

1I(J),RCNII(J),RCNIII(J)

160 FORMAT (/5X,'LAND USE ',I2//10X,'(A) AREA IN POND WATERSHED--  ',

1F6.2,' ACRES'/10X,'(B) CROP--  ',A16/10X,'(C) SOIL TYPE--',I3,

2' (SCS IRRIGATION CLASS)'/10X,'(D) CONSERVATION PRACTICES-- ',A2,'

3 FLOW INTO POND',/,15X,'TERRACES-- ',A20,'RESIDUE FACTOR =',F4.2,

4/10X,'(E) RUNOFF CURVE NUMBERS: AMC I   = ',F3.0,/36X,'AMC II  = '

5,F3.0/36X,'AMC III = ',F3.0)

C     *****

C     *****************************************************************

C     *****

C     *****  PRINT IRRIGATION INFORMATION FOR LAND USE

IF (IMOBEG(J).EQ.0) GOTO 169

WRITE (5,165) PSCH(J),ARATE(J),IMOBEG(J),IDYBEG(J),IMOEND(J),

1IDYEND(J)

165 FORMAT (10X,'(F) IRRIGATED UPPER ZONE AVAILABLE SOIL MOISTURE BELO

1W: ',F3.0,'%'/16X,'WITH',F5.2,' INCHES BEGINNING 'I2,

2'-',I2,' AND ENDING ',I2,'-',I2)

169 IF (ROTATE(J).NE.1) WRITE (5,170) FLRCN1(J),FLRCN2(J),FLRCN3(J)

170 FORMAT (10X,'(G) CROP ROTATION WITH -- FALLOW'/14X,'FALLOW RUNOFF

1CURVE NUMBERS: AMC I   = ',F3.0/43X,'AMC II  = ',F3.0/43X,'AMC III

2 = ',F3.0)

180 CONTINUE

C     *****

C     *****************************************************************

C     ***** OPEN FILE FOR DAILY OUTPUT OF LAST YEAR OF SIMULATION.

C     ***** ADDED OCTOBER 13, 1993

open(16,file='DAILY.OUT',status='UNKNOWN')

WRITE (16,181) LOCATION,YEND

181 FORMAT ('Sub-basin simulated: 'a48,/,

1'Daily information for ',I4,', the last year of the simulation per

2iod.',//,25X,'-------- LAND USE 1 --------   -------- LAND USE 2 -

3-------   -------- LAND USE 3 --------   -------- LAND USE 4 -----

3---   -------- LAND USE 5 --------   -------- LAND USE 6 --------

5  -------- LAND USE 7 --------   -------- LAND USE 8 --------   --

6------ LAND USE 9 --------   -------- LAND USE 10 -------   ------

7-- LAND USE 11 -------   -------- LAND USE 12 -------   -------- L

8AND USE 13 -------   -------- LAND USE 14 -------   -------- LAND

9USE 15 -------   -------- LAND USE 16 -------   -------- LAND USE

917 -------   -------- LAND USE 18 -------')

WRITE (16, 182)

182 FORMAT ('MO DY TMX TMN PREC. PET IRRG.INTC. RUNF.PERC.AET SLMO. IR

1RG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.I

2NTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC.

3 RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUN

4F.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PE

5RC.AET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.A

6ET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.AET S

7LMO. IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO.

8 IRRG.INTC. RUNF.PERC.AET SLMO. IRRG.INTC. RUNF.PERC.AET SLMO.')

C     *****

C     *****************************************************************

WRITE (*,185) LOCATION

185 FORMAT('   Now reading weather data for ',A48,/)

C     *****************************************************************

C     *****

C     *****   ***** ENTER YEARLY LOOP *****

C     *****

DO 960 NY=1,YEARS

C     *****

C     *****   INITIALIZE VARIABLES

SMDI = 0

PERS = 0

SPER = 0

MAXVOL = 0.0

LKEVPT = 0.0

VOLCHG = 0.0

PETACT(13) = 0.0

C     *****  A LINE WAS REMOVED AT THIS POINT IN SEPTEMBER, 1982

DO 210 J=1,9

DO 200 I=1,13

DO 190 K=1,NPLOTS

190 SMACCT(I,J,K) = 0.0

200 PDACCT(I,J) = 0.0

210 CONTINUE

DO 220 K=1,NPLOTS

PCROP(K) = ICROP(K)

220 PEACCT(K) = 0.0

C     *****   ESTABLISH CROP ROTATIONS

DO 250 K=1,NPLOTS

IROT = ROTATE(K)

GOTO (250,230,240), IROT

C     *****   ROTATE=3 FOR WHEAT PLANTING YEAR ('FALLOW YEAR')

C     *****   THE FALLOW RCN'S ARE ASSUMED TO BE REPRESENTATIVE

C     *****    THROUGHOUT THE FALLOW YEAR.

230 ROTATE(K) = 3

RCNI(K) = FLRCN1(K)

RCNII(K) = FLRCN2(K)

RCNIII(K) = FLRCN3(K)

GOTO 250

C     *****   ROTATE=2 FOR WHEAT HARVESTING YEAR ('WHEAT YEAR')

C     *****   THE WHEAT RCN'S ARE ASSUMED TO BE REPRESENTATIVE

C     *****    THROUGHOUT THE WHEAT YEAR.

240 ROTATE(K) = 2

RCNI(K) = INRCN1(K)

RCNII(K) = INRCN2(K)

RCNIII(K) = INRCN3(K)

250 CONTINUE

WRITE (5,260)

260 FORMAT (//,26X,'****** ANNUAL SUMMARY ******')

C     *****

C     *****

C     *****   ***** ENTER MONTHLY LOOP *****

C     *****

C     *****  MONTHLY LOOP ALTERED IN SEPTEMBER, 1982 TO ELIMINATE

C     *****  MSTART FROM THE VARIABLE LIST (NAMELIST OMEGA).

C     *****   FOLLOWING LINE REVISED IN SEPTEMBER, 1982

DO 710 NM=1,12

PETACT(NM)=0.0

C     *****

C     *****   ESTABLISH CROP ROTATIONS FOR WHEAT

DO 290 II=1,NPLOTS

IROT = ROTATE(II)

GOTO (290,270,280), IROT

C     *****   IROT=2 FOR A WHEAT HARVESTING YEAR ('WHEAT YEAR')

270 IF (NM.GT.MGSEP(II)) ICROP(II) = 7

GOTO 290

C     *****   IROT=3 FOR A WHEAT PLANTING YEAR ('FALLOW YEAR')

280 IF (NM.GE.MGSBP(II)) ICROP(II) = INCROP(II)

290 CONTINUE

C     *****   READ DAILY METEOROLOGICAL DATA FOR ONE MONTH

300 READ (2,*,END=970) PSTIND,YEAR,MONTH,(PREC(I),I=1,31)

IF (PSTIND.NE.PINDX) GOTO 320

IF (YEAR.LT.YSTART) GOTO 300

305 READ (3,*,END=970) TSTIND,YEAR,MONTH,(TMAX(I),TMIN(I),I=1,31)

IF (TSTIND.NE.TINDX) GOTO 330

IF (YEAR.LT.YSTART) GOTO 305

C     *****

C     ******************************************************************

C     *****

C     *****  WRITE MESSAGE TO SCREEN TO SHOW PROGRAM PROGRESS DURING

C     *****  THE RUN.

C     *****

IF (COUNT.EQ.0) WRITE (*,310)

310 FORMAT('                       Now doing simulation for',/)

COUNT=1

if (nm.lt.2) write(*,311) year

311 format(i5,\,TL5)

C     *****

C     ******************************************************************

C     *****

IF (YEAR.GT.YEND) GOTO 970

GOTO 340

320 WRITE (*,325) PINDX, FILE2, PSTIND

325 FORMAT(/,' The precipitation station index specified in the input

1file is ',I6,',',/,' however, the precipitation station index in f

2ile, ',A12,' is ',I6,'.',/,' THEREFORE, YOUR RUN WAS TERMINATED.

3PLEASE CORRECT THE ERROR.')

GOTO 2900

330 WRITE (*,335) TINDX, FILE3, TSTIND

335 FORMAT(/,' The temperature station index specified in the input fi

1le is ',I6,',',/,' however, the temperature station index in file,

2 ',A12,' is ',I6,'.',/,' THEREFORE, YOUR RUN WAS TERMINATED.  PLEA

3SE CORRECT THE ERROR.')

GOTO 2900

340 NDIM(2) = 28

IF (NM.EQ.2.AND.TMAX(29).LT.900) NDIM(2) = 29

NDAYS = NDIM(NM)

C     *****

C     *****

C     *****   ***** ENTER DAILY LOOP *****

C     *****

DO 670 ND=1,NDAYS

C     *****

C     *****   THE FOLLOWING STATEMENTS CORRECT FOR MISSING DATA

C     *****    IN INPUT FILE.  MISSING DATA ENTERED AS 999 IN FILES.

IF (TMAX(ND).GT.150.0) TMAX(ND) = PDT

IF (TMIN(ND).GT.150.0) TMIN(ND) = PDT

IF (PREC(ND).GT.99.97) PREC(ND) = 0.0

C     *****   TAVG=AVERAGE DAILY AIR TEMPERATURE, DEGREES FAHRENHEIT

TAVG(ND) = (TMAX(ND)+TMIN(ND))/2.0

C     *****   SUBROUTINE PETRT CALCULATES THE POTENTIAL ET,

C     *****    THE BARE SOIL EVAPORATION, AND THE LAKE EVAPORATION.

ZWIND = WIND(NM)

ZRHD = RHD(NM)

ZRA = RA(NM)

ZPSUNS = PSUNS(NM)

AVGT = TAVG(ND)

ZMAXT = TMAX(ND)

ZMINT = TMIN(ND)

ZMN = NM

ZND = ND

TAVGM=MMAT(NM)

STRVOL = 0.0

CALL PETRT (ZPSUNS,ZRA,ZRHD,AVGT,ZWIND,BRUNTA,BRUNTB,DELTA,E,EPRI

1M,GAMMA,LAKEVP,PACK(1),PET,PETBS,RCROP,ZMAXT,ZMINT,MMMAXT,MMMINT,

2ZNM,ZNM)

C     *****

C     *****   ***** ENTER INDIVIDUAL LAND USE LOOP *****

C     *****

DO 580 JJ=1,NPLOTS

C     *****

STRNOF = 0.0

CROP = ICROP(JJ)

SOIL = ISOIL(JJ)

KROPKO = KCROP(CROP,NM)

ZSMUZ = SMUZ(JJ)

C     *****   FOLLOWING 2 LINES REVISED SEPTEMBER, 1982

ZPWPUZ = PWPUZ(SOIL)

ZAVFCU = AVLFCU(SOIL)

RCN1 = RCNI(JJ)

RCN2 = RCNII(JJ)

RCN3 = RCNIII(JJ)

ZIASTN = IASTOR(JJ)

ZSM = SM(JJ)

ZPACK = PACK(JJ)

C     *****   SUBROUTINE SNOWRT CALCULATES THE MOISTURE ADDED TO THE

C     *****    LAND USE DUE TO SNOWMELT ON THE AREA

PRECIP = PREC(ND)

SNOVAP = 0.0

WATER = PRECIP

CALL SNOWRT (PRECIP,KROPKO,WATER,ZPACK,PET,PETBS,TAVG(ND),SNOVAP)

C     *****

PACK(JJ)=ZPACK

RAIN = WATER

IF (RAIN) 360,360,350

C     *****   SUBROUTINE RNOFRT EVALUATES PRECIPITATION EXCESS USING

C     *****   THE SCS EQUATION.

350 CALL RNOFRT (ZAVFCU,ZPWPUZ,RAIN,RCN1,RCN2,RCN3,RNOF,ZSMUZ)

GOTO 362

360 RNOF = 0.0

C     *****

C     *****************************************************************

C     *****

C     ***** IF SNOW EVAPORATION OCCURRS, INTERCEPTION EQUAL SNOVAP FOR

C     ***** THE DAY AND THE INTERCEPTION STORAGE ACCOUNT DOES NOT CHANGE.

C     ***** THE SNOVAP IS SUBTRACTED FROM PACK.

C     ***** ADDED OCTOBER 14, 1993 JKK

C     *****

C     *****   SUBROUTINE IART EVALUATES THE INTERCEPTION AMOUNT AND STORAGE.

362 CALL IART (IA,SNOVAP,ZIASTN,KROPKO,PRECIP,PET,PETBS,ZIASTT,ZIAET,

1TAVG(ND))

IASTOR(JJ) = ZIASTT

IAET(JJ) = ZIAET

C     *****

C     ******************************************************************

C     *****

C     *****  IRRIGATION ADDED HERE.  IRRIGATION ONLY ON DAYS WITH NO

C     *****  RAIN.  CHECK IF IMOBEG(J).GT.0 TO SEE IF IRRIGATED LAND USE

C     *****  AND IF IS DATE WITHIN IRRIGATION PERIOD OF THE YEAR AND IF

C     *****  AVAILU/SMMAXU*100.LT.PSCH.  IF SO, ADD AMOUNT EQUAL TO ARATE

C     *****  TO UPPER ZONE SOIL MOISTURE ACCOUNT FOR (J).  KEEP ACCOUNT OF

C     *****  AMOUNT ADDED FOR MONTH, YEAR, AND TOTAL RUN.

C     *****

C     *****  IMOBEG = MONTH IRRIGATION BEGINS

C     *****  IDYBEG = DAY OF IMOBEG IRRIGATION BEGINS

C     *****  IMOEND = MONTH IRRIGATION ENDS

C     *****  IDYEND = DAY OF IMOEND IRRIGATION ENDS

C     *****  SWTCH = AVAILABLE SOIL MOISTURE IN PROFILE, %

C     *****  PSCH = AVAILABLE SOIL MOISTURE IN PROFILE AT WHICH IRRIGATION

C     *****  BEGINS, %

C     *****  AMTIRR = AMOUNT OF WATER ADDED BY ONE IRRIGATION, INCHES

C     *****

365 IF (RAIN.GT.0.0) GOTO 384

IF (AVGT.LT.40.) GOTO 384

IF (FROZE.EQ.1) GOTO 384

IF (PACK(JJ).GT.0.0) GOTO 384

IF (IMOBEG(JJ).EQ.0) GOTO 384

IF (ICROP(JJ).EQ.1) GOTO 374

IF (NM.LT.IMOBEG(JJ)) GOTO 384

IF (NM.GT.IMOEND(JJ)) GOTO 384

IF (NM.EQ.IMOBEG(JJ)) GOTO 371

IF (NM.EQ.IMOEND(JJ)) GOTO 372

GOTO 378

371 IF (ND.LT.IDYBEG(JJ)) GOTO 384

GOTO 378

372 IF (ND.GT.IDYEND(JJ)) GOTO 384

GOTO 378

374 IF (NM.LT.IMOEND(JJ)) GOTO 378

IF (NM.GT.IMOBEG(JJ)) GOTO 378

IF (NM.EQ.IMOEND(JJ)) GOTO 375

IF (NM.EQ.IMOBEG(JJ)) GOTO 376

GOTO 384

375 IF (ND.LE.IDYEND(JJ)) GOTO 378

376 IF (ND.GE.IDYBEG(JJ)) GOTO 378

GOTO 384

378 SWTCH = 100*((SMUZ(JJ)-PWPUZ(SOIL))/(FCU(SOIL)-PWPUZ(SOIL)))

IF (SWTCH.GT.PSCH(JJ)) GOTO 381

AMTIRR = ARATE(JJ)

GOTO 385

381 SWTCH = 100*((SMUZ(JJ)+SMLZ(JJ)-PWPUZ(SOIL)-PWPLZ(SOIL))/(FCU(SOIL

1)+FCL(SOIL)-PWPUZ(SOIL)-PWPLZ(SOIL)))

IF (SWTCH.GT.PSCH(JJ)) GOTO 384

AMTIRR = ARATE(JJ)

GOTO 385

384 AMTIRR = 0.

C     *****

C     ******************************************************************

C     *****

C     *****   EVALUATE INFILTRATION INTO THE UPPER ZONE

385 PERC = RAIN-RNOF-IA+AMTIRR

UZEVAP = 0.0

C     *****   CALCULATE PRESENT STORAGE AVAILABLE IN UPPER ZONE

SMMAXU = 0.9*SMSATU(SOIL)-SMUZ(JJ)

C     *****   EVALUATE WATER CASCADED TO LOWER ZONE FOR STORAGE

PERCL = PERC-SMMAXU

IF (PERC.GT.SMMAXU) PERC = SMMAXU

IF (PERCL.LT.0.0) PERCL = 0.0

IF (SMUZ(JJ).GT.FCU(SOIL)) GOTO 390

EXCESS = 0.0

GOTO 395

C     *****   EVALUATE GRAVITATIONAL WATER IN UPPER ZONE

390 EXCESS = SMUZ(JJ)-FCU(SOIL)

C     *****

C     *****   IF THE CROP IS DORMANT OR THE SOIL LIES FALLOW, SOIL

C     *****    EVAPORATION IS EVALUATED

395 IF (KCROP(CROP,NM).LE.0.0) GOTO 510

T(JJ) = 0.0

C     *****   MODIFY PET BY THE PLANT CONSUMPTIVE USE COEFFICIENT AND

C     *****   INTERCEPTION AMOUNT THAT SATISFIES PET TO OBTAIM AET BEFORE

C     *****   CHECKING TO SEE IF IT IS LIMITED FURTHER BY SOIL MOISTURE

C     *****   AVAILABILITY

IF (SNOVAP) 397,397,396

396 AET = 0.0

GOTO 510

397 IF (PET-IAET(JJ)) 399,399,400

399 AET = 0.0

GOTO 405

400 AET = KCROP(CROP,NM)*(PET-IAET(JJ))

C     *****   CHECK WHETHER SOIL MOISTURE LIMITS AET FROM THE UPPER ZONE

C     *****

C     *****   CHANGED FROM CONSTANT FROM O.3 TO 0.5 SEPTEMBER 14, 1993

C     *****   THIS CHANGE MEANS THAT ONCE AVAILABLE SOIL MOISTURE (ASM)

C     *****   IS BELOW 50%, THERE IS A LINEAR DECREASE IN THE RATIO OF

C     *****   AET/PET FROM 1 TO 0 AS ASM DECREASES FROM 50% TO 0%

C     *****   PREVIOUSLY THE DECREASE BEGAN AT 30% ASM.  THIS CHANGE

C     *****   REDUCES THE RATE OF AET AND KEEPS THE SOIL WETTER WHICH

C     *****   MORE REALISTICALLY REPRESENTS FIELD CONDITIONS.

C     *****

405 IF (SMUZ(JJ)-(0.5*(AVLFCU(SOIL))+PWPUZ(SOIL))) 410,410,440

C     *****   CALCULATE AET FROM THE UPPER ZONE WHEN LIMITED BY SOIL

C     *****    MOISTURE.

410 AVAILU = SMUZ(JJ)-PWPUZ(SOIL)

IF (AVAILU.LE.0.0) AVAILU = 0.0

AETUZ = 0.7*AET*(AVAILU/(0.5*AVLFCU(SOIL)))

C     *****   EVALUATE AVAILABLE WATER IN THE LOWER ZONE

AVAILL = SMLZ(JJ)-PWPLZ(SOIL)

IF (AVAILL.LE.0.0) AVAILL = 0.0

C     *****   CHECK WHETHER SOIL MOISTURE LIMITS AET FROM THE LOWER ZONE

IF (SMLZ(JJ)-(0.5*(AVLFCL(SOIL))+PWPLZ(SOIL))) 420,420,430

C     *****   CALCULATE AET FROM THE LOWER ZONE WHEN LIMITED BY SOIL

C     *****    MOISTURE.

420 AETLZ = 0.3*AET*(AVAILL/(0.5*AVLFCL(SOIL)))

GOTO 450

430 AETLZ = AET-AETUZ

GOTO 450

C     *****   EVALUATE AET FROM BOTH ZONES UNDER WET CONDITIONS

440 AETUZ = 0.7*AET

AETLZ = 0.3*AET

AVAILL = SMLZ(JJ)-PWPLZ(SOIL)

IF (SMLZ(JJ).LE.0.5*(AVLFCL(SOIL))+PWPLZ(SOIL)) GOTO 420

450 IF (PERC-SMMAXU) 460,470,470

C     *****

C     *****   EVALUATE SOIL MOISTURE

460 SMUZ(JJ) = SMUZ(JJ)+PERC-AETUZ-EXCESS

SMLZ(JJ) = SMLZ(JJ)-AETLZ+EXCESS

GOTO 560

470 SMUZ(JJ) = SMUZ(JJ)+SMMAXU-EXCESS-AETUZ

480 SMMAXL = 0.9*FCL(SOIL)-SMLZ(JJ)

IF (PERCL+EXCESS-SMMAXL) 490,490,500

490 SMLZ(JJ) = SMLZ(JJ)+PERCL-AETLZ+EXCESS

GOTO 560

500 SMLZ(JJ) = SMLZ(JJ)+SMMAXL-AETLZ

DPERC = PERCL+EXCESS-SMMAXL

GOTO 570

C     *****   CALCULATE EVAPORATION FROM SOIL SURFACE FOR MONTHS

C     *****    WHEN KCROP=0.0

C     *****

C     ******************************************************************

C     *****

C     *****   EFFECT OF RESIDUE ON EVAPORATION ADDED SEPTEMBER 15, 1993

C     *****   MUL = DECIMAL REDUCTION IN STAGE 1 & STAGE 2 EVAPORATION

C     *****   DURING NON-GROWING OR FALLOW SEASON WHEN KCROP=0.0

C     *****

C     ******************************************************************

C     *****

510 AETUZ = 0.0

AETLZ = 0.0

IF (SNOVAP.GT.0.0) GOTO 540

IF (SMUZ(JJ).LT.(FCU(SOIL)-U(SOIL))) GOTO 520

EO(JJ) = FCU(SOIL)-SMUZ(JJ)

IF (SMUZ(JJ).GE.FCU(SOIL)) EO(JJ) = 0.0

C     *****   CALCULATE STAGE 1 SOIL EVAPORATION

UZEVAP = (1-MUL(JJ))*(PETBS-IAET(JJ))

EO(JJ) = EO(JJ)+UZEVAP

IF (EO(JJ).GT.U(SOIL)) UZEVAP = EO(JJ)-U(SOIL)

T(JJ) = 0.0

GOTO 530

C     *****   CALCULATE STAGE 2 SOIL EVAPORATION

520 T(JJ) = T(JJ)+1

UZEVAP =(1-MUL(JJ))*(C(SOIL)*(T(JJ)**0.5)-C(SOIL)*((T(JJ)-1)**0.5)

1)

530 IF (UZEVAP.GT.((1-MUL(JJ))*(PETBS-IAET(JJ)))) UZEVAP = (1-MUL(JJ))

1*(PETBS-IAET(JJ))

IF (UZEVAP.LT.0.0) UZEVAP = 0.0

IF (SMUZ(JJ)-PWPUZ(SOIL).LT.UZEVAP) UZEVAP = SMUZ(JJ)-PWPUZ(SOIL)

GOTO 550

540 UZEVAP = 0.0

550 SMUZ(JJ) = SMUZ(JJ)-UZEVAP+PERC-EXCESS

IF (SMUZ(JJ).LE.PWPUZ(SOIL)) SMUZ(JJ) = PWPUZ(SOIL)

GOTO 480

560 IF (SMLZ(JJ).LT.PWPLZ(SOIL)) AETLZ = AETLZ-(PWPLZ(SOIL)-SMLZ(JJ))

IF (SMLZ(JJ).LE.PWPLZ(SOIL)) SMLZ(JJ) = PWPLZ(SOIL)

DPERC = SMLZ(JJ)-0.9*FCL(SOIL)

IF (DPERC.LT.0.0) DPERC = 0.0

IF (SMLZ(JJ).GT.0.9*FCL(SOIL)) SMLZ(JJ) = 0.9*FCL(SOIL)

570 AETUZ = AETUZ+UZEVAP

C     *****   SM=SOIL MOISTURE IN THE GROWING ZONE, IN INCHES

C     *****   IAET=AMOUNT OF ET DEDUCTED FROM INTERCEPTION

C     *****    STORAGE (IN.)

C     *****   AETU=ET AMOUNT FROM THE UPPER ZONE (IN.)

C     *****   AETL=ET AMOUNT FROM THE LOWER ZONE (IN.)

C     *****   NDPERC=AMOUNT OF DEEP PERCOLATION (IN.)

C     *****   NIA=INITIAL ABSTRACTION AMOUNT (IN.)

C     *****   NRNOF=PRECIPITATION EXCESS AMOUNT (IN.)

C     *****   STRNOF=PRECIPITATION EXCESS AMOUNT WHICH FLOWS

C     *****    INTO POND (IN.)

C     *****   STRVOL=PRECIPITATION EXCESS VOLUME WHICH FLOWS

C     *****    INTO THE POND (ACRE-INCHES)

SM(JJ) = SMUZ(JJ)+SMLZ(JJ)

C     *****SOIL MOISTURE DROUGHT INDEX ROUTINE

C     *****THE FOLLOWING CALCULATIONS DETERMINE A SOIL MOISTURE

C     *****DROUGHT INDEX FOR A GIVEN CROP DURING A PREDETERMINED

C     *****GROWING PERIOD - MBEGIN TO MEND.  IT SUMS THE NUMBER

C     *****OF DAYS THAT SOIL MOISTURE IN THE UPPER ZONE IS LESS THAN

C     *****30 PERCENT OF FIELD CAPACITY AND THE MAXIMUM NUMBER OF

C     *****DAYS THE CONDITION PERSISTS DURING EACH YEAR.

C      IF (JJ.EQ.SKPLOT) GOTO 575

C      IF (CROP.NE.JCROP) GOTO 575

C      IF (NM.LT.MBEGIN.OR.NM.GT.MEND) GOTO 575

C      IF (SMUZ(JJ).GE.(0.3*(AVLFCU(SOIL))+PWPUZ(SOIL))) GOTO 574

C      SMDI=SMDI+1

C      PERS=PERS+1

C      IF (PERS.GT.SPER) SPER=PERS

C      GOTO 575

C  574 PERS=0

575 AETU(JJ) = AETUZ

AETL(JJ) = AETLZ

NDPERC(JJ) = DPERC

NIA(JJ) = IAET(JJ)

NRNOF(JJ) = RNOF

TIRRG(JJ) = AMTIRR

IF (POND(JJ).GT.1) STRNOF = RNOF

STRVOL = STRVOL+STRNOF*AREA(JJ)

580 CONTINUE

C     *****

C     *****

C     *****   **** CALCULATION OF SURFACE AREA AND DETERMINATION OF

C     *****          SURFACE EVAPORATION FROM STORAGE FACILITY ***

C     *****

IF (PONVOL.LE.0.0) GOTO 600

C     *****   THE FOLLOWING CALCULATION EXPRESSES THE VOLUME OF WATER

C     *****    IN THE POND IN CUBIC FEET.

V = PONVOL*3630

C     *****        THE FOLLOWING CALCULATIONS DETERMINE THE SURFACE

C     *****   AREA OF THE POND AS A FUNCTION OF THE STORAGE VOLUME.

C     *****   AREA IS IN SQUARE FEET AND VOLUME IS IN CUBIC FEET.

C     *****   THE POND IS SHAPED LIKE AN INVERTED FRUSTUM OF A

C     *****   PYRAMID. INPUT PARAMETERS TO SIZE THE FACILITY ARE

C     *****   LENGTH (L) AND WIDTH (W) OF THE BASE IN FEET, AND SIDE

C     *****   SLOPES EXPRESSED AS A RATIO OF RUN:RISE.

C     *****        INPUTS TO THE POND INCLUDE PRECIPITATION EXCESS

C     *****   AND DIRECT PRECIPITATION ONTO THE SURFACE. LOSSES

C     *****   INCLUDE EVAPORATION, EXFILTRATION, AND OVERFLOWS.

C     *****

C     *****   B2 IS THE AREA OF THE SURFACE LIQUID IN SQUARE FEET.

HAPRX = (PONVOL/VOLMAX)*HMAX

590 VC = A1*HAPRX+A2*HAPRX**2+A3*HAPRX**3

DV = V-VC

DVDH = A1+A4*HAPRX+A5*HAPRX**2

H = HAPRX+DV/DVDH

IF (ABS(H-HAPRX).LT.0.1) GOTO 600

HAPRX = H

GOTO 590

600 IF (H.GT.HMAX) H = HMAX

B2 = (W+2.*S*H)*(L+2.*S*H)

IF (FROZE.EQ.1) LAKEVP = 0.0

LKEVPT = LKEVPT+LAKEVP

TOTLKE(NM) = TOTLKE(NM)+LAKEVP

SEVAP = B2*(LAKEVP/12)

C     *****   SEVAP IS THE VOLUME OF WATER EXTRACTED FROM THE STORAGE

C     *****    FACILITY BY FREE SURFACE EVAPORATION.

IF ((SEVAP/3630).GT.PONVOL) SEVAP = PONVOL*3630

PONVOL = PONVOL-(SEVAP/3630)

IF (PONVOL.LE.0.0) PONVOL = 0.0

C     *****   SUBROUTINE SEEPGE CALCULATES THE DAILY AMOUNT OF

C     *****    EXFILTRATION FORM THE POND

CALL SEEPGE (B2,PONVOL,DASEEP,DSEPRT)

C     *****

C     *****   THE VOLUMES OF CALCULATED RUNOFF AND DIRECT PRECIPITATION

C     *****    ARE ADDED TO THE POND VOLUME

PRCPVL = PRECIP*B2/43560.0

C     *****   THE VOLUME OF WATER REMAINING AT THE END OF THE DAY IS

C     *****    EXPRESSED IN ACRE-INCHES.

PONVOL = PONVOL+PRCPVL+STRVOL

C     *****

C     *****   THE FOLLOWING STATEMENTS DETERMINE WHETHER THE POND HAS

C     *****   OVERFLOWED AND IF SO, THE QUANTITY DISCHARGED

DSCHRG = 0.0

IF (PONVOL-VOLMAX) 630,630,610

610 DSCHRG = PONVOL-VOLMAX

IF (DSCHRG.GE.PEAK) PEAK = DSCHRG

C     *****   NODSCH=TOTAL NUMBER OF DISCHARGE DAYS

NODSCH = NODSCH+1

DSCVOL = DSCVOL+DSCHRG

C     *****   VOLUME CALCULATIONS TO INCREASE THE POND SIZE FOR 100%

C     *****    CONTROL

PCNTRL = CONTRL*100.0

VOLCHG = CONTRL*PONVOL-VOLMAX+VOLCHG

VOLMX1 = VOLMAX+VOLCHG

VCB = 2*S*HMAX

VCC = ((4./3.)*S**2.0)-(VOLMX1*3630./HMAX)

VCD = VCB**2-(4.*VCC)

VC1 = SQRT(VCD)

DIM = (VC1-VCB)/2.0

C     *****   WRITE DAILY OVERFLOW MESSAGE

C     *****************************************************************

C     *****

C     *****  RESIZING OF POND NOT NEEDED.  THESE LINES

C     *****  WERE COMMENTED OUT SEPTEMBER 08, 1993.   JKK

C     *****

C     *****************************************************************

C     *****

C      WRITE (5,620) NM,ND,YEAR,DSCHRG,VOLMX1,PCNTRL,DIM

C  620 FORMAT (/,1X,I2,'/',I2,'/',I2,' - DISCHARGE OF ',F7.2,' ACRE-IN ',

C     1'REQUIRES VOLUME OF',F8.2,' ACRE-IN FOR ',F6.2,' % CONTROL WHERE L

C     2 = W = ',F8.2)

PONVOL = VOLMAX

C     *****

C     *****************************************************************

C     *****

630 CONTINUE

C     *****

C     *****              UPDATE SOIL MOISTURE ACCOUNTS

C     *****

DO 632 I3=1,NPLOTS

SMACCT(NM,2,I3) = SMACCT(NM,2,I3)+PRECIP

SMACCT(NM,3,I3) = SMACCT(NM,3,I3)+NIA(I3)

SMACCT(NM,4,I3) = SMACCT(NM,4,I3)+NRNOF(I3)

SMACCT(NM,5,I3) = SMACCT(NM,5,I3)+NDPERC(I3)

SMACCT(NM,6,I3) = SMACCT(NM,6,I3)+AETU(I3)+AETL(I3)

C      WRITE (*,576) I3,AETU(I3),AETL(I3)

SMACCT(NM,7,I3) = SMACCT(NM,7,I3)+SM(I3)-SMPD(I3)

C     *****

C     ***** IRRIGATION AMOUNT STORAGE ACCOUNT

C     *****

SMACCT(NM,9,I3) = SMACCT(NM,9,I3)+TIRRG(I3)

C     *****

632 SMPD(I3) = SM(I3)

C     *****              UPDATE POND ACCOUNT

C     *****

PDACCT(NM,2) = PDACCT(NM,2)+PRCPVL

PDACCT(NM,3) = PDACCT(NM,3)+STRVOL

PDACCT(NM,4) = PDACCT(NM,4)+SEVAP/3630.0

PDACCT(NM,5) = PDACCT(NM,5)+DASEEP

PDACCT(NM,6) = PDACCT(NM,6)+DSCHRG

PDACCT(NM,7) = PDACCT(NM,7)+(PONVOL-PDVOL)

IF (ND.EQ.NDAYS) PDACCT(NM,8) = H

PDVOL = PONVOL

IF (PONVOL.GT.MAXVOL) MAXVOL = PONVOL

C     ******************************************************************

C     ***** PET AMOUNT STORAGE ACCOUNT

PETACT(NM) = PETACT(NM)+PET

C     *****

C     *****

C     *****

C     *****************************************************************

C     *****  PRINT DAILY INFORMATION FOR LAST YEAR OF SIMULATION TO A FILE.

C     *****  DVAL IS AN ARRAY TO STORE DAILY VALUES FOR PRINTING.

C     *****

IF (NY.NE.YEARS) GO TO 649

DO 635 LL=1,NPLOTS

DVAL(LL,1)=TIRRG(LL)

DVAL(LL,2)=NIA(LL)

DVAL(LL,3)=NRNOF(LL)

DVAL(LL,4)=NDPERC(LL)

DVAL(LL,5)=AETU(LL)+AETL(LL)

DVAL(LL,6)=SM(LL)

635 CONTINUE

NTMX=TMAX(ND)

NTMN=TMIN(ND)

WRITE (16,640) NM,ND,NTMX,NTMN,PRECIP,PET,

1((DVAL(L6,L5),L5=1,6),L6=1,NPLOTS)

640 FORMAT (I2,I3,2I4,2F5.2,18(5F5.2,F6.2))

C     *****   STATISTICAL PRECIPITATION AND RUNOFF FREQUENCY

C     *****    CALCULATIONS

649 IF (PREC(ND).GT.0.0) CTPDAY = CTPDAY+1.0

IPLOT = NPLOTS

IF (NPLOTS.GT.4) NPLOTS = 4

DO 660 II=1,25

IF (PREC(ND).GT.FREQ(II)) CTP(II) = CTP(II)+1.0

CNTR = 0.0

DO 650 KI=1,NPLOTS

IF (NRNOF(KI).GT.0.0.AND.II.EQ.1) CTRDAY(KI) = CTRDAY(KI)+1.0

IF (NRNOF(KI).GT.0.0) CNTR = 1.0

650 IF (NRNOF(KI).GT.FREQ(II)) CTR(II,KI) = CTR(II,KI)+1.0

660 IF (PREC(ND).GT.FREQ(II).AND.CNTR.EQ.1.0) CTPR(II) = CTPR(II)+1.0

NPLOTS = IPLOT

C      IF (NM.EQ.MPOND.AND.ND.EQ.1) VOLPER=100.*PONVOL/VOLMAX

670 CONTINUE

C     *****

C     *****   ***** EXIT DAILY LOOP *****

C     *****

C     *****   UPDATE ACCOUNTS

PETACT(13)=PETACT(13)+PETACT(NM)

PDACCT(NM,1) = AMONTH(NM)

DO 680 J=2,7

PDACCT(13,J) = PDACCT(13,J)+PDACCT(NM,J)

680 CONTINUE

DO 700 MP=1,NPLOTS

SMACCT(NM,8,MP) = SM(MP)

DO 690 J=2,9

690 SMACCT(13,J,MP) = SMACCT(13,J,MP)+SMACCT(NM,J,MP)

SMACCT(NM,1,MP) = AMONTH(NM)

700 CONTINUE

710 CONTINUE

C     *****

C     *****   ***** EXIT MONTHLY LOOP *****

C     *****

PDACCT(13,1) = AMONTH(13)

DO 720 MP=1,NPLOTS

SMACCT(13,1,MP) = AMONTH(13)

SMACCT(13,8,MP) = SMACCT(12,8,MP)

720 CONTINUE

C     *****   DSNOW=CHANGE IN SNOWPACK STORAGE (IN.)

C     *****   PACKPY=SNOWPACK STORAGE FOR NEXT YEAR'S CALCULATIONS (IN)

C     *****   DSRNFF=ANNUAL PRECIPITATION EXCESS FROM EACH LAND USE (IN.)

C     *****   AINTER=ANNUAL INTERCEPTION AMOUNT FOR EACH LAND USE (IN.)

C     *****   AAETRS=ANNUAL ACTUAL ET FROM EACH LAND USE (IN.)

C     *****   ACHSOM=ANNUAL CHANGE IN SOIL MOISTURE FOR EACH

C     *****    LAND USE (IN.)

C     *****   DSPERC=ANNUAL DEEP PERCOLATION AMOUNT FOR EACH

C     *****    LAND USE (IN.)

C     *****   PEACCT=ANNUAL PRECIPITATION EXCESS FROM EACH LAND USE (IN.)

C     *****   TPREC=TOTAL PRECIPITATION FOR SIMULATION RUN (IN.)

C     *****   DRY=LOWEST ANNUAL PRECIPITATION AMOUNT (IN.)

C     *****   WET=GREATEST ANNUAL PRECIPITATION AMOUNT (IN.)

C     *****   NYDSCH=TOTAL NUMBER OF YEARS HAVING A DISCHARGE

C     *****   PRCPT=TOTAL VOLUME OF DIRECT PRECIPITATION ON THE

C     *****    POND SURFACE DURING THE SIMULATION RUN (ACRE-INCHES)

C     *****   PEINT=TOTAL VOLUME OF PRECIPITATION EXCESS FLOWING INTO

C     *****    POND DURING THE SIMULATION RUN (ACRE-INCHES)

C     *****   EVAPT=TOTAL VOLUME OF EVAPORATION FROM THE POND SURFACE

C     *****    DURING THE SIMULATION RUN (ACRE-INCHES)

C     *****   EXFILT=TOTAL VOLUME OF EXFILTRATION FROM THE POND

C     *****    DURING THE SIMULATION RUN (ACRE-INCHES)

C     *****   MAXVOL=MAXIMUM % OF POND VOLUME UTILIZED DURING THE

C     *****    SIMULATION RUN

C     *****   EVAPLK=TOTAL CALCULATED LAKE EVAPORATION FOR SIMULATION

C     *****    RUN (INCHES)

C     *****   DAYLD=VOLUME OF PRECIPITATION EXCESS FROM THE POND

C     *****    WATERSHED (INCHES)

PETTAC=PETTAC+PETACT(13)

PDACCT(13,8) = PDACCT(12,8)

DSNOW = PACK(1)-PACKPY

PACKPY = PACK(1)

DO 730 KT=1,NPLOTS

DSRNFF(KT) = DSRNFF(KT)+SMACCT(13,4,KT)

AINTER(KT) = AINTER(KT)+SMACCT(13,3,KT)

AAETRS(KT) = AAETRS(KT)+SMACCT(13,6,KT)

ACHSOM(KT) = ACHSOM(KT)+SMACCT(13,7,KT)

DSPERC(KT) = DSPERC(KT)+SMACCT(13,5,KT)

PEACCT(KT) = SMACCT(13,4,KT)

AIRRIG(KT) = AIRRIG(KT)+SMACCT(13,9,KT)

730 CONTINUE

TPREC = TPREC+SMACCT(13,2,1)

IF (YEAR.EQ.YSTART) DRY = SMACCT(13,2,1)

IF (SMACCT(13,2,1).GE.WET) WET = SMACCT(13,2,1)

IF (SMACCT(13,2,1).LE.DRY) DRY = SMACCT(13,2,1)

IF (PDACCT(13,6).GT.0.0) NYDSCH = NYDSCH+1

PRCPT = PRCPT+PDACCT(13,2)

PEINT = PEINT+PDACCT(13,3)

EVAPT = EVAPT+PDACCT(13,4)

EXFILT = EXFILT+PDACCT(13,5)

MAXVOL = MAXVOL*100.0/VOLMAX

EVAPLK = EVAPLK+LKEVPT

DAYLD = PDACCT(13,6)/DA

DAYPE = (PDACCT(13,4)-PDACCT(13,2))/DA

PDSEEP =PDACCT(13,5)/DA

C     *****

C     *****

C     *****                         PRINT POND ACCOUNT

C     *****

GOTO (740,770,740,770,740,770) OUTPUT

C     *****

C     *****             PRINT POND ACCOUNT ANNUAL VALUES ONLY

C     *****

740 WRITE (5,750) YEAR

750 FORMAT (/,15X,'WATER ACCOUNT FOR THE POND IN ACRE-INCHES FOR ',

1I4,/,79('-'),/,7X,'INFLOWS',25X,'OUTFLOWS',/,22('-'),2X,38('-'),1X

2,'DELTA   ENDING',/,'PRECIP. PRECIP. EXCESS  SURFACE EVAP.   SEEPA

3GE     DISCHARGE  VOLUME  DEPTH,FT')

WRITE (5,760) (PDACCT(13,K),K=2,8)

760 FORMAT (F6.1,F11.1,F15.1,F13.1,F14.1,F9.1,F9.2)

GOTO 800

C     *****

C     *****             PRINT POND ACCOUNT ON MONTHLY BASIS

C     *****

770 WRITE (5,780) YEAR

780 FORMAT (/,15X,'WATER ACCOUNT FOR THE POND IN ACRE-INCHES FOR ',

1I4,/,79('-'),/,13X,'INFLOWS',21X,'OUTFLOWS',/,6X,22('-'),2X,31('-'

2),'  DELTA   ENDING'/,'MONTH PRECIP. PRECIP. EXCESS  SURFACE EVAP.

3 SEEPAGE DISCHARGE  VOLUME  DEPTH,FT')

WRITE (5,790) ((PDACCT(I,K),K=1,8),I=1,13)

790 FORMAT (A4,F6.1,F13.1,F14.1,F12.1,F9.1,F10.1,F9.2)

800 WRITE (5,810) PACK(1),YEAR,DSNOW

810 FORMAT (/,'LAND USE NO. 1 SNOW PACK MOISTURE STORAGE INFORMATION (

1INCHES):',/,'  MOISTURE STORED IN SNOW PACK ON DEC. 31---',F5.2,/,

2'  CHANGE IN STORAGE DURING ',I4,' =',F5.2//,'  CHANGE IN SOIL MOI

3STURE = (INPUTS) - (OUTPUTS) +/- (CHANGE IN SNOW STORAGE)')

DO 920 N3=1,NPLOTS

GOTO (840,870,840,870,840,870) OUTPUT

C     *****

C     *****      PRINT SOIL MOISTURE ACCOUNT ANNUAL TOTALS ONLY

C     *****

840 WRITE (5,850) N3,YEAR

850 FORMAT (/,'LAND USE NO.',I3,'  WATER BALANCE (INCHES) FOR LAND USE

1 FOR ',I4,/,79('-'),/,3X,'INPUTS',23X,'OUTPUTS',20X,'SOIL MOISTURE

2',/,13('-'),4X,39('-'),2X,15('-'),/,'PRECIP. IRRIG.    INTRCPTN PR

3ECIP.EXCESS PERC.   AET       CHANGE  TOTAL')

WRITE (5,860) SMACCT(13,2,N3),SMACCT(13,9,N3),(SMACCT(13,N2,N3),N2

1=3,8)

860 FORMAT (F6.2,F7.2,F11.2,F12.2,F9.2,F8.2,F10.2,F9.2,F8.2)

C      IF (CROP.NE.JCROP) GOTO 920

C      IF (SKPLOT.EQ.N3) GOTO 920

C      WRITE (5,865) SMDI,SPER,AMONTH(MBEGIN),AMONTH(MEND)

C  865 FORMAT (4X,'SOIL MOISTURE DROUGHT INDEX = ',I4,' DAYS, WITH ',

C     1I4,' CONSECUTIVE DAYS'/5X,'WITH LESS THAN ADEQUATE SOIL MOISTURE

C     2 DURING ',A4,' THROUGH ',A4)

C      WRITE (5,866) VOLPER,AMONTH(MPOND)

C  866 FORMAT ('POND WAS ',F6.2,'% FULL AT BEGINNING OF 'A4)

GOTO 920

C     *****

C     *****       PRINT SOIL MOISTURE ACCOUNTS ON MONTHLY BASIS

C     *****

870 WRITE (5,880) N3,YEAR

880 FORMAT (/,'LAND USE NO.',I3,'  WATER BALANCE (INCHES) FOR LAND USE

1 FOR ',I4,/,79('-'),/,10X,'INPUTS',19X,'OUTPUTS',20X,'SOIL MOISTUR

2E',/,6X,14('-'),2X,36('-'),3X,15('-'),/,'MONTH PRECIP. IRRIG.  INT

3RCPTN PRECIP.EXCESS PERC.  AET       CHANGE  TOTAL')

DO 900 N1=1,13

WRITE (5,890) (SMACCT(N1,N2,N3),N2=1,2),SMACCT(N1,9,N3),(SMACCT(N1

1,N2,N3),N2=3,8)

890 FORMAT (A4,F7.2,F8.2,F9.2,F11.2,F10.2,F7.2,F11.2,F8.2)

900 CONTINUE

920 CONTINUE

GOTO (930,941,930,941,930,941) OUTPUT

C     *****************************************************************

C     *****

C     *****  This section revised November 26, 1990 to include precip.,

C     *****  lake evap. pond excess, and the set of annual values,

C     *****  and to report in actual depths.

C     *****  Further revised September 03, 1993 to include NET pond evap.

C     *****  (Evaporation from pond- precipitation onto pond surface)

C     *****  in depth over its drainage area.

C     *****

C     *****   PRINT DATA FILE ANNLSURF.OUT FOR ANNUAL VALUES

C     *****    as follows:

C     *****

C     *****  YEAR, PRECIP, LAKE EVAP, NET POND EVAP, POND OVERFLOW,

C     *****  PRECIP EXCESS FOR EACH PLOT

C     *****

C     *****************************************************************

930 WRITE (4,931) YEAR,SMACCT(13,2,1),LKEVPT,DAYPE,DAYLD,

1(PEACCT(I),I=1,NPLOTS)

931 FORMAT (I4,22F6.2)

GOTO 946

C     *****

C     ******************************************************************

C     *****  Added November 30, 1989 to print monthly values to disk.

C     *****  Revised September 03, 1993 to add NET pond evaporation.

C     *****  (Evaporation from pond- precipitation onto pond surface)

C     *****  PDOUTF=Pond outflow in inches over its drainage area

C     *****  PDOUTE=NET Pond evaporation in inches over its drainage area

C     *****

C     *****  PRINT DATA FILE MNTHSURF.OUT FOR MONTHLY VALUES

C     *****   as follows:

C     *****

C     *****  YR, MO, PRECIP, LAKE EVAP, NET POND EVAP, POND OUTFLOW,

C     *****    PRECIP EXCESS FOR EACH PLOT

C     *****

C     *****  TOTLKE(I)=Total lake evaporation for month (I)

C     *****

941 DO 945 I=1,12

PDOUTF=PDACCT(I,6)/DA

PDOUTE=(PDACCT(I,4)-PDACCT(I,2))/DA

WRITE (6,942) YEAR,I,SMACCT(I,2,1),TOTLKE(I),PDOUTE,PDOUTF,

1(SMACCT(I,4,J),J=1,NPLOTS)

942 FORMAT (I4,I3,2F6.2,20F6.3)

TOTLKE(I) = 0.0

945 CONTINUE

946 GOTO (950,953,950,953,950,953) OUTPUT

C     *****

C     ******************************************************************

C     *****  Added September 03, 1993 to print annual values of perco-

C     *****  lation to disk.  PDSEEP=Pond seepage expressed as a depth

C     *****  over its drainage area.  This keeps units consistent with

C     *****  the rest of the water balance accounts.

C     *****

C     *****  PRINT DATA FILE ANNLPERC.OUT FOR ANNUAL VALUES

C     *****   as follows:

C     *****

C     *****  YR, POND SEEPAGE, DEEP PERCOLATION FROM EACH PLOT

C     *****

C     ******************************************************************

C     *****

950 WRITE (7,951) YEAR,PDSEEP,(SMACCT(13,5,I),I=1,NPLOTS)

951 FORMAT (I4,19F6.2)

C     *****

C     ******************************************************************

C     *****

C     *****  ADDED SEPTEMBER 15, 1993 TO PRINT ANNUAL IRRIGATION AMOUNTS

C     *****

IF (IPLOT.EQ.0) GOTO 960

WRITE (10,952) YEAR,(SMACCT(13,9,I),I=1,NPLOTS)

952 FORMAT (I4,18F6.2)

GOTO 960

C     *****

C     ******************************************************************

C     *****

C     *****  ADDED SEPTEMBER 15, 1993 TO PRINT MONTHLY IRRIGATION AMOUNTS

C     *****

953 IF (IPLOT.EQ.0) GOTO 956

DO 955 I=1,12

WRITE (9,954) YEAR,I,(SMACCT(I,9,J),J=1,NPLOTS)

954 FORMAT (I4,I3,18F6.2)

955 CONTINUE

C     *****

C     ******************************************************************

C     *****

C     *****  Added September 03, 1993 to print monthly values of perco-

C     *****  lation to disk.  PONDSP=Pond seepage expressed as a depth

C     *****  over its drainage area.  This keeps units consistent with

C     *****  the rest of the water balance accounts.

C     *****

C     *****  PRINT DATA FILE MNTHPERC.OUT FOR MONTHLY VALUES

C     *****   as follows:

C     *****

C     *****  YR, MO, POND SEEPAGE, DEEP PERCOLATION FROM EACH PLOT

C     *****

956 DO 958 I=1,12

PONDSP=PDACCT(I,5)/DA

WRITE (8,957) YEAR,I,PONDSP,(SMACCT(I,5,J),J=1,NPLOTS)

957 FORMAT (I4,I3,19F6.3)

958 CONTINUE

C     *****

C     ******************************************************************

960 CONTINUE

C     *****

WRITE (*,961)

961 FORMAT (//,'      Now preparing final summary and doing post-proce

1ssing, if requested.'//)

C     *****

C     *****   ***** EXIT YEARLY LOOP *****

C     *****

C     *****

C     *****   CALCULATE AVERAGE ANNUAL VALUES

C     *****   RANGE=RANGE OF ANNUAL PRECIPITATION AMOUNTS (IN.)

C     *****   AVGMD=AVERAGE MOISTURE DEFICIT (IN.)

970 EVAP = EVAPLK/YEARS

APREC = TPREC/YEARS

RANGE = WET-DRY

AVGMD = EVAP-APREC

PRCPA = PRCPT/YEARS

PEINA = PEINT/YEARS

EVAPA = EVAPT/YEARS

EXFILA = EXFILT/YEARS

DSCRGA = DSCVOL/YEARS

PETAVG = PETTAC/YEARS

IF (NYDSCH) 990,990,980

C     *****   ADYD=AVG. NUMBER OF DISCHARGES/YEARS HAVING A DISCHARGE

C     *****   AVANDC=AVERAGE ANNUAL DISCHARGE VOLUME

980 ADYD = NODSCH/NYDSCH

AVANDC = DSCVOL/NYDSCH

DSCRG = DSCVOL/NODSCH

GOTO 1000

990 ADYD = 0.0

DSCRG = 0.0

AVANDC = 0.0

1000 DO 1010 J=1,NPLOTS

DSPERC(J) = DSPERC(J)/YEARS

DSRNFF(J) = DSRNFF(J)/YEARS

ACHSOM(J) = ACHSOM(J)/YEARS

AINTER(J) = AINTER(J)/YEARS

AIRRIG(J) = AIRRIG(J)/YEARS

1010 AAETRS(J) = AAETRS(J)/YEARS

IF (NPLOTS.GT.4) NPLOTS = 4

DO 1030 J=1,25

PRECAC(J,3) = CTP(J)

IF (CTPDAY.GT.0.0) CTP(J) = CTP(J)/CTPDAY*100.0

PRECAC(J,1) = ASTAT(J)

PRECAC(J,2) = CTP(J)

PRECAC(J,4) = CTPR(J)

DO 1020 I=1,NPLOTS

INUM1 = 5+I

RUNACC(J,INUM1) = CTR(J,I)

IF (CTRDAY(I).GT.0.0) CTR(J,I) = CTR(J,I)/CTRDAY(I)*100.0

INUM = I+1

1020 RUNACC(J,INUM) = CTR(J,I)

1030 RUNACC(J,1) = ASTAT(J)

NPLOTS = IPLOT

C     *****

C     *****   PRINT FINAL SUMMARY

C     *****

WRITE (5,1040)

1040 FORMAT (////,26X,'***** FINAL SUMMARY *****',/)

WRITE (5,1050) LOCATION

1050 FORMAT ('SUB-BASIN:  ',A48,/)

WRITE (5,1060) YSTART,YEND,YEARS

1060 FORMAT (5X,'PERIOD SIMULATED:  'I4,' TO ',I4,' =',I3,' YEARS',//,

1'METEOROLOGICAL SUMMARY',/)

WRITE (5,1070) EVAP

1070 FORMAT (5X,'AVERAGE ANNUAL LAKE EVAPORATION=',F6.2,' INCHES')

WRITE (5,1080) APREC

1080 FORMAT (5X,'AVERAGE ANNUAL PRECIPITATION=',F6.2,' INCHES')

WRITE (5,1090) RANGE,DRY,WET

1090 FORMAT (5X,'PRECIPITATION RANGE=',F6.2,' INCHES',/,8X,'(FROM A LOW

1 OF',F6.2,' INCHES TO A HIGH OF',F6.2,' INCHES)')

WRITE (5,1100) AVGMD

1100 FORMAT (5X,'AVERAGE MOISTURE DEFICIT=',F6.2,' INCHES')

WRITE (5,1105) PETAVG

1105 FORMAT (5X,'AVERAGE POTENTIAL EVAPOTRANSPIRATION=',F6.2,' INCHES',

1/)

WRITE (5,1110)

1110 FORMAT ('SUMMARY OF POND OPERATIONS (ALL VOLUMES ARE IN ACRE-INCHE

1S)',/)

WRITE (5,1120) NYDSCH

1120 FORMAT (5X,'NUMBER OF YEARS HAVING AN OVERFLOW =',I3)

WRITE (5,1130) ADYD

1130 FORMAT (5X,'AVERAGE NUMBER OF OVERFLOWS PER YEAR HAVING AN OVERFLO

1W =',F6.2)

WRITE (5,1140) DSCRG

1140 FORMAT (5X,'AVERAGE DAILY OVERFLOW VOLUME =',F8.2)

WRITE (5,1150) PEAK

1150 FORMAT (5X,'MAXIMUM DAILY OVERFLOW VOLUME =',F8.2)

WRITE (5,1160) AVANDC

1160 FORMAT (5X,'AVERAGE OVERFLOW VOLUME PER YEAR HAVING AN OVERFLOW =

1',F9.2)

WRITE (5,1170) DSCVOL

1170 FORMAT (5X,'TOTAL OVERFLOW VOLUME=',F10.2)

WRITE (5,1180) PRCPA

1180 FORMAT (5X,'AVERAGE ANNUAL DIRECT PRECIPITATION VOLUME=',F8.2)

WRITE (5,1190) PEINA

1190 FORMAT (5X,'AVERAGE ANNUAL PRECIPITATION EXCESS VOLUME FLOWING INT

1O POND=',F9.2)

WRITE (5,1200) EVAPA

1200 FORMAT (5X,'AVERAGE ANNUAL EVAPORATION VOLUME=',F8.2)

WRITE (5,1210) EXFILA

1210 FORMAT (5X,'AVERAGE ANNUAL SEEPAGE VOLUME=',F8.2)

WRITE (5,1220) DSCRGA

1220 FORMAT (5X,'AVERAGE ANNUAL OVERFLOW VOLUME =',F10.2,/)

WRITE (5,1240)

1240 FORMAT ('SUMMARY OF AVERAGE ANNUAL WATER BALANCE FOR EACH LAND USE

1')

DO 1310 J=1,NPLOTS

WRITE (5,1250) J

1250 FORMAT (/,5X,'LAND USE NO.',I3)

1255 WRITE (5,1256) AIRRIG(J)

1256 FORMAT (/,9X,'IRRIGATION ADDED =',F6.2,' INCHES')

WRITE (5,1260) DSRNFF(J)

1260 FORMAT (9X,'PRECIPITATION EXCESS =',F6.2,' INCHES')

WRITE (5,1270) DSPERC(J)

1270 FORMAT (9X,'PERCOLATION =',F5.2,' INCHES')

WRITE (5,1280) AINTER(J)

1280 FORMAT (9X,'INTERCEPTION =',F5.2,' INCHES')

WRITE (5,1290) AAETRS(J)

1290 FORMAT (9X,'EVAPOTRANSPIRATION = ',F5.2,' INCHES')

WRITE (5,1300) ACHSOM(J)

1300 FORMAT (9X,'CHANGE IN SOIL MOISTURE =',F5.2,' INCHES')

1310 CONTINUE

C     *****

C     ***** PRINT STATISTICAL SUMMARY

C     *****

WRITE (5,1320)

1320 FORMAT (//,'SUMMARY OF STATISTICAL DATA',/)

WRITE (5,1330)

1330 FORMAT ('PRECIPITATION FREQUENCY DATA',//,'INTENSITY',5X,

1'FREQUENCY',6X,'FREQUENCY',4X,'RUNOFF FREQ.',/2X,'(IN.)',8X,'( %

2)',9X,'(DAYS)',9X,'(DAYS)',/)

WRITE (5,1340) ((PRECAC(I,J),J=1,4),I=1,25)

1340 FORMAT (A6,3F15.2)

WRITE (5,1350)

1350 FORMAT (//,'RUNOFF FREQUENCY DATA',//,'INTENSITY',8X,

1'FREQUENCY ( % )',22X,'FREQUENCY (DAYS)',/,'  (IN.)',2X,'L USE 1 L

2 USE 2 L USE 3 L USE 4',5X,'L USE 1 L USE 2 L USE 3 L USE 4',/)

WRITE (5,1360) ((RUNACC(I,J),J=1,9),I=1,25)

1360 FORMAT (2X,A4,2X,4F8.2,4X,4F8.2)

DO 1370 I=3,10

CLOSE (I,STATUS='KEEP')

1370 CONTINUE

GOTO (1400,1400,1400,1400,2900,2900) OUTPUT

C     *****

C     ******************************************************************

C     *****

C     *****  POST-PROCESSING SECTION TO CALCULATE SURFACE AND

C     *****  PERCOLATION/GROUNDWATER YIELD FOR SUB-BASIN

C     *****

C     *****  TAREA = SUB-BASIN AREA, ACRES

C     *****  MD = ANNUAL MOISTURE DEFICIT,INCHES

C     *****  MT = MONTH

C     *****  TLF = TRANSMISSION LOSS FACTOR

C     *****  PXCLU = ANNUAL PRECIP EXCESS FROM EACH LAND USE, INCHES

C     *****  PLU = PERCENT OF LAND USE

C     *****  PRC = ANNUAL PRECIPITATION, INCHES

C     *****  PNDYLD = TOTAL POND SURFACE YIELD FOR SUB-BASIN, ACRE-IN.

C     *****  PNDSEP = TOTAL POND SEEPAGE YIELD FOR SUB-BASIN, ACRE-IN.

C     *****  PNDEV = POND EVAPORATION OVER DRAINAGE AREA, INCHES

C     *****  PNDOUT = POND OVERFLOW OVER DRAINAGE AREA, INCHES

C     *****  NPD = NUMBER OF PONDS IN SUB-BASIN

C     *****  PSEEP = SEEPAGE FROM POND OVER TYPICAL DRAINAGE AREA, INCHES

C     *****  TSUYLD = TOTAL SURFACE YIELD FOR SUB-BASIN, INCHES

C     *****  TPRYLD = TOTAL PERCOLATION YIELD FOR SUB-BASIN, INCHES

C     *****  SYLDLU = SURFACE YIELD FOR A LAND USE, ACRE-IN.

C     *****  TAREA = TOTAL SUB-BASIN AREA, ACRES

C     *****  PRYLD = PERCOLATION YIELD FOR A LAND USE, ACRE-IN.

C     *****  TBSUYL = RUNNING TOTAL GROSS SUB-BASIN SURFACE YIELD, ACRE-IN.

C     *****  SYLDNT = NET SUB-BASIN SURFACE YIELD, ACRE-IN.

C     *****  TTSUYL = RUNNING TOTAL NET SUB-BASIN SURFACE YIELD, ACRE-IN.

C     *****  SYLDAF = NET SUB-BASIN SURFACE YIELD, ACRE-FT

C     *****  TSYLDA = RUNNING TOTAL NET SUB-BASIN YIELD, ACRE-FT

C     *****  PYLDAF = TOTAL PERCOLATION YIELD FOR SUB-BASIN, ACRE-FT

C     *****  TPYLDA = RUNNING TOTAL PERCOLATION YIELD FOR SUB-BASIN, ACRE-FT

C     *****  TTYLD = RUNNING TOTAL SURFACE AND PERCOLATION YIELD, ACRE-FT

C     *****  WTLF = SURFACE YIELD WEIGHTED TRANSMISSION LOSS FACTOR FOR RUN

C     *****  AVGSUF = AVERAGE NET SURFACE YIELD FOR SUB-BASIN FOR RUN, ACRE-FT

C     *****  AVGPER = AVERAGE PERCOLATION YIELD FOR SUB-BASIN FOR RUN, ACRE-FT

C     *****  AVGYLD = AVERAGE TOTAL YIELD FOR SUB-BASIN FOR RUN, ACRE-FT

C     *****  PRP = MONTHLY PRECIPITATION, INCHES

C     *****  PRCT = RUNNING TOTAL PRECIPITATION FOR YEAR, INCHES

C     *****  LAKEVT = RUNNING TOTAL LAKE EVAPORATION FOR YEAR, INCHES

C     *****  LAKEP = MONTHLY LAKE EVAPORATION, INCHES

C     *****  PNDOT = MONTHLY POND OVERFLOW, INCHES

C     *****  PXCLUM = PRECIP EXCESS FOR EACH LAND USE ON A MONTHLY BASIS, IN.

C     *****

1400 OPEN (12,FILE='TEMP',STATUS='OLD')

1500 GOTO (1600,2000,1600,2000) OUTPUT

C     *****

C     ******************************************************************

C     *****

C     *****  OPEN PREVIOUS OUTPUT FILES FOR POST PROCESSING ON ANNUAL

C     *****  BASIS

C     *****

C     ******************************************************************

1600 OPEN(4,FILE='ANNLSURF.OUT',STATUS='OLD')

OPEN(7,FILE='ANNLPERC.OUT',STATUS='OLD')

OPEN(10,FILE='ANNLYLD.OUT',STATUS='UNKNOWN')

STRNG=FIXED

IF (LYEAR.EQ.999) STRNG=HIST

WRITE(10,1610) LOCATION,STRNG

C     *****

C     ******************************************************************

C     *****

1610 FORMAT ('SUMMARY OF SURFACE AND PERCOLATION/GROUNDWATER YIELD FROM

1',//5X,A48,//,A34,/)

IF (LYEAR.NE.999) GOTO 1611

GOTO 1612

1611 WRITE (10,2112) LYEAR

GOTO 1613

1612 WRITE (10,2115)

1613 WRITE (10,1614) TAREA, DA

1614 FORMAT('TOTAL SUB-BASIN AREA = ',F7.0,' ACRES AND TYPICAL POND WAT

1ERSHED = ',F5.0,' ACRES',//,33X,'ALL VALUES UNLESS SPECIFIED ARE I

2N INCHES')

WRITE (10,1620)

1620 FORMAT ('      ESTIMATED,ACRE-FEET TRLS        LAKE MOIST. NO. OVR

1 WTRSHD  LAND USE 01    LAND USE 02    LAND USE 03    LAND USE 04

2   LAND USE 05    LAND USE 06    LAND USE 07    LAND USE 08    LAN

3DUSE 09    LAND USE 10    LAND USE 11    LAND USE 12    LAND USE 1

43    LAND USE 14    LAND USE 15    LAND USE 16    LAND USE 17    L

5AND    5 USE 18')

WRITE (10,1630)

1630 FORMAT ('YEAR  SURF   PERC   TOTAL FCTR PRECIP EVAP DEFCT PNDS SUR

1F SEEP ',18(' -%- SURF PERC '))

DO 1800 I=1,YEARS

TPRYLD=0.0

TSUYLD=0.0

READ (4,*) YEAR,PRC,LAKEVP,PNDEV,PNDOUT,(PXCLU(II),II=1,NPLOTS)

MD = LAKEVP-PRC

TLF = 1.00+6.00E-6*MD**3+5.13E-24*EXP(MD)

IF (MD.LT.10.) TLF = 1.00

IF (TLF.GT.5.) TLF = 5.00

READ (12,*) YEAR,NPD,(PLU(J),J=1,NPLOTS)

READ (7,*) YEAR,PSEEP,(PERCLU(LL),LL=1,NPLOTS)

PNDYLD=DA*NPD*PNDOUT

PNDSEP=DA*NPD*PSEEP

TSUYLD=PNDYLD

TPRYLD=PNDSEP

DO 1700 III=1,NPLOTS

SYLDLU(III) = (TAREA-(DA*NPD))*PLU(III)*PXCLU(III)/100.

TSUYLD=TSUYLD+SYLDLU(III)

PRYLD(III) = TAREA*PLU(III)*PERCLU(III)/100.

TPRYLD=TPRYLD+PRYLD(III)

1700 CONTINUE

TBSUYL=TBSUYL+TSUYLD

SYLDNT=TSUYLD/TLF

TTSUYL=TTSUYL+SYLDNT

SYLDAF=SYLDNT/12.

TSYLDA=TSYLDA+SYLDAF

PYLDAF=TPRYLD/12.

TPYLDA=TPYLDA+PYLDAF

TTYLD=SYLDAF+PYLDAF

WRITE (10,1750) YEAR,SYLDAF,PYLDAF,TTYLD,TLF,PRC,LAKEVP,MD,NPD,PND

1OUT,PSEEP,(PLU(JJ),PXCLU(JJ),PERCLU(JJ),JJ=1,NPLOTS)

1750 FORMAT (I4,3F7.0,F5.2,3F6.2,I5,2F5.2,18(F5.1,2F5.2))

1800 CONTINUE

WTLF= TBSUYL/TTSUYL

AVGSUF=TSYLDA/YEARS

AVGPER=TPYLDA/YEARS

AVGYLD=AVGSUF+AVGPER

WRITE (10,1850) AVGSUF,AVGPER,AVGYLD,WTLF

1850 FORMAT ('AVG.',3F7.0,F5.2)

GOTO 2900

C     *****

C     ******************************************************************

C     *****

C     *****  OPEN PREVIOUS FILES FOR POST PROCESSING ON A MONTHLY BASIS

C     *****

C     ******************************************************************

C     *****

2000 OPEN(6,FILE='MNTHSURF.OUT',STATUS='OLD')

OPEN(8,FILE='MNTHPERC.OUT',STATUS='OLD')

OPEN(10,FILE='MNTHYLD.OUT',STATUS='UNKNOWN')

STRNG=FIXED

IF (LYEAR.EQ.999) STRNG=HIST

WRITE(10,2010) LOCATION,STRNG

2010 FORMAT ('SUMMARY OF SURFACE AND PERCOLATION/GROUNDWATER YIELD FROM

1',//5X,A48,//,A34,I4,/)

IF (LYEAR.NE.999) GOTO 2111

GOTO 2114

2111 WRITE(10,2112) LYEAR

2112 FORMAT (31X,'YEAR = ',I4,/)

2114 WRITE (10,2115)

2115 FORMAT (/)

WRITE(10,2118) TAREA, DA

2118 FORMAT('TOTAL SUB-BASIN AREA = ',F7.0,' ACRES AND TYPICAL POND WAT

1ERSHED = ',F5.0,' ACRES',//,33X,'ALL VALUES UNLESS SPECIFIED ARE I

2N INCHES')

WRITE (10,2020)

2020 FORMAT ('         ESTIMATED,ACRE-FEET TRLS        LAKE MOIST. NO.

1OVR WTRSHD  LAND USE 01    LAND USE 02    LAND USE 03    LAND USE

204    LAND USE 05    LAND USE 06    LAND USE 07    LAND USE 08

3LAND USE 09    LAND USE 10    LAND USE 11    LAND USE 12    LAND U

4SE 13    LAND USE 14    LAND USE 15    LAND USE 16    LAND USE 17

5   LAND USE 18')

WRITE (10,2030)

2030 FORMAT ('YEAR MO  SURF   PERC   TOTAL FCTR PRECIP EVAP DEFCT PNDS

1SURF  SEEP',18(' -%- SURF PERC '))

DO 2800 I=1,YEARS

PRCT=0.0

LAKEVT=0.0

TSUYLD=0.0

TPRYLD=0.0

DO 2200 II=1,12

READ (6,*) YEAR,MT,PRP(II),LAKEP(II),PNDEV,PNDOT(II),(PXCLUM(III,

1II),III=1,NPLOTS)

PRCT=PRCT+PRP(II)

LAKEVT=LAKEVT+LAKEP(II)

2200 CONTINUE

MD = LAKEVT-PRCT

TLF = 1.00+6.00E-6*MD**3+5.13E-24*EXP(MD)

IF (MD.LT.10.) TLF = 1.00

IF (TLF.GT.5.) TLF = 5.00

READ (12,*) YEAR,NPD,(PLU(J),J=1,NPLOTS)

DO 2700 KK=1,12

TSUYLD=0.0

TPRYLD=0.0

READ (8,*) YEAR,MT,PSEEP,(PERCLU(LL),LL=1,NPLOTS)

PNDYLD=DA*NPD*PNDOT(KK)

PNDSEP=DA*NPD*PSEEP

TSUYLD=TSUYLD+PNDYLD

TPRYLD=TPRYLD+PNDSEP

DO 2400 III=1,NPLOTS

SYLDLU(III) = (TAREA-(DA*NPD))*PLU(III)*PXCLUM(III,KK)/100.

TSUYLD=TSUYLD+SYLDLU(III)

PRYLD(III) = TAREA*PLU(III)*PERCLU(III)/100.

TPRYLD=TPRYLD+PRYLD(III)

2400 CONTINUE

TBSUYL=TBSUYL+TSUYLD

SYLDNT=TSUYLD/TLF

TTSUYL=TTSUYL+SYLDNT

SYLDAF=SYLDNT/12.

TSYLDA=TSYLDA+SYLDAF

PYLDAF=TPRYLD/12.

TPYLDA=TPYLDA+PYLDAF

TTYLD=SYLDAF+PYLDAF

WRITE (10,2500) YEAR,MT,SYLDAF,PYLDAF,TTYLD,TLF,PRP(KK),LAKEP(KK),

1MD,NPD,PNDOT(KK),PSEEP,(PLU(JJ),PXCLUM(JJ,KK),PERCLU(JJ),JJ=1,NPLO

2TS)

2500 FORMAT (I4,I3,3F7.0,F5.2,3F6.2,I5,2F5.2,18(F5.1,2F5.2))

2700 CONTINUE

2800 CONTINUE

WTLF= TBSUYL/TTSUYL

AVGSUF=TSYLDA/YEARS

AVGPER=TPYLDA/YEARS

AVGYLD=AVGSUF+AVGPER

WRITE (10,2850) AVGSUF,AVGPER,AVGYLD,WTLF

2850 FORMAT ('AVG.   ',3F7.0,F5.2)

2900 CLOSE (12,STATUS='DELETE')

3000 STOP

END

C     *****

C     ******************************************************************

SUBROUTINE CROPCO (CROP,MGSB,DGSB,MGSE,DGSE,KCROP,NDIM,MMAT)

C     SUBROUTINE CROPCO CALCULATES THE CROP COEFFICIENTS FOR USE IN

C     THE MAIN PROGRAM.  THE CROP COEFFICIENTS ARE CALCULATED BY THE

C     PROCEDURES OUTLINED IN TECHNICAL RELEASE NO 21, IRRIGATION

C     WATER REQUIREMENTS, UNITED STATES DEPARTMENT OF AGRICULTURE,

C     SOIL CONSERVATION SERVICE, ENGINEERING DIVISION, APRIL 1967.

C     SLIGHT MODIFICATIONS HAVE BEEN MADE FOR ADAPTION TO THE MODEL.

C     EQUATIONS FOR THE CROP GROWTH STAGE COEFFICIENT CURVES WERE

C     DEVELOPED WHICH ELIMINATES THE NECESSITY OF READING THE VALUES

C     FROM THE CURVES.  INPUTS TO THE SUBROUTINE INCLUDE THE CROP,

C     MONTH AND DAY GROWING BEGINS AND ENDS, NUMBER OF DAYS IN EACH

C     MONTH, AND THE MEAN MONTHLY AVERAGE TEMPERATURES IN FAHRENHEIT

C     DEGREES.

INTEGER CROP,DGSB,DGSE

INTEGER NDIM(12),SHIFT

C     FOLLOWING LINE REVISED IN SEPTEMBER, 1982

REAL MID(12),DBMD(12),ACC(12),PCGS(12)

REAL MMAT(12),KT(12),KCROP(7,12),PCGS1(12)

C     ACC=ACCUMULATIVE DAYS IN GROWING SEASON

C     MGSB=MONTH GROWING SEASON BEGINS EXPRESSED NUMERICALLY IE 1-12

C     DGSB=DAY GROWING SEASON BEGINS EXPRESSED NUMERICALLY

C     MGSE=MONTH GROWING SEASON ENDS EXPRESSED NUMERICALLY IE 1-12

C     DGSE=DAY GROWING SEASON ENDS EXPRESSED NUMERICALLY

C     MID=MEDIAN DATES OF THE MONTHS IN THE GROWING SEASON

C     DBMD=DAYS BETWEEN MID DATES

C     PCGS=PERCENT OF GROWING SEASON REACHED AT MID DATES

C     MMAT=MEAN MONTHLY AVERAGE TEMPERATURES

C     MGSB1=TEMPORARY STORAGE FOR MGSB

C     MGSE1=TEMPORARY STORAGE FOR MGSE

C     PCGS1=TEMPORARY STORAGE FOR PCGS

C     *****

C     THE FOLLOWING LOOP (DO 5) WAS ADDED IN SEPTEMBER, 1982 TO ZERO

C     THE ARRAYS LOCAL TO CROPCO FOR EACH CALL TO THE SUBROUTINE.

DO 5 I=1,12

PCGS(I) = 0.0

PCGS1(I) = 0.0

MID(I) = 0.0

DBMD(I) = 0.0

ACC(I) = 0.0

5 CONTINUE

MGSB1 = MGSB

MGSE1 = MGSE

IF (MGSB.GT.MGSE) GOTO 10

GOTO 20

C     WHEN MGSB IS GREATER THAN MGSE SUCH AS IN WINTER WHEAT THE

C     SUBROUTINE "SHIFTS" OR ADDS 1 TO MGSB AND MGSE UNTIL MGSB = 13

C     WHICH CORRESPONDS TO JANUARY.  THIS SHIFT WAS NECESSARY TO

C     FACILITATE PROGRAM LOOPING.  AFTER CALCULATIONS ARE MADE THE

C     CROP COEFFICIENTS ARE  "SHIFTED" BACK TO THEIR ORIGINAL MONTHS.

C     ******************************************************************

C     ******************************************************************

C     *****

C     *****

C     *****  CAUTION TO USER ***  THIS ROUTINE WILL NOT WORK IF THE

C     *****  GROWING SEASON EXCEEDS ONE YEAR.

C     *****

C     ******************************************************************

C     ******************************************************************

10 SHIFT = 13-MGSB

MGSE = MGSE+SHIFT

MGSB = 1

20 NPLUS = MGSB+1

NMINUS = MGSE-1

MID(MGSB) = ((NDIM(MGSB)-DGSB)/2.)+DGSB

DO 30 N=NPLUS,NMINUS

30 MID(N) = NDIM(N)/2.0

MID(MGSE) = DGSE/2.0

DBMD(MGSB) = MID(MGSB)-DGSB

DO 40 N=NPLUS,NMINUS

40 DBMD(N) = NDIM(N-1)-MID(N-1)+MID(N)

DBMD(MGSE) = NDIM(MGSE-1)-MID(MGSE-1)+DGSE

ACC(MGSB) = DBMD(MGSB)

DO 50 N=NPLUS,MGSE

50 ACC(N) = ACC(N-1)+DBMD(N)

ACC(MGSE) = ACC(MGSE)-MID(MGSE)

DO 60 N=MGSB,MGSE

60 PCGS(N) = (ACC(N)*100.)/(ACC(MGSE)+MID(MGSE))

IF (MGSB1.LE.MGSE1) GOTO 100

DO 80 N=1,12

NN = N-SHIFT

IF (NN.LE.0) NN = NN+12

IF (NN.GT.MGSE1.AND.NN.LT.MGSB1) GOTO 70

PCGS1(NN) = PCGS(N)

GOTO 80

70 PCGS1(NN) = 0.0

80 CONTINUE

DO 90 N=1,12

90 PCGS(N) = PCGS1(N)

100 MGSB = MGSB1

MGSE = MGSE1

DO 110 J=1,12

C     KT IS A CLIMATIC COEFFICIENT APPLIED TO THE CROP GROWTH

C      COEFFICIENT.  IT IS CALCULATED BY THE FOLLOWING EQUATION:

KT(J) = .0173*MMAT(J)-.314

IF (MMAT(J).LT.36.) KT(J) = .3

110 CONTINUE

C     *****   CROP=1 FOR WHEAT

C     *****   CROP=2 FOR SORGHUM

C     *****   CROP=3 FOR CORN

C     *****   CROP=4 FOR SOYBEANS

C     *****   CROP=5 FOR PASTURE

C     *****   CROP=6 FOR ALFALFA

C     *****   CROP=7 FOR FALLOW

GOTO (120,130,140,150,160,180,200), CROP

120 XBAR = 50.

A = 1.39093399

B = -0.00368378

C = -0.00004976

D = -0.00000233

E = -0.00000004

GOTO 210

130 XBAR = 50.

A = 1.05528355

B = 0.00198600

C = -0.00051577

D = 0.00000045

E = 0.00000011

GOTO 210

140 XBAR = 50.

A = 1.02805328

B = 0.00880046

C = -0.00031919

D = -0.00000194

E = 0.00000007

GOTO 210

150 XBAR = 50.

A = 0.74790430

B = 0.01474796

C = -0.00013486

D = -0.00000443

E = 0.

GOTO 210

C     FOR PERENNIAL CROPS SUCH AS ALFALFA AND PASTURE, VALUES OF THE

C     CROP COEFFICIENTS ARE BEST PLOTTED ON A MONTHLY BASIS. THEREFORE

C     EQUATIONS WERE NOT DEVELOPED.  MONTHLY VALUES WERE INTEGRATED

C     WITHIN THE ROUTINE FOR PASTURE AND ALFALFA.

160 KCROP(5,1) = 0.49

KCROP(5,2) = 0.57

KCROP(5,3) = 0.73

KCROP(5,4) = 0.85

KCROP(5,5) = 0.90

KCROP(5,6) = 0.92

KCROP(5,7) = 0.92

KCROP(5,8) = 0.91

KCROP(5,9) = 0.87

KCROP(5,10) = 0.79

KCROP(5,11) = 0.67

KCROP(5,12) = 0.55

DO 170 J=1,12

KCROP(5,J) = KCROP(5,J)*KT(J)

IF (PCGS(J).LE.0.0) KCROP(5,J) = 0.0

170 CONTINUE

GOTO 230

180 KCROP(6,1) = 0.63

KCROP(6,2) = 0.73

KCROP(6,3) = 0.86

KCROP(6,4) = 0.95

KCROP(6,5) = 1.00

KCROP(6,6) = 1.00

KCROP(6,7) = 1.00

KCROP(6,8) = 1.00

KCROP(6,9) = 0.95

KCROP(6,10) = 0.91

KCROP(6,11) = 0.78

KCROP(6,12) = 0.64

DO 190 J=1,12

KCROP(6,J) = KCROP(6,J)*KT(J)

190 IF (PCGS(J).LE.0.0) KCROP(6,J) = 0.0

GOTO 230

200 XBAR = 0.

A = 0.

B = 0.

C = 0.

D = 0.

E = 0.

210 DO 220 J=1,12

Z = PCGS(J)-XBAR

KCROP(CROP,J) = (A+B*Z+C*Z**2+D*Z**3+E*Z**4)*KT(J)

IF (PCGS(J).LE.0.0) KCROP(CROP,J) = 0.0

220 CONTINUE

230 CONTINUE

C     SINCE THE MAIN PROGRAM APPLIES THE CROP COEFFICIENT (KCROP) TO

C     THE ENTIRE MONTH, THE KCROP WAS PROPORTIONED ACCORDINGLY TO

C     COMPENSATE FOR THIS.  THE NEXT TWO CARDS DO THIS.

KCROP(CROP,MGSB) = KCROP(CROP,MGSB)*(NDIM(MGSB)-DGSB+1)/NDIM(MGSB)

KCROP(CROP,MGSE) = KCROP(CROP,MGSE)*DGSE/NDIM(MGSE)

RETURN

END

SUBROUTINE IART (IA,SNOVAP,IASTN,KROPKO,P,PET,PETBS,IASTT,IAET,

1TEMP)

C     *****   SUBROUTINE IART CALCULATES:

C     *****     1. THE EVAPOTRANSPIRATION AMOUNT WHICH IS DEDUCTED

C     *****        FROM INTERCEPTION STORAGE, IAET

C     *****     2. THE AMOUNT OF PRECIPITATION WHICH IS INTERCEPTED, IA

C     *****     3. THE VALUE OF INTERCEPTION STORAGE FOR THE NEXT DAY'S

C     *****        SIMULATION, IASTT

C     *****   IA=INITIAL ABSTRACTION, ASSUMED TO BE 0.1 INCH OR LESS

C     *****   IAET=AMOUNT OF EVAPOTRANSPIRATION WHICH IS DEDUCTED FROM

C     *****    INTERCEPTION STORAGE (INCHES)

C     *****   IASTOR = TEMPORARY INTERCEPTION STORAGE ACCOUNT

REAL IA,IAET,IASTN,IASTT,IASTOR,KROPKO

IF (SNOVAP.GT.0.0) GOTO 3

C     *****  SNOVAP CONSIDERED PART OF INTERCEPTION PROCESS OCTOBER 1993 JKK.

IF (TEMP.LT.32.) GOTO 3

IA = 0.10

IF (IA.GT.P) IA=P

IF ((IASTN+IA).GT.0.10) IA=0.10-IASTN

IASTOR = IASTN+IA

IF (KROPKO.LE.0.0) GOTO 5

IAET = IASTOR - PET

IF (IAET.LT.0.0) IAET = IASTOR

IASTT=IASTOR-IAET

GOTO 10

3 IAET = SNOVAP

IA = 0.0

GOTO 10

5 IAET = IASTOR-PETBS

IF (IAET.LT.0.0) IAET = IASTOR

IASTT=IASTOR-IAET

10 RETURN

END

SUBROUTINE PETRT (SUN,RAD,HUM,AVGT,BLOW,BRUNTA,BRUNTB,DELTA,E,

1EPRIM,GAMMA,LAKEVP,PACK,PET,PETBS,RCROP,MAXT,MINT,MMMAXT,

2MMMINT,NONM,NOND)

REAL LAKEVP,MAXT,MINT,MMMAXT,MMMINT,NONM,NOND

R = RCROP

IF (AVGT-20.0) 1,1,2

1 PET = 0.0

PETBS = 0.0

LAKEVP = 0.0

GOTO 50

C     *****

C     *****   CALCULATION OF POTENTIAL EVAPOTRANSPIRATION BY MEANS OF

C     *****                 PENMAN EQUATION

C     *****   THE FOLLOWING LINE CHECKS FOR SNOW COVER

2 IF (PACK.GT.0.1) R = 0.70

C     *****   THE NEXT LINE CONVERTS MINT TO DEGREES C

CENTM = (MINT-32.0)*100.0/180.0

C     *****   ESM=DAILY CALCULATED SATURATED VAPOR PRESSURE (MILLIBARS)

C     *****     AT THE MINIMUM DAILY AIR TEMPERATURE

C     *****   RELATIVE HUMIDITY EARLY IN THE MORNING IS USED AS INPUT.

C     *****   SO, ESM SHOULD BE CALCULATED WITH MINT, TOO, TO GET THE

C     *****   ACTUAL VAPOR PRESSURE FOR THE DAY. SEPTEMBER 02, 1993.  JKK

ESM = 33.9*((0.00738*CENTM+0.8072)**8-1.9E-5*ABS(1.8*CENTM+48)+

113.16E-4)

IF (ESM.LE.0.0) ESM = 0.0

C     *****   IF MINT IS BELOW THE MONTHLY AVERAGE MINIMUM TEMPERATURE,

C     *****   THEN THE RELATIVE HUMIDITY IS LIKELY TO BE GREATER THAN

C     *****   AVERAGE FOR THE MONTH.  SO, A FACTOR FOR HUM BASED UPON

C     *****   THE ACTUAL MINT IS ADDED.  IF MINT IS GT 5 DEG BELOW THE

C     *****   AVERAGE, HUM = 100%.  IF MINT IS ABOVE AVERAGE, HUM IS THE

C     *****   MONTHLY AVERAGE.  BETWEEN THESE TWO EXTREMES IS A LINEAR

C     *****   INCREASE IN HUM FROM MONTHLY AVERAGE TO 100%.  SEP 28, 1993 JKK

IF (MINT.GT.MMMINT) GOTO 5

HUM = HUM+0.20*(MMMINT-MINT)*(100.-HUM)

IF (HUM.GT.100.) HUM = 100.

C     *****   ESL = ACTUAL VAPOR PRESSURE FOR THE DAY (MILLIBARS)

5 ESL = ESM*HUM/100.

C     *****   ESA=DAILY CALCULATED DAILY AVERAGE VAPOR PRESSURE (MILLIBARS)

C     *****   (ESA-ESL) VAPOR PRESSURE DEFICIT FOR THE DAY.

C     *****   THE NEXT LINE CONVERTS AVGT TO DEGREES C.

CENTA = (AVGT-32.0)*100.0/180.0

ESA = 33.9*((0.00738*CENTA+0.8072)**8-1.9E-5*ABS(1.8*CENTA+48)+

113.16E-4)

IF (ESA.LE.0.0) ESA = 0.0

C     *****   TO INCREASE VARIABILITY IN RADIATION, EFFECT OF TEMPERATURE

C     *****   DIFFERENCE ON EXPECTED RADIATION WAS ADDED SEPT. 1993.

C     *****   BY (MAXT-MINT)/(MMMAXT-MMMINT) AND CHECK TO SEE IF

C     *****   SUN GT. 100%. SUN = 100% WOULD BE A COMPLETELY CLEAR DAY.

C     *****   (MMMAXT-MMMINT) IS THE AVERAGE DIFFERENCE BETWEEN MMMAXT AND

C     *****   MMMINT. (MAXT-MINT) IS ACTUAL DIFFERENCE FOR THE DAY.

SUNF = (MAXT-MINT)/(MMMAXT-MMMINT)

IF (SUNF*SUN.GT.100.) SUNF = 100./SUN

C     *****   RN=CALCULATED DAILY NET RADIATION (MM OF WATER)

C     *****   AVGT AS ABSOLUTE TEMPERATURE IS NEEDED FOR CALCULATING RN

ABST = ((AVGT-32.0)*5./9.)+273.16

RN = (1-R)*RAD*(0.22+0.54*SUNF*SUN/100.)-2.010E-09*ABST**4*(0.98

1*(1.-BRUNTA-BRUNTB*SQRT(ESA)))*(0.1+0.9*SUNF*SUN/100.)

IF (RN.LT.0.0) RN = 0.0

C     *****   WINDD=MONTHLY AVERAGE WIND RUN AT 2 METERS HEIGHT (MI/DAY)

WINDD = (BLOW*24)*0.555

C     *****   EA=CONVECTIVE LOSSES (MM WATER)

EA = 0.26*(E+0.01*WINDD)*(ESA-ESL)

EALAKE = 0.26*(EPRIM+0.01*WINDD)*(ESA-ESL)

IF (AVGT) 10,10,20

10 DELTA = 0.0

GOTO 30

20 DELTA = 0.039*AVGT**0.673

30 GAMMA = 1-DELTA

C     *****

C     *****************************************************************

C     *****

C     ***** HEAT FACTOR ADDED TO COMPENSATE FOR EFFECT OF STORAGE IN

C     ***** LAKE AND SOIL.  IT REDUCES PET WHEN AIR TEMPS ARE RISING AND

C     ***** INCREASES WHEN AIR TEMPS ARE COOLING.  ADDED OCT. 07 1993 JKK

C     ***** IT IS BASED UPON A PAN TO LAKE DISTRIBUTION FOR LAKE HEFNER

C     ***** WHICH IS USED IN KANSAS.  EQUATION IS FIT OF THE CURVE.

C     ***** DYR = DECIMAL FRACTION THROUGH THE YEAR

C     ***** HF IS THE FACTOR WHICH WILL AVERAGE ABOUT 0.68 FOR THE YEAR.

C     ***** MINIMUM = 0.43 ABOUT MID-MARCH AND MAX IS 0.88 IN MID OCT.

DYR=(NONM-1.)+NOND/30.5

HF= (0.488-0.253*DYR+.0599*DYR**2.0-.00316*DYR**3)**0.50

C     *****

PET = (((DELTA*RN)+(GAMMA*EA))/25.4)*HF

C     *****

C     *****   CALCULATE LAKE AND BARE SOIL EVAPORATION

C     *****

RNSOIL = RN*((1.0-0.20)/(1.0-R))

RNLAKE = RN*((1.0-0.05)/(1.0-R))

PETBS = (((DELTA*RNSOIL)+(GAMMA*EA))/25.4)*HF

LAKEVP = (((DELTA*RNLAKE)+(GAMMA*EALAKE))/25.4)*HF

50 RETURN

END

SUBROUTINE RNOFRT (AVLFCU,PWPUZ,P,RCN1,RCN2,RCN3,RNOF,SMUZ)

C     *****

C     *****   SUBROUTINE RNOFRT CALCULATES THE RUNOFF FROM THE LAND USE

C     *****   USING THE SCS EQUATION:

C     *****        RNOF=((P-0.2*S)**2)/(P+0.8*S)

C     *****   WHERE

C     *****        P=PRECIPITATION AMOUNT, INCLUDING SNOWMELT (INCHES)

C     *****        S=(1000/CN)-10

C     *****        CN=RUNOFF CURVE NUMBER

C     *****        RNOF=CALCULATED RUNOFF AMOUNT (INCHES)

REAL NUM

C     *****

C     *****************************************************************

C     *****

C     *****  RCN CHANGED TO SLIDING SCALE BASED UPON AVAILABLE SOIL

C     *****  IN UPPER ZONE OF SOIL (FASMUZ) BY SIMPLE FITTED EQUATION.

C     *****  CHANGED SEPTEMBER 07 and 20, 1993 JKK

C     *****  DIFFERENT THRESHOLD VALUES FOR GROWING AND NON-GROWING

C     *****  SEASONS WERE ELIMINATED BECAUSE FASMUZ SHOULD ACCOUNT FOR

C     *****  CHANGES BECAUSE SOIL MOISTURE IS LIKELY HIGHER DURING THE

C     *****  NON-GROWING SEASON.

C     *****  IF FASMUZ.LT.50%, AMCI; IF .GT.90%, AMCIII, IN BETWEEN

C     *****  SLIDES FROM I TO III WITH AMCII AT 70%.

C     *****

C     *****************************************************************

C     *****

FASMUZ=(SMUZ-PWPUZ)/AVLFCU

IF (FASMUZ.LT.0.50) GOTO 40

IF (FASMUZ.GT.0.90) GOTO 50

AMC=1.00+5.00*(FASMUZ-0.50)

IF (AMC.LT.2.00) CN=RCN2+(RCN2-RCN1)*(AMC-2.00)

IF (AMC.GE.2.00) CN=RCN2+(RCN3-RCN2)*(AMC-2.00)

GOTO 60

40 CN = RCN1

GOTO 60

50 CN =RCN3

60 S = 1000.0/CN-10.0

NUM = P-0.2*S

IF (NUM) 70,70,80

70 RNOF = 0.0

GOTO 90

80 RNOF = NUM**2/(P+0.8*S)

90 RETURN

END

SUBROUTINE SEEPGE (B2,PONVOL,DASEEP,DSEPRT)

C     *****     SUBROUTINE SEEPGE CALCULATES THE VOLUME OF EXFILTRATION

C     *****   FROM THE POND EACH DAY. THE EXFILTRATION FUNCTION IS

C     *****   ASSUMED TO BE A CONSTANT RATE (DSEPRT) IN INCHES/DAY. THE

C     *****   VOLUME IS FOUND BY MULTIPLYING THIS RATE BY THE POND'S

C     *****   SURFACE AREA. THIS EXFILTRATION VOLUME IS THEN SUBTRACTED

C     *****   FROM THE POND VOLUME.

C     *****   B2=POND SURFACE AREA (SQUARE FEET)

C     *****   DASEEP=DAY'S EXFILTRATION VOLUME (ACRE-INCHES)

C     *****   PONVOL=POND VOLUME (ACRE-INCHES)

DASEEP = B2*DSEPRT/43560.0

IF (DASEEP.GT.PONVOL) DASEEP = PONVOL

PONVOL = PONVOL-DASEEP

IF (PONVOL.LT.0.0) PONVOL = 0.0

RETURN

END

SUBROUTINE SNOWRT (PRECIP,KROPKO,WATER,PACK,PET,PETBS,TEMPAV,

1SNOVAP)

C     *****

C     *****   SUBROUTINE SNOWRT CALCULATES THE MOISTURE ADDED TO THE

C     *****    LAND USE DUE TO MELT OF THE SNOWPACK

C     *****

REAL M,MA,MR,KROPKO

M = 0.0

C     *****   SNOVAP=DEDUCTION FROM THE MOISTURE STORED IN THE

C     *****    SNOWPACK DUE TO SUBLIMATION (INCHES)

C     *****  THE FOLLOWING LINE WAS CHANGED BY ADDING 0.5, OCT. 1993 JKK

IF (KROPKO) 5,5,6

5 IF (PACK.GT.0.) SNOVAP = 0.5*PETBS

IF (SNOVAP.GT.PACK) SNOVAP=PACK

PETBS=PETBS-SNOVAP

GOTO 8

6 IF (PACK.GT.0.) SNOVAP = 0.5*PET

C     *****  THE FOLLOWING LINE WAS ADDED IN SEPTEMBER, 1982

IF (SNOVAP.GT.PACK) SNOVAP=PACK

PET=PET-SNOVAP

8 PACK = PACK-SNOVAP

C     *****  SNOVAP WILL BE INCLUDED IN WATER BUDGET AS INC FOR ACCOUNTING

C     *****  PURPOSES.  WHEN SNOVAP IS GREATER THAN 0. AET WILL BE ZEROED

C     *****  AND NO INITIAL ABSTRACTION WILL BE TAKEN IN SUBROUTINE IART.

C     *****  JKK OCTOBER 18, 1993

IF (TEMPAV-32.) 10,10,20

10 IF (PRECIP) 70,70,30

20 IF (PACK) 90,90,40

30 PACK = PACK+PRECIP

WATER = 0.0

GOTO 90

C     *****   MA=SNOWMELT DUE TO ATMOSPHERIC CONDITIONS (INCHES)

40 MA = 0.05*(TEMPAV-34.)

IF (MA.LT.0.0) MA = 0.0

IF (PACK-MA) 60,60,50

C     *****   MR=SNOWMELT DUE TO RAIN (INCHES)

50 MR = (PRECIP*(TEMPAV-32.))/144

C     *****   M=TOTAL SNOWMELT (INCHES)

M = MR+MA

IF (PACK-M) 60,70,70

60 M = PACK

PACK = 0.0

GOTO 80

70 PACK = PACK-M

C     *****   WATER=SUM OF PRECIPITATION ON THE AREA AND SNOWMELT (IN.)

80 WATER = M+PRECIP

90 RETURN

END

SUBROUTINE VOLRT (A1,A2,A3,A4,A5,L,PSAREA,S,VOLMAX,W,HMAX)

REAL L

C     *****

C     *****   SIZING POND VOLUME ROUTINE

A1 = L*W

A2 = S*(L+W)

A3 = 4./3.*S**2

A4 = 2.*A2

A5 = 4.*S**2

C     *****   VOLMAX=MAXIMUM STORAGE VOLUME IN THE POND (ACRE-IN.)

VOLMAX = (A1*HMAX+A2*HMAX**2+A3*HMAX**3)/3630.0

C     *****   PSAREA=MAXIMUM POND SURFACE AREA (ACRES)

PSAREA = ((W+2.*S*HMAX)*(L+2.*S*HMAX))/43560

RETURN

END